Home-Protecting System

By

Xiangjie Wang (Electrical Engineering)

Course Instructor:

Dr. Guilherme DeSouza

ECE 4220 Real Time Embedded Computing

Department of Electrical and Computer Engineering

University of Missouri – Columbia

Columbia, Missouri

May 10, 2012

# Abstract

This project implements a home-protecting system which consists of auxiliary board with a speaker and five buttons, pressure sensor and keypad. The project aims to help people protecting their home and property if someone wants to break in without the correct password. With this protecting system, people will have fewer probabilities to encounter a theft at home. Even a theft occurs, people could be alarmed. The pressure sensor will be placed near the door and it is used to detect the motion of people who wants to get close to the home. The speaker will alarm if someone touches the pressure sensor and the keypad is used for inputting password. The general working idea of this overall system is that if someone is close to the door, the pressure sensor will be triggered and then speaker alarms. After that, the system requires people to input the default password. If the password is correct, the alarm will be released; if the password is wrong, the alarm is still working. In order to prevent the following scenario that someone touches the pressure sensor accidentally, a button on the auxiliary board is used to release the alarming under this circumstance. In order to implement this home-protecting system, the keypad and pressure sensor should be placed outside, while the speaker should stay inside. From now on, this project could provide a general perception of the home-protection, but all the components used in this project seem to be a little simple to provide a highly safety protection. Therefore, some more advanced devices such as camera and fingerprint scanner could be added to offer a more complicated and safety protection.

# Introduction

This project report describes a home-protecting system. It can be divided into two parts, hardware and software. The hardware part consists of pressure sensor, keypad and the provided auxiliary board with a speaker. All these components are implemented on TS-7250 embedded device. To implement this project, two TS-7250 devices are needed. One should be placed together with pressure sensor and keypad; the other one should be placed with speaker inside the room. Both implementations are done on the data input/output (DIO) header. The difference is that first device use DIO header as input while the second one use DIO header as output. The software part contains four programs, server program, client program and two kernel modules. Server and client programs are implemented on each embedded device and they are used to communicate with each other. One kernel module loads the code that can capture the touch of pressure sensor and keypad; while the other kernel module loads the real time task that can drive the speaker.

# Implementation

## Hardware part

The hardware part contains pressure sensor and keypad. When pressure sensor is touched by several pounds force, two terminals of the sensor will generate up to 3-4 MΩ resistance, but if it is not touched, the resistance can be regarded as zero.

Figure 1 Pressure sensor schematic

The above schematic is for the pressure sensor. According to the characteristics of this product, the output voltage from the LM324N (amplifier) is zero if the blue point is not touched. Contrarily, the output voltage is around 3.3V, which is compatible with the DIO port of TS-7250 which allows a maximum voltage of 3.3V. It is connected to the DIO port and it is regarded as an input data.

Figure 2 Keypad Schematic

This is the schematic of the keypad that I use. My initial idea was to use a 3-digit default password. So I only used the first row (1, 2, and 3) to compose the default password. The principle of this keypad is simple. When number 1 is pressed, line 1 and line 5 is connected (inner the keypad); when number 2 is pressed, line1 and line6 is connected (inner the keypad); when number 3 is pressed, line 1 and line 7 is connected (inner the keypad). Then the output V1, V2 and V3 will generate a voltage of 3.3V (the resistance inside the keypad is negligible). These three terminals are connected to the DIO port of the TS-7250, which is regarded as the input data.

### Hardware Results

Both hardware components work on 0V or 3.3V condition. Theoretically, TS-7250 could recognize these two working mode and do the correct response. But unfortunately, I did not make these hardware components work on TS-7250. First, I tested my hardware components on the DIO port of TS-7800, it worked as I expected. TS-7800 could recognize the digital high when I pressed the blue region on the pressure sensor and the keypad also worked. But when I tried these two parts on TS-7250 in the Linux lab, it was totally different as I expected. TS-7250 failed to recognize the digital high output from the sensor and the keypad. I tried to figure out this problem in the manual, but it only says that the input voltage should not be larger than 3.3V, so I could not solve this problem on TS-7250.

## Software part

The software part contains four programs, one is server, one is client and the other two are kernel modules. The server is client program is executed on the embedded device that implements the pressure sensor and keypad while the server program is executed on the other one.

Outside

Inside

Figure 3 Software Flowchart

This is the simple flowchart of the overall system. First, the kernel module in the “Outside window” is a real time task which is used to sense the pressure sensor and capture the input password. Once the sensor is touched, the kernel Module will store a symbol into a FIFO, and then client program get the data from FIFO and then send this symbol to server program via socket. In this case, I use UDP communication instead of TCP in order to broadcast messages if we have multiple devices in the future.

Once server grasps the symbol, another kernel module could capture this symbol through another FIFO. Besides receiving the data, another purpose of the server program is used to enable the software interrupt signal. The kernel Module inside the room is another real time task, the main purpose of which is to implement software interrupt to make the speaker sound. Once this kernel module gets the symbol, this module can trigger the speaker.  At this time, the client program will indicate user to type a 3-digit password. The following steps are almost the same as the previous ones, the only difference is that the kernel module in the “Inside window” should verify the password. If it is correct, the alarm is released; if not, it still alarms. In order to prevent some unexpected situations, I also implement hardware interrupt in the kernel module in the “Inside window” which can release the alarm anytime you want.

### Software Results

As I discussed in the hardware implementation part, my hardware components work only on TS-7800 but not TS-7250. So I tried to use TS-7800 to install the first kernel module and run the client program. But this time, I also had some problems on the client and server programs. The issue was that the client program (on TS-7800) could receive messages from server program (on TS-7250), but the server program could not receive signals from client programs. I tried both UDP and TCP methods, it was always this situation.  I was wondering if this problem was caused by the different form of IP address. Because IP format of TS-7250 is 10.3.52.XX, while the format of TS-7800 is another format. I could not figure out why the socket was always unidirectional. Therefore, the alternative of using TS-7800 as the client was failure.

# Project Results

As I already discussed in the last few passages, my first two methods could not implement the project successfully. Therefore, I had to abandon my own hardware components and TS-7800; instead, I used another auxiliary board to act as pressure sensor and keypad. Three buttons could be regarded as the password input and another one button could be regarded as pressure sensor. Under this scenario, the overall project could work. The video could demonstrate my overall project results.

# Conclusion

Since I only showed the idea of the project, it was not successful overall. I failed to implement the hardware components on TS-7250. Although it could work on TS-7800 when I used the same circuit, the TS-7800 could not be a client because of the socket communication between client and server programs. Some further work still needs to be done. One is to solve the hardware problems of the pressure sensor and the keypad.  Another is that I can use a more sensitive pressure sensor not connecting to DIO port but to ADC instead. The reason is that the pressure sensor is always proportional to force, and the output voltage is also proportional to force. Therefore, the ADC converter is more suitable and practical to implement the pressure sensor.