ECE7220 Real Time Embedded System Final Project

---- Shopping Assistant Embedded System

STUDENT                    Ruidong Gu

DATE HANDED IN             05/14/2013

Catalogue

Motivation	2

Introduction to Project and Structure	4

Hardware of project	4

Software of project	4

Detailed implementation algorithm and method	4

Button press part	8

Socket to exchange message	10

Build, update and rank path of dynamic chain	12

Conclusion, Problem Solving and Future Work	16

Conclusion, Problem Solving	16

Future Work	18

Code Section	19

ECE7220 Real Time Embedded System Final Project

Shopping Assistant Embedded System

Ruidong Gu, Electrical Engineering

Dates:

05/14/2012

Objective:

- To apply the knowledge learned this semester to the practical application;

- To learn how to incorporate different part to form a integral system;.

Motivation

At beginning of this semester I begin to consider one project to implement, and then I find these inconvenient in real life and think about how to develop an embedded system like this.

- Various of items spread in a large area, cost too much time to find

There are amount of goods in different area of supermarket, every time you could think about where is it and which area I need to visit next, it really spends a lot time on such thing; especially in outlets, we do not visit these market as often as supermarket, hence, every time we need to waste time to consider the path and location for brand we want. Hence, we need one machine for customers to enter the items or brand they want to buy and then this machine would return one path to tell customers which area they should visit next and what they should buy in this area.

- Where is your friend?

When you are shopping with friends, sometimes they would go to different areas to find their own items, but if you want to find him, it would also waste your time think about the path; so if you can just type a command into one machine with a number belonging to your friends’ machine, it would tell you where are you friends and you could find them easily.

- Recommendation for your friends

In outlets, there would be great discount in all kinds of brand and goods. If you find really great discount need to share with your friend, it would be convenient to send it through one machine directly.

- Add new shopping brand or goods during your trip

It is also possible for you to add any new items during your trip, the system would redesign a path for you to follow.

- Allow you to check how many time you spend in on visit

System you record initial time when you begin one trip, and return past time when you press the time look up button.

In summary, a system within five main functions would be developed:

- Design path in order and display items in each area;

- Add new items during trip and rebuild routine;

- Communicate with friends to ask for his location;

- Communicate with friends to recommend discount;

- Look up time consummation during trip;

- Go to the next area and show the items on screen

Introduction to Project and Structure

- Hardware of project

Shopping assistant embedded system in one board

Keyboard as input

Button press part

Network part:  to communicate with another board within this system

One terminal

Screen to display

Each board within this system need keyboard and button press part to be input and screen to be output as a whole “terminal” and the network is needed to do communication with other terminal.

- Software of project

There are six main features I list in the motivation parts, the communication of friends search and recommendation part could be incorporated into one button, hence these five buttons would be applied according to the five features as following:

- 1st button: Begin trip, accept input from keyboard and build path according to area ID and items, finally display the first area and items:

- 2nd button: Go to next area and display area ID and goods to buy in this area;

- 3rd button: receive new items you want to add during trip, rebuild the path and display new area ID message and items;

- 4th button: this button has two features, single click is to show the past time from the time you begin the trip; double click is to exit the system;

- 5th button: click this button, it would show two options, one is to find friends and then type your friends name and system would return the area ID of your friends; another option is to type the recommendation items and send it to other terminals.

- There would be one thread to accept message through socket and testify the message: if it is find friends, compare with own name, if it is, send my next area ID out; if it is recommendation items, compare the message name with my, if it is, do not display, if it is not mine, accept and display;

- For the button press part, if I only check button, there is no need to use real time in kernel space since there is no time constraint need for button press, however, all five button has incorporated with features, I need another button to exit of system, so I try to create a method to implement double click through time constraint: once this button is pressed, a time would be recorded, the time difference would be calculated in terms of last recorded time to determine if this operation is single click or double click. Hence, for the button with double click and single click is need to be real time, since the time should be rigid control to justify; then I use other four button also in real time module although there is no need to do this.

- I would show the whole process using chart below:

Blue blank: represent input part;

Green blank: represent screen display

Red blank: show network message exchange through socket

Build path

For supermarket

For outlets

Market database

Outlets database

Keyboard

Select Market Type

Add Nodes

Find area

Market or Outlets select

Combine nodes in same area based on area ID

Rank nodes according to area ID

Print Nodes:

1st button: display first node

2nd button: once press, one pointer point to the next one from current position, initial position is head

Head

Node 2

Node 2

………………….

Tail

4th button Put “4”

1stbutton

2nd button

3rd button

4th button

5th button

Interrupt

Handler

Justify

1st button Put “1”

2nd button Put “2”

3rd button Put “3”

Fifo

1

5th button Put “5”

Thread

Start

Network Send

Socket

Keyboard

Input name

Receive message, verify, display recommendation items and send back area ID now back to your friend

Select Market Type

While (1)

2

Fifo

Check fifo 1

If “1”

Keyboard

Receive items through keyboard and build path and display area 1 iterms

If “2”

Receive items through keyboard

Go to the next area and show the items in this area, if goes to end, it would display trip end and exit

If “3”

Screen Display

Build path

If “4”

Receive new items, rebuild the path and show the area 1 and items

Receive 1: friend search, get input name from keyboard

Send out friends name through network

Get the time from fifo 2 and calculate time different

If single click, show time past time

If double click

Network Send

Socket

END

Receive keyboard input to choose communication content

Receive 2: Recommendation, get input from keyboard

Send out items through network

For message exchange, the message has following types:

- Friend search: send “@ nameoffriend”

- Recommendation: send “#,recommendationitems,nameofmine”

- Return to friend of my area ID: send “*,areaID,myname”

From the chart above we could see the whole process of the system.

Detailed implementation algorithm and method

- Button press part

In this part, there are two main tasks in this part, one is set hardware interrupt of five buttons, second part is how to recognize single click and double click.

- For hardware interrupt, register and handler should be written and FIFO would be needed to pass the button press sign to main program

  - Register to set:

- Five buttons should be set as interrupt: GPIOBintEn (0808400B8), all the low five bits should be set  “1” to be an interrupt;

- interrupt type should be falling edge, since the button press is to create pulse from “1” to ”0”:

- GPIOBIntType1 (0x8084_00AC): all low five bits are set “1” to be edge sensitive;

- GPIOBIntType2 (0x8084_00BC): all low five bits are set “0” to be falling edge;

- Debounce register is to debounce the input avoiding one press creating two pulse:

GPIOBDB (0x8084_00C4): all low five bits are set “1”;

- Clear interrupt register is to clear an edge sensitive interrupt to tell the request line this interrupt has been done and could accept another, or it would never receive other interrupt:

GPIOBEOI (0x8084_00B4): bit is set “1” to clear relative interrupt;

  - Hardware handler, is to build hardware handler and bind it with one request line, for GPIO, GPIO combined interrupt (59) and enable the request line, and handler would be called once one button as interrupt is pressed (triggered), inside the handler:

    - at first it need to disable this request line since only one interrupt could be accept in each time within one request line: rt_disable_irq(IRQ_NUM);

    - check which button is pressed, put relative number into FIFO 1 which is to recognize which button pressed in main program;

    - after main operation, you should clear the interrupt in EOI register to tell the request line this interrupt has been accept and you could accept another one;

    - at last you need to enable the interrupt since you have disable it at beginning, let the request line could accept new interrupt;

- To recognize the single or double click in 4th button, there would be another FIFO to record the time of the this button to calculate the time difference to implement the double click operation:

Each time 4th button is pressed, except the put number into FIFO 1, time should be put into FIFO2; accordingly, the main program would create time variable to store the time stamp in FIFO 2, each time main program read a time stamp, get time difference with the value in time variable in main program and to check this time, if it is between 100ms and 500ms, it would be seen as double click; otherwise, it is single click. Although we use debounce register, the button press still cause some one click create two button press signal in a short time, through experiment I set time different below 100ms are all the noise signal, hence, I pick time different among 100-500ms as double click period.

Compare and get double click or single click information

Initialize Time variable time0=0

Time different=time1-time0

Read fifo, get “time1”

time0=time1

Through this procedure, we could justify the double click and single click for each press of 4th button;

In sum, through the method above, we could get a really great effect for button press.

- Socket to exchange message

In this part, there are two main tasks in this part, one is build socket to do communication between two terminals, second part is to testify the message received, and then response, display what really need on screen; in order to receive message all the time, there would be a thread to receive, verify and response message.

- For socket building, the broadcast socket is needed to make sure every terminal receive all the message, but only pick what it wants through testify, in this part how to build my socket should be presented:

  - socket (AF_INET, SOCK_DGRAM,0): the second parameter should be set like this which is conectionless to do broadcast;

  - seradd.sin_addr.s_addr = INADDR_ANY: IP address of the machine on which the server is running;

      - seradd.sin_family = AF_INET: symbol constraint for internet domain

      - seradd.sin_port = htons(PORT_NUM): port number need to bind, we need to convert host to network type;

  - bind(soc, (struct sockaddr *)&seradd, serlen): this command is to bind socket to the port we set before;

  - setsockopt(soc, SOL_SOCKET, SO_BROADCAST, &boolval, sizeof(boolval)): just like the third parameter type, we need to set this socket as broadcast type to communication;

- To verify and response message, thread tasks would also be pivotal, one is to build on thread to do all these verify and response tasks; another one is to create how many types message; final task is how to verify the message:

- To construct one thread:

The thread would accept a data structure pointer including all the variables needed to send message through socket, one buffer initial address to send message and then it could be used to receive, verify and response message.

- For message exchange, the message has following types:

      - Friend search: send “@ nameoffriend”

      - Recommendation: send “#,recommendationitems,nameofmine”

      - Return to friend of my area ID: send “*,areaID,myname”

- How to verify the message:

Hence, when you want to find friends, send “@ nameoffriend”, when your friends ’systems receive this message, get the “nameoffriend” and compare it with their terminal name, if correct, send back their area ID with their names in “*,areaID, myname”, if not ,do nothing; and then your system could receive “*,areaID,myname”, get the “myname”, compare it with your friends’ name you have searched, if it is, display the area ID, if not, it means this message is not from your friends, do nothing.

When you want to send recommendation items, send “#,recommendationitems,nameofmine”, when you and other terminals all receive this message, system would get “nameofmine” and compare with your name, if it is the same, do not display message, if not display it on screen.

Through these steps, the socket communication could be implemented well as required.

- Build, update and rank path in dynamic data chain

In this part, the main C++ part to implement dynamic chain of data including build dynamic chain, add nodes, combine nodes, rank nodes, delete nodes, display node. This part would describe the basic implement method and algorithm.

- Build dynamic chain, is mainly to add node and connect them to form one data chain, and finally to find area ID in database. The inheritance, dynamic allocating memory are applied to build one chain

- Inheritance: there is one parent class called “AreaList”, two son classes of “MarketList and StoreList” is inherited from parent class, once you select the market type at beginning in the process, the data type would be set as “NameString” type or “BrandString” type for Market and Outlets respectively. These two classes are to set the data for each node including area ID, items names for each node.

- Dynamic allocating memory: a class called “AreaNode” is to build, once a name string has been accepted, it would be cut into piece one by one, each name would be one node at first; and then class AreaNode” contains three pointers, one for storing the data which is the same type of data, another one called “nextNode” is to store the next node address which owns the same type of node; the other one called “head” within same type of node is to store the head address of chain. Every adding operation is to first allocate memory for new node and set it data within the one item node, then set its “nextNode” to point to the previous node address, and then let “head” pointer points to this new node.

Name String: carrot phone milk laptop

carrot

phone

milk

laptop

Node 1:

Data: carrot

nextNode: NULL

Node 2:

Data: phone

nextNode: Node 1

Node 3:

Data: milk

nextNode: Node 2

Node 4:

Data: laptop

nextNode: Node 3

Head

Head

Node 1:

Data: carrot

nextNode: NULL

Node 2:

Data: phone

nextNode: Node 1

Node 3:

Data: milk

nextNode: Node 2

Node 4:

Data: laptop

nextNode: Node 3

Add process                                    Final effect

- Find area is just based two simple functions as database to be searched, it could compare the name of items with information stored in function with known area ID, if the items name is the same with the one in database, it would be set using the database one area ID

- Update chain

Main task for this part is to combine the node with same area ID, a trick is applied in this part, I just let one pointer called “holder A” hold every node address once; when holder A” holds one node address called “A”, let another pointer called “holder B” goes around this chain from head to tail, once “holder B” met some node with the same area ID with the one in “holder A”, this function would copy the name string stored in “holder A”  into “holder B” node data; hence, the “holder B” node would have two items and then delete node in “holder A”, let “holder A” point to next node until goes to the tail, if there is only one node for each area, it would stop.

Node 1:

Data: carrot

nextNode: NULL

Node 2:

Data: phone

nextNode: Node 1

Node 3:

Data: milk

nextNode: Node 2

Node 4:

Data: laptop

nextNode: Node 3

Holder B

Holder A: Node 4

Holder B

Node 2:

Data: phone laptop

nextNode: Node 1

Compare ID, different to next

Compare ID, Same and copy

Delete Node: 4

- Rank chain

Main task for this part is to rank the chain in terms of the area ID; I use the algorithm of selection sort in class this semester.

The basic point is to use two pointers, one points to the head and the other one point to the tail, let the tail pointer goes around the chain from tail to head, and compare area ID of tail pointer with the head node, once the node in tail pointer within lower area ID, switch their name string and area ID, this could not make chain node structure change, only switch the data to implement the ranking task.

Conclusion, problem solving and future work

- Conclusion, problem solving

From the description, there is an overview for the whole system, this system combined sort algorithm, method to build dynamic chain and node operation of chain, interrupt, thread and socket to form a multiple function system. Each part has its own usage:

  - Build dynamic chain, is the core part to store the shopping list of customer, and process the items and design the path, during this process, sort algorithm is used to design path based on the area ID;

  - Interrupt and FIFO is used to process the button press operations, only 4th button is needed to be deterministic for its press time which should be set as real time interrupt, in order to accept the button press convenient I set other four button also as interrupts, then main program could clear know button press operations through reading FIFO.

  - Thread and socket is combined together to form a communication system between terminals. There are two types of communication, socket is to send out the message, and the system should continue receiving message to make sure response and accept useful information. Accordingly the verification for the message is a main task for system to recognize message needed.

Throughout the whole project implementation, there are also lots of problems in each part, I try multiple methods and really learned a lot from this procedure:

- During building chain problem, segmentation fault should not be ignored, at beginning I only use “cout” to print out sign sentence to find out the error position; later on I find it really takes a long time and sometimes you could not well understand the point of problem, then I search the debugging method in linux, I find “GDB”, it is really a powerful command to find out the error and the error reason;

- Interrupt and FIFO part is really smooth since the lab give me a good practice, the only problem is to find a method to detect double click and to debounce through the double click and single click;

- Thread and socket also came out several problems:

  - first one is how many types of messages I need, I focus on the two communication method and which one is needed to response and then get the answer;

  - Second one is the string operation, when I use “strcpy” only and then do comparison, there is a problem, two buffer with same content but they are seen not equal by “strcmp”, then I find the “stcpy” sometime would not add “\0” to mark the end of a buffer, then I use “sprintf” to add “\0” for buffer, it works well;

- The command “sort ()” is useful to make a order of the function value from small to large, and then we could pick what we want;

- There are also a lot of matrix operations like the calculation i.e. multiple, eigen-decomposition.

- Then the method to convert the angle in decimal to the model using 20 length binary model also takes certain time to design,  I do this obey the normal method to convert it by calculation.

- Future work

For this project, there should be lots of places to implement better:

- There should be simpler and clearer menu for user to select;

- The input part also need to add more protection if user input wrong characters;

- The data chain part could be packed better to read and implement;

In this class, really a wide overview of an embedded system, no matter some algorithm in software part, or multiple thread to use; accordingly strategies to avoid starvation, priority inversion and deadlock; and the schedule strategy and scheduler work process; to the device and communication way inter process and between devices; also the communication and protocol between different device or network; at last the interface usage, all of this really bring me a deep impact on this area, obviously I find there is a long way to go in this area for me. Hence, there would be a continuous learning in this area, and I will still keep what I have got in this course to make future learning with higher efficiency and confidence.

Code section:

- Database header file to store two functions including the items names and their area ID for supermarket and outlets respectively:

/*
 * Database.h
 *
 *  Created on: May 1, 2013
 *      Author: rgcxc
 */

#ifndef DATABASE_H_
#define DATABASE_H_
#include <stdio.h>
#include <stdlib.h>
#include <string.h>
static int Datasearch(char *Stringname)
{
    int sign=0;
    char Database[100];
    strcpy(Database,Stringname);
    if(!strcmp(Database,"cabbage"))
        sign=1;
    else if(!strcmp(Database,"carrot"))
        sign=1;
    else if(!strcmp(Database,"tomato"))
        sign=1;
    else if(!strcmp(Database,"rice"))
        sign=1;
    else if(!strcmp(Database,"bread"))
        sign=1;

    if(!strcmp(Database,"laptop"))
        sign=2;
    else if(!strcmp(Database,"earphone"))
        sign=2;
    else if(!strcmp(Database,"phone"))
        sign=2;
    else if(!strcmp(Database,"tv"))
        sign=2;
    else if(!strcmp(Database,"game"))
        sign=2;

    if(!strcmp(Database,"coffee"))
        sign=3;
    else if(!strcmp(Database,"beverage"))
        sign=3;
    else if(!strcmp(Database,"wine"))
        sign=3;
    else if(!strcmp(Database,"milk"))
        sign=3;

    if(!strcmp(Database,"potato"))
        sign=4;
    else if(!strcmp(Database,"chocolate"))
        sign=4;
    else if(!strcmp(Database,"sugar"))
        sign=4;
    else if(!strcmp(Database,"cookie"))
        sign=4;

    if(!strcmp(Database,"table"))
        sign=5;
    else if(!strcmp(Database,"chair"))
        sign=5;
    else if(!strcmp(Database,"cushion"))
        sign=5;
    return sign;
}
static int Brandsearch(char *Stringnamebrand)
{
    int signband=0;
    char bramdbase[100];
    strcpy(bramdbase,Stringnamebrand);
    if(!strcmp(bramdbase,"nike"))
        signband=1;
    else if(!strcmp(bramdbase,"adidas"))
        signband=1;
    else if(!strcmp(bramdbase,"kappa"))
        signband=1;
    else if(!strcmp(bramdbase,"puma"))
        signband=1;
    else if(!strcmp(bramdbase,"converse"))
        signband=1;

    if(!strcmp(bramdbase,"gap"))
        signband=2;
    else if(!strcmp(bramdbase,"ck"))
        signband=2;
    else if(!strcmp(bramdbase,"levis"))
        signband=2;

    if(!strcmp(bramdbase,"coach"))
        signband=3;
    else if(!strcmp(bramdbase,"gucci"))
        signband=3;
    else if(!strcmp(bramdbase,"parade"))
        signband=3;

    if(!strcmp(bramdbase,"crocs"))
        signband=4;
    else if(!strcmp(bramdbase,"lids"))
        signband=4;
    else if(!strcmp(bramdbase,"bass"))
        signband=4;
    return signband;
}
#endif /* DATABASE_H_ */

- Shop_Class.h header file is to store the class of data type, one parent class “AreaList” and two son classes: “StoreList” and “MarketList”:

/*
 * Shop_Class.h
 *
 *  Created on: May 1, 2013
 *      Author: rgcxc
 */

#ifndef SHOP_CLASS_H_
#define SHOP_CLASS_H_
#include "Database.h"
#include <iostream>
using namespace std;
class AreaList{
protected:
    int areaID;
    char GOODlist[50];
    int visittype;
    int order;
public:
    AreaList* Createnewdata(int rid, int rorder, char *tem);
    void getprint(AreaList* getdataPtr){
        getdataPtr->printToConsole();
    };
    int iniset(AreaList* getid){
        int gid;
        gid=getid->order;
        return(gid);
    };
    int areareturn(){
        return this->areaID;
    };
    int orderreturn(){
        return this->order;
    };
    void rankarea(int rankid){
        this->areaID=rankid;
    };
    void recordnew(char *newstring){
        strcpy(this->GOODlist,newstring);
    };
    AreaList(){
        order = 0;
        areaID = 0;
        visittype = 0;
        memset(GOODlist,0,sizeof(GOODlist));
    };
    //int retrieve();
    ~AreaList(){
    };
/*
============================================================================
virtual function to implement in son class
============================================================================
*/
    virtual void initFromConsole() = 0;
    virtual void printToConsole() = 0;
    virtual char * stringreturn() = 0;
    virtual void recordstring() = 0;
    virtual void rankexchange() = 0;
    virtual void updatearea() = 0;
};

class MartketList:public AreaList{
private:
    char *NameString;
public:
    MartketList(){
        NameString=NULL;
    };
    ~MartketList(){
        if (NameString=NULL)
            delete []NameString;
    };
    AreaList* Createnewdata(int rid, int rorder, char *tem);
    void getprint();
    void updatearea(){
        if (this->areaID==0)
            areaID=Datasearch(this->GOODlist);
    };
    char * stringreturn(){
        return this->NameString;
    };
    void rankexchange(){
        memset(this->NameString,0,sizeof(this->NameString));
        strcpy(this->NameString,this->GOODlist);
    };
    void recordstring(){
        if(strcmp(this->NameString,this->GOODlist)!=0)
        {
            strcat(this->NameString," ");
            strcat(this->NameString,this->GOODlist);
        }
    };
    void initFromConsole(){
        this->NameString = new char(50);
        strcpy(this->NameString,this->GOODlist);
    };
    void printToConsole(){
        cout<<"###############"<<endl<<"ID: "<<visittype<<endl<<"Goods name: "<<NameString<<endl<<"The area ID is: "<<areaID<<endl<<"###############"<<endl<<endl;
    };
};

class StoreList:public AreaList{
private:
    char *BrandString;
public:
    StoreList(){
        BrandString=NULL;
    };
    ~StoreList(){
        delete []BrandString;
    };
    AreaList* Createnewdata(int rid, int rorder, char *tem);
    void getprint();
    void updatearea(){
        if (this->areaID==0)
            areaID=Brandsearch(this->GOODlist);
    };
    void recordstring(){
        strcat(this->BrandString," ");
        strcat(this->BrandString,this->GOODlist);
    };
    void rankexchange(){
        memset(this->BrandString,0,sizeof(this->BrandString));
        strcpy(this->BrandString,this->GOODlist);
    };
    char * stringreturn(){
        return this->BrandString;
    };
    void initFromConsole(){
        this->BrandString = new char(50);
        strcpy(this->BrandString,this->GOODlist);
    };
    void printToConsole(){
        cout<<"###############"<<endl<<"ID: "<<visittype<<endl<<"Brand name: "<<BrandString<<endl<<"The area ID is: "<<areaID<<endl<<"###############"<<endl<<endl;
    };
};

#endif /* SHOP_CLASS_H_ */

- Shop_Class.cpp is a implementation file for its headerfile:

/*
 * Shop_Class.cpp
 *
 *  Created on: May 5, 2013
 *      Author: rgcxc
 */

#include "Shop_Class.h"
inline AreaList* AreaList::Createnewdata(int rid, int rorder, char *tem){
    AreaList *Initiald=NULL;
/*
============================================================================
This it used to create data memory for each node
============================================================================
*/
    if(rid==1){
        Initiald=new MartketList;
        (*Initiald).visittype=rid;
        Initiald->order=rorder;
        strcpy((*Initiald).GOODlist,tem);
        Initiald->initFromConsole();
    }
    else {
        Initiald=new StoreList;
        Initiald->visittype=rid;
        Initiald->order=rorder;
        strcpy((*Initiald).GOODlist,tem);
        Initiald->initFromConsole();
    }

    return Initiald;
}

- Data_Class.h is the class of “AreaNode” and “ChainList” which is to build and update chain:

/*
 * Data_Class.h
 *
 *  Created on: May 1, 2013
 *      Author: rgcxc
 */

#ifndef DATA_CLASS_H_
#define DATA_CLASS_H_
#include "Shop_Class.h"
//#include "Data_Class.cpp"
using namespace std;
//The MUPNode class
class AreaNode{
private:
    AreaList* dataPtr;
    AreaNode* nextNode;
public:
    AreaNode(){
        dataPtr=NULL;
        nextNode=NULL;
    };
    ~AreaNode(){
    };
    void setNext(AreaNode* nNext);         //This is used to set the next node for the new member of the list
    void setData(AreaList* nData);     //This is used to set the next data for the new member of the list

    AreaNode* next();   //This is used to return the next node given the node information
    AreaList* data();   //This is used to return the data given the node information
    void findarea();
};

//The MUPList class
class ChainList{
private:
    AreaNode* head;
    AreaNode* currenthold;
    ChainList *nextlist;
    char *updatestore;
    int orderstore;
    int updateinfo;
    int updateneed;
public:
    ChainList(){
        head=NULL;
        currenthold=NULL;
        updatestore=NULL;
        nextlist = NULL;
        orderstore=0;
        updateinfo=0;
        updateneed=0;
    };
    ~ChainList(){
        if(head==NULL)
            delete head;
        if (updatestore=NULL)
            delete []updatestore;
        if (nextlist->updatestore=NULL)
            delete [] nextlist->updatestore;
    };
    void add(AreaList* data);
    void find();
    void update();
    void rank();
    void locatememo();
    void freememo();
    int areaget(){
        return ((this->currenthold)->data())->areareturn();
    };
    int findorder(int nID);
    void remove(int nID);
    void print(int flag);
    void clear(int flagclear);
};

#endif /* DATA_CLASS_H_ */

- Data_Class.cpp is a implementation file for its headerfile:

/*
 * Data_Class_IM.h
 *
 *  Created on: May 1, 2013
 *      Author: rgcxc
 */

#include "Data_Class.h"
inline void AreaNode::setNext(AreaNode* nNext){         //This is used to set the next node for the new member of the list
    if(nNext == NULL)
        nextNode=NULL;
    nextNode=nNext;
}
inline void AreaNode::setData(AreaList* nData){     //This is used to set the next data for the new member of the list
    dataPtr=nData;
}
inline AreaNode* AreaNode::next(){    //This is used to return the next node given the node information
    //    cout<<"    Data must address:"<<this->nextNode<<endl;
    return this->nextNode;
}
inline AreaList* AreaNode::data(){   //This is used to return the data given the node information
    return this->dataPtr;
}
inline void AreaNode::findarea(){
    this->dataPtr->updatearea();
}
inline void ChainList::add(AreaList* data){
    AreaNode *movPtr=new AreaNode();     //allocate dynamic space for new members
    movPtr->setData(data);                 //set the data of new member using the received data in this function
    if (head==NULL)
    {
        head=new AreaNode;
        movPtr->setNext(NULL);                 //set the next node of new member
    }
    else
        movPtr->setNext(head);                 //set the next node of new member
    head = movPtr;                         //using head to point to the new member
}
inline void ChainList::find(){
    AreaNode *datafind=head;
    while(1)
    {
        if (datafind == NULL)
            break;
        datafind->findarea();
        datafind=datafind->next();
    }
}
inline void ChainList::locatememo(){
    this->updatestore=new char(100);
    this->nextlist = new ChainList;
    this->nextlist->updatestore = new char(100);
}
inline void ChainList::freememo(){
    this->updatestore=NULL;
    delete []this->updatestore;
    this->nextlist = NULL;
    delete this->nextlist;
}
/*
============================================================================
This function is used to update the chain to combine the node with same area
ID
============================================================================
 */
inline void ChainList::update(){
    AreaNode *cdatapick=new AreaNode;
    AreaNode *cdataupdate=new AreaNode;
    int begin=0;
    cdatapick->setData(this->head->data());
    cdatapick->setNext(this->head->next());
    cdataupdate->setNext(this->head->next());
    this->updateneed == 0;
    while(1)
    {
        if(cdataupdate->next()==NULL)
        {
            this->updatestore=NULL;
            this->nextlist=NULL;
            break;
        }
        if((this->orderstore)!=0)
        {
            cdatapick->setData(this->head->data());
            cdatapick->setNext(this->head->next());
            this->orderstore=0;
        }
        else if(begin!=0)
        {
            cdatapick->setData(cdatapick->next()->data());
            cdatapick->setNext(cdatapick->next()->next());
        }
        strcpy(this->updatestore,(cdatapick->data())->stringreturn());
        this->updateinfo=(cdatapick->data())->areareturn();
        cdataupdate->setNext(cdatapick->next());
        begin=1;
        while(1)
        {
            if (cdataupdate->next() == NULL || this->updateneed != 0)
                break;
            cdataupdate->setData(cdataupdate->next()->data());
            this->nextlist->updateinfo=(cdataupdate->next()->data())->areareturn();
            if(this->nextlist->updateinfo==this->updateinfo)
            {
                strcpy(this->nextlist->updatestore,(cdataupdate->data())->stringreturn());
                this->orderstore=(cdatapick->data())->orderreturn();
                (cdataupdate->next()->data())->recordnew(this->updatestore);
                (cdataupdate->next()->data())->recordstring();
                this->updateneed=1;
            }
            else
            {
                cdataupdate->setNext((cdataupdate->next())->next());
                memset(this->nextlist->updatestore,0,sizeof(this->nextlist->updatestore));
                this->nextlist->updateinfo=0;
            }
        }
        if((this->orderstore)!=0)
        {
            this->remove(this->orderstore);
            this->updateneed=0;
        }
        cdataupdate->setNext(cdatapick->next()->next());
    }
}
/*
============================================================================
This function is to do sort for the chain
============================================================================
*/
inline void ChainList::rank(){
    AreaNode *dataupdate=head;
    AreaNode *datapick=head;
    AreaNode *datafor=head;
    AreaList *datagrab;
    AreaList *dataser;
    int ranksign=0,rankinver=0,ranknor=1,stop=1;
    memset(this->updatestore,0,sizeof(this->updatestore));
    memset(nextlist->updatestore,0,sizeof(nextlist->updatestore));
    while(datapick!=NULL)
    {
        datapick=datapick->next();
        ranksign++;
    }
    rankinver=ranksign;
    datapick=head;
    while (1)
    {
        if (ranknor==ranksign)
            break;
        rankinver=ranksign;
        while(1)
        {
            datapick=head;
            dataupdate=datafor;
            if (rankinver==stop)
                break;
            for(int ii=1;ii<rankinver;ii++)
            {
                datapick=datapick->next();
            }
            //behind data
            datagrab=datapick->data();
            strcpy(this->updatestore,datagrab->stringreturn());
            this->updateinfo=datagrab->areareturn();
            //head data
            dataser=dataupdate->data();
            strcpy(nextlist->updatestore,dataser->stringreturn());
            nextlist->updateinfo=dataser->areareturn();
            //compare
            if (nextlist->updateinfo>this->updateinfo)
            {
                dataser->recordnew(this->updatestore);
                dataser->rankexchange();
                dataser->rankarea(this->updateinfo);
                datagrab->recordnew(nextlist->updatestore);
                datagrab->rankexchange();
                datagrab->rankarea(nextlist->updateinfo);
                //                cout<<"!!!!!"<<endl;
            }
            rankinver--;
        }
        datafor=datafor->next();
        ranknor++;
        stop++;
    }
}
/*
============================================================================
This function is to find the node order to know which node it is for system
to delete it, all this is used in the remove ()
 ============================================================================
 */
inline int ChainList::findorder(int nID){
    AreaNode *mark=NULL;
    AreaList *received=NULL,*MUp=NULL;
    int sign=0,temid;
    mark=this->head;
    while(mark!=NULL){                        //send the head to the "mark",using mark to testify if the given "nID" has existed in the databadse
        received=mark->data();            // if the ID has been here, then set the flag "sign" to "1", or keeping the original value "0", then return the sign
        temid=received->iniset(received);
        if(temid==nID){
            sign=1;break;}
        else mark=mark->next();}
    return sign;
}
/*
============================================================================
This function is to remove node, when you combine a node, one node should be
delete
============================================================================
 */
inline void ChainList::remove(int nID){
    AreaList* ptrdata;
    AreaNode* usehead;
    AreaNode* ptrnext;
    AreaNode* holdcurrent;
    int nid,signhere;
    usehead=this->head;
    holdcurrent =NULL;
    // if head is NULL, output empty information directly
    if(head==NULL)
        cout<<"This is an empty Database!"<<endl;
    else{
        signhere=this->findorder(nID);
        if(signhere==0)
            cout<<"There is no this ID in database"<<endl;
        while (usehead != NULL)
        {
            ptrnext=usehead->next();
            ptrdata=usehead->data();
            nid=(*ptrdata).iniset(ptrdata);
            //decide if the ID of head is equal to the given one
            if (nid == nID)
            {
                //if "holdcurrent" is empty, it means that the nID is just the head of the list, so let head directly point to the third element in the list
                if (holdcurrent ==NULL)
                {
                    //                    head = NULL;
                    head = ptrnext;
                }
                //If not, let the current element point to the next second element, which jump current node whose id is equal to the given one
                else
                {
                    holdcurrent->setNext(ptrnext);
                }
                break;
            }
            // if the first data is the one we want, let it point to the next one and let "holdcurrent" point to the current one accordingly in terms of deleting the node we want
            else
            {
                holdcurrent =usehead;
                usehead=ptrnext;
            }
        }
    }
}

/*
============================================================================
This function is to clear this chain
============================================================================
*/
inline void ChainList::clear(int flagclear){
    AreaNode* ptrnext;
    // if head is NULL, output empty information directly and then exit this procedure
    if(head==NULL && flagclear==0){
        cout<<"This is an empty Database!\n";
        exit(0);}
    //if there is element in the list, delete node one by one and then exit this procedure
    else{
        while(head!=NULL)
        {
            ptrnext=head;
            head=head->next();
            delete ptrnext;

        }

    }
}

/*
============================================================================
This function is to print the result, for first time it would only display
first node, for mode 2, it would display next node
============================================================================
*/

inline void ChainList::print(int flag){
    AreaNode *preprint=head;
    AreaList *Areatem=NULL;
    if(head==NULL)
        cout<<"This is an empty Database!\n";
    if (flag==1)
    {
        Areatem=preprint->data();
        Areatem->getprint(Areatem);
        currenthold=(preprint->next());
    }
    else if (flag==2 && currenthold!=NULL)
    {
        Areatem=currenthold->data();
        Areatem->getprint(Areatem);
        currenthold=(currenthold->next());
    }
    else if (flag==2 && currenthold==NULL)
    {
        if(head==NULL){
            cout<<"This is an empty Database!\n";
            exit(0);}
        cout<<"The trip has gone to end"<<endl;
        exit(0);
    }
}

- Socketstr.h is a data structure inside used to send this structure pointer into thread:

/*
 * socketstr.h
 *
 *  Created on: May 4, 2013
 *      Author: rgcxc
 */

#ifndef SOCKETSTR_H_
#define SOCKETSTR_H_
typedef struct{
    socklen_t frolen;
    struct sockaddr_in rec;
    char *recbuff;
    struct sockaddr_in sen;
    char *senbuff;
    int socp;
    ChainList *list;
} socpass;

#endif /* SOCKETSTR_H_ */

- Shop_kernel.c is the kernel module to set up the interrupt for five buttons:

/*
 ============================================================================
 Name        : Shop_kernel.c
 Author      : Ruidong
 Version     :
 Copyright   : Final Project
 Description : Hello World in C, Ansi-style
 ============================================================================
 */

#define MODULE
#define __KERNEL__

#include <linux/module.h>
#include <linux/kernel.h>
#include <rtai.h>
#include "rtai_sched.h"
#include <rtai_fifos.h>
#include <linux/time.h>
MODULE_LICENSE("GPL");
RT_TASK rttask;
int signms=0;
unsigned long portvalue;
unsigned long *pfdr;
unsigned long *ptr;
unsigned long *ptrsoft;
unsigned long *pftrsoft;
unsigned long *pfddr;
unsigned long *sfdr;
unsigned long *sfddr;
unsigned long *binten;
unsigned long *bintype1;
unsigned long *bintype2;
unsigned long *beoi;
unsigned long *bdb;
unsigned long *intstsb;
unsigned long kerneltime;
#define IRQ_NUM 59
static void rt_hardinterrup(int irq,void *cookie);
void rt_keepkernel();
int period;
int clear=0;
int signsoft=0;
int signmessage=0;
int sign=0;
int init_module(void) {
    /*create fifo*/
    rtf_create(0, sizeof(int));
    //        rtf_create(1,sizeof(int));
    rtf_create(2,sizeof(int));
    /*return virtual address to map the physical address of the port*/
    ptr = (unsigned long *)__ioremap(0x80840000,4096,0);
    ptrsoft = (unsigned long *)__ioremap(0x800C0000,4096,0);
    pftrsoft = (unsigned long *) ((char *) ptrsoft + 0x1C);
    /*offset to port B of two register DR and DDR*/
    pfdr = (unsigned long *) ((char *) ptr + 0x04);
    pfddr = (unsigned long *) ((char *)pfdr + 0x10);
    /*set the high three bit to 1 to make three LEDs to be output*/
    * pfddr =* pfddr | 0xE0;
    sfdr = (unsigned long *) ((char *) ptr + 0x30);
    sfddr = (unsigned long *) ((char *)sfdr + 0x04); //offsets to DDR for port F
    *sfddr = *sfddr | 0x02; //Set 2nd bits to 1 to output
    /*
    ============================================================================
    offset to port B and set the register
    ============================================================================
     */
    binten = (unsigned long *) ((char *) ptr + 0x00B8);
    bintype1 = (unsigned long *) ((char *) ptr + 0x00AC);
    bintype2 = (unsigned long *) ((char *) ptr + 0x00B0);
    beoi = (unsigned long *) ((char *) ptr + 0x00B4);
    bdb = (unsigned long *) ((char *) ptr + 0x00C4);
    //      intstsb = (unsigned long *) ((char *) ptr + 0x00BC);
    /*
    ============================================================================
    initialize the register
    ============================================================================
     */
    * binten = * binten | 0x1F;
    * bintype1 = * bintype1 | 0x1F;
    * bintype2 = * bintype2 & 0xE0;
    * bdb = * bdb | 0x1F;
    /*if the module is installed, turn off the three lights*/
    * pfdr = *pfdr & 0xFFFFFF1F;
    // declaring the variables
    printk("lab6 install successfully!\n");
    /*
        ============================================================================
        Configure soft interrupt
        ============================================================================
     */
    rt_request_irq(IRQ_NUM, rt_hardinterrup, NULL, 1);
    rt_enable_irq(IRQ_NUM);
    period=nano2count(500000);
    /*set the period of the real time task*/
    /*let real time task periodic*/
    rt_set_periodic_mode();
    /*initialize the real time task*/
    rt_task_init(&rttask, rt_keepkernel, 0, 512, 0, 0, 0);
    /*start the timer of real time*/
    start_rt_timer(period);
    /*define the real time task period*/
    rt_task_make_periodic(&rttask,rt_get_time(),400*period);
    return 0;
}
static void rt_hardinterrup(int irq,void *cookie)
{
    int sendch=0;
    struct timeval times;
    rt_disable_irq(irq);
    portvalue = 0;
    portvalue = * pfdr;
    sign = portvalue & 0x1F;
    switch (sign)
    {
    case 31:
        break;
    case 30:
        sendch=1;
        rtf_put(0,&sendch,sizeof(sendch));
        break;
    case 29:
        sendch=2;
        rtf_put(0,&sendch,sizeof(sendch));
        break;
    case 27:
        sendch=3;
        rtf_put(0,&sendch,sizeof(sendch));
        break;
    case 23:
/*
============================================================================
Get the time if 4th button is interrupt to testify if it is single click or
double click
============================================================================
*/
        do_gettimeofday(&times);
        sendch=4;
        rtf_put(0,&sendch,sizeof(sendch));
        rtf_put(2,&times,sizeof(times));
        break;
    case 15:
        sendch=5;
        rtf_put(0,&sendch,sizeof(sendch));
        break;
    }
    * beoi = * beoi | 0x1F;
    rt_enable_irq(irq);

}
void rt_keepkernel(){
    int i=0;
    while(1)
    {
        for(i=0;i<10;i++)
        {;}
        rt_task_wait_period();
    }

}

void cleanup_module(void){
    /*if the module is uninstalled, turn on the three lights*/
    * pfdr = *pfdr | 0xE0;
    /*stop the timer of real time*/
    stop_rt_timer();
    /*create fifo*/
    rtf_destroy(0);
    rtf_destroy(2);
    * beoi = * beoi | 0x1F;
    rt_release_irq(IRQ_NUM);
    /*delete the realtime task when module is uninstalled*/
    rt_task_delete(&rttask);
    printk("lab6 is uninstall!\n");

}

- Shop_Main.cpp is the main implementation file:

/*
 * Shop_Main.cpp
 *
 *  Created on: May 11, 2013
 *      Author: rgcxc
 */

//============================================================================
// Name        : Shop_Assistant.cpp
// Author      : Ruidong
// Version     :
// Copyright   : Final Project
// Description : Hello World in C++, Ansi-style
//============================================================================

#include<iostream>
#include<string.h>
#include<cstring>
#include "rtai.h"
#include <pthread.h>
#include <unistd.h>
#include <sys/mman.h>
#include <fcntl.h>
#include <sys/time.h>
#include "Data_Class.cpp"
#include "Shop_Class.cpp"
#include <arpa/inet.h>
#include <netdb.h>
#include <netinet/in.h>
#include <sys/types.h>
#include <sys/socket.h>
#include <sys/ioctl.h>
#include <net/if.h>
#include "socketstr.h"
#define PORT_NUM 2000
#define Max_SIZE 40
using namespace std;
//declare variables for button operations
unsigned long *sfdr;
unsigned long *sfddr;
unsigned long *pfdr;
unsigned long *ptr;
unsigned long *pfddr;
unsigned long portvalue;
char NAME[10];
char FRIENDNAME[10];
static void speaker();
int main(){
    using namespace std;
    ChainList *templist=new ChainList();
    AreaNode *Areap=new AreaNode;
    char temstore[50];
    char *temcut=NULL;
    int a,sign=0,signid=0;
    int select=0;
    int locorder=0;
    int remo=0;
    int pipe_m;
    int threadstore=0;
    int fd;
    int clearzero=0;
    struct timeval timestc;
    struct  timeval timelook;
    unsigned long resultstart=0;
    unsigned long resultstamp=0;
    unsigned long result=0;
    unsigned long doubleclicktime=0;
    //    void *Getts_Time(void* ptr);
    void* Get_Socket(void* ptr);
    pthread_t receivesocket;
/*
============================================================================
soc is the socket used to send and receive data; socip is used to get ip;
sersendlen and serlen is the length of two variable of sockaddr for send and
reveive; sign is used to mark if there is ip received which is bigger than
mine; temkeep is to keep the random number in one vote; cliadd is used to
receive message, the sersend is used to send message; seradd is used to
define socket; buffer is to store the received message, the ip_string is to
store my IP.
============================================================================
*/
    int soc, socip, n, rannum, serlen, sersendlen, temkeep=0;
    int boolval = 1,master=0,count=0;
    socklen_t fromlen;
    struct sockaddr_in seradd, cliadd, ip_addr, sersend;
    struct ifreq ipinfo;
    char buffer[Max_SIZE];
    char buffersend[Max_SIZE];
    char temsend[50];
/*
============================================================================
socket create, bind and set as broadcast.
============================================================================
*/
    soc=socket(AF_INET, SOCK_DGRAM,0);
    if (soc<0)
        printf("Opening error for socket!\n");
    serlen = sizeof(seradd);
    seradd.sin_addr.s_addr = INADDR_ANY;
    seradd.sin_family = AF_INET;
    seradd.sin_port = htons(PORT_NUM);
    if (bind(soc, (struct sockaddr *)&seradd, serlen)<0)
        printf("Binding error of soc!\n");
    if (setsockopt(soc, SOL_SOCKET, SO_BROADCAST, &boolval, sizeof(boolval))<0)
    {
        printf("Setting socket options error for soc!\n");
        exit(-1);
    }
/*
============================================================================
Initialization for the send sockaddr and receive sockaddr
============================================================================
*/
    sersend.sin_addr.s_addr = INADDR_ANY;
    sersend.sin_family = AF_INET;
    sersend.sin_port = htons(PORT_NUM);
    fromlen = sizeof(struct sockaddr_in);    // size of structure
    cliadd.sin_addr.s_addr = inet_addr("10.3.52.255");
    sersend.sin_addr.s_addr = inet_addr("10.3.52.255");
    socpass *sptr=new socpass;
/*
============================================================================
Initialization for using speaker
============================================================================
*/
    fd = open("/dev/mem",O_RDWR); //opens up special file /dev/mem
    ptr = (unsigned long *)mmap(NULL, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED, fd, 0x80840000);
    sfdr = (unsigned long *) ((char *) ptr + 0x30);
    sfddr = (unsigned long *) ((char *)sfdr + 0x04); //offsets to DDR for port F
    *sfddr = *sfddr | 0x02; //Set 2nd bits to 1 to output
/*
============================================================================
Initialization fifo to accept button press information from kernel space
============================================================================
*/
    int fd_fifo_button,fd_fifo_time;
    fd_fifo_button=open("/dev/rtf/0",O_RDWR);
    fd_fifo_time=open("/dev/rtf/2",O_RDWR);
/*
============================================================================
Give customer a menu to collect name, select market type and enter items
============================================================================
*/
    cout<<"Please type your name:"<<endl;
    cin.clear();
    fflush(stdin);
    cin>>NAME;
    cout<<"Please press 1 to begin your trip:"<<endl;
    while(threadstore!=6){
        threadstore=0;
        if(read(fd_fifo_button,&threadstore,sizeof(threadstore))<0){
            cout<<"Fifo read error"<<endl;
            exit(-1);
        }
        cout<<"*********************************************"<<endl<<"Please type your commander:"<<endl<<"<Button 1> Begin trip! "<<endl;
        cout<<"<Button 2> Go to next area"<<endl;
        cout<<"<Button 3> Add New Items"<<endl;
        cout<<"<Button 4> Check Time Spend"<<endl;
        cout<<"<Button 5> Communicate With Friend together"<<endl;
        cout<<"*********************************************"<<endl<<"Your commander is: "<<threadstore<<endl<<endl;
        switch(threadstore){
        case 1:
/*
============================================================================
Create socket receive and response thread to receive, verify and response
message
============================================================================
*/
            gettimeofday(&timestc,NULL);
            sptr->frolen = fromlen;
            sptr->rec = cliadd;
            sptr->recbuff = buffer;
            sptr->sen = sersend;
            sptr->senbuff = buffersend;
            sptr->socp = soc;
            sptr->list = templist;
            pthread_create(&receivesocket,NULL,Get_Socket,(void *)sptr);

            templist->clear(1);
            sign=0;
            while(sign!=1){
                cout<<"Enter which type of Martket you would like? "<<endl<<"<1> Super Market"<<endl<<"<2> Outlets"<<endl<<endl;
                cin.clear();
                fflush(stdin);
                cin>>a;
                if(a!=1 && a!=2)
                    cout<<"The number you type is invalid\n";
                else sign=1;}
            if (a==1)
            {
                cout<<"Please enter Goods name you want to buy"<<endl<<endl;
                fflush(stdin);
                cin.ignore();
                scanf("%[^\n]",temstore);
            }
            else if(a==2)
            {
                cout<<"Please enter Brand name you want to visit"<<endl<<endl;
                fflush(stdin);
                cin.ignore();
                scanf("%[^\n]",temstore);
            }
/*
============================================================================
Build data chain through add(), find area ID from find(), combine node
through update(), sort data chain through rank(), and print first data node
through print(1) and print next node through print(2);
============================================================================
*/
            locorder=1;
            temcut=strtok(temstore," ");
            templist->add((Areap->data())->Createnewdata(a,locorder,temcut));
            while(1)
            {
                temcut=strtok(NULL," ");
                if (temcut==NULL)
                    break;
                locorder+=1;
                templist->add((Areap->data())->Createnewdata(a,locorder,temcut));
            }
            templist->find();
            templist->locatememo();
            templist->update();
            templist->freememo();
            templist->locatememo();
            templist->rank();
            templist->print(1);
            break;
        case 2:
            templist->print(2);
            break;
        case 3:
            memset(temstore,0,sizeof(temstore));
            cout<<"Please enter Items name you want to add"<<endl<<endl;
            fflush(stdin);
            cin.ignore();
            scanf("%[^\n]",temstore);
            temcut=strtok(temstore," ");
            locorder++;
            templist->add((Areap->data())->Createnewdata(a,locorder,temcut));
            while(1)
            {
                temcut=strtok(NULL," ");
                if (temcut==NULL)
                    break;
                cout<<"Error"<<endl;
                locorder+=1;
                templist->add((Areap->data())->Createnewdata(a,locorder,temcut));
            }
            templist->find();
            templist->freememo();
            templist->locatememo();
            templist->update();
            templist->freememo();
            templist->locatememo();
            templist->rank();
            cout<<endl<<"Your new path is: "<<endl;
            templist->print(1);
            break;
        case 4:
/*
============================================================================
Get the time for 4th button to testify single click or double click: single
click to display the time spend, double click to exit the system
============================================================================
*/
            if(read(fd_fifo_time,&timelook,sizeof(timelook))<0){
                cout<<"Fifo read error"<<endl;
                exit(-1);
            }
            resultstamp=1000000*(timelook.tv_sec)+timelook.tv_usec;
            if((resultstamp-doubleclicktime)>500000)
            {
                resultstart=1000000*(timestc.tv_sec)+timestc.tv_usec;
                result=resultstamp-resultstart;
                cout<<"                            The time you spend is:"<<result<<" us"<<endl;
            }
            else if((resultstamp-doubleclicktime)<=500000 &&(resultstamp-doubleclicktime)>100000)
            {
                cout<<"You select exit!"<<endl;
                threadstore=6;
            }
            doubleclicktime=resultstamp;
            break;
        case 5:
/*
============================================================================
Communication function between terminal through socket
============================================================================
*/
            cout<<"**************"<<endl<<"Please select your choice:"<<endl<<"<1> Find people"<<endl<<"<2> Send discount message"<<endl;
            cin.clear();
            fflush(stdin);
            cin>>select;
            cout<<endl<<"Your commander is: "<<select<<endl<<"**************"<<endl;
            memset(buffersend,0,sizeof(buffersend));
            memset(temsend,0,sizeof(temsend));
            switch (select)
            {
            case 1:
                cout<<"Friends name: "<<endl;
                cin>>temsend;
                memset(FRIENDNAME,0,sizeof(FRIENDNAME));
                sprintf(FRIENDNAME,"%s\0",temsend);
                sprintf(buffersend,"@ %s",temsend);
                n = sendto(soc, buffersend,Max_SIZE,0,(struct sockaddr *)&sersend,fromlen);
                if (n<0)
                    printf("Send message error!\n");
                break;
            case 2:
                cout<<"Recommendation name: "<<endl;
                fflush(stdin);
                cin.ignore();
                scanf("%[^\n]",temsend);
                sprintf(buffersend,"#,%s,%s",temsend,NAME);
                n = sendto(soc, buffersend,Max_SIZE,0,(struct sockaddr *)&sersend,fromlen);
                if (n<0)
                    printf("Send message error!\n");
                break;
            default:cout<<"The commander is invalid"<<endl;
            }
            break;
            default:cout<<"The commander is invalid"<<endl;
        }
    }
    delete templist;
    return 0;
}

void* Get_Socket(void* ptr){
    socpass *usesoc = (socpass *)ptr;
    char *temsoc=NULL;
    char temsocstore[50];
    char temname[10];
/*
============================================================================
receive message from other terminals, verify and response through this thread:
if receive "@", represent some terminal is finding friends, if your terminal
is what it found, send back your own area ID with * and your name;if you
receive "*" represent there is someone sending back his location, system would
verify the name with this message to see if this is who i am finding, if it
is, display the area ID;if you receive "#", it means this message is
recommendation items, check name, if it is sending by myself, do not display,
if not mine, display it.
============================================================================
*/
    while(1)
    {
        if((recvfrom(usesoc->socp,usesoc->recbuff,Max_SIZE,0,(struct sockaddr *)&(usesoc->rec),&(usesoc->frolen)))<0)
            printf("Receive message error!\n");
        if(*(usesoc->recbuff)=='@')
        {
            temsoc = strtok(usesoc->recbuff," ");
            temsoc = strtok(NULL," ");
            strcpy(temsocstore,temsoc);
            if(strcmp(temsocstore,NAME)==0)
            {

                sprintf(usesoc->senbuff,"*,%d,%s",usesoc->list->areaget(),NAME);
                if((sendto(usesoc->socp, usesoc->senbuff,Max_SIZE,0,(struct sockaddr *)&(usesoc->sen),(usesoc->frolen)))<0)
                    printf("Send message error!\n");
            }
        }
        else if (*(usesoc->recbuff)=='#')
        {
            temsoc = strtok(usesoc->recbuff,",");
            temsoc = strtok(NULL,",");
            strcpy(temsocstore,temsoc);
            temsoc = strtok(NULL,",");
            strcpy(temname,temsoc);
            if((strcmp(temname,NAME))!=0)
            {
                cout<<"                       Recommendation discount: "<<temsocstore<<endl;
                speaker();
            }
        }
        else if (*(usesoc->recbuff)=='*')
        {
            temsoc = strtok(usesoc->recbuff,",");
            temsoc = strtok(NULL,",");
            strcpy(temsocstore,temsoc);
            temsoc = strtok(NULL,",");
            strcpy(temname,temsoc);
            if((strcmp(temname,NAME))!=0 && (strcmp(temname,FRIENDNAME)==0))
            {
                cout<<"                        Your Friends is in area: "<<temsocstore<<endl;
                speaker();
            }
        }
    }
    pthread_exit(0);
}
static void speaker(){
    int si,sk;
    for(si=0;si<1000;si++)
    {
    *sfdr=*sfdr | 0x02;  //activate speaker
 msync(ptr, getpagesize(), MS_ASYNC);//synching the arrival of signal to the port
    for(sk=0;sk<10000;sk++){}  // the period of time the speaker open
    *sfdr=*sfdr & 0xFD;  //close speaker
 msync(ptr, getpagesize(), MS_ASYNC);//synching the arrival of signal to the port
    for(sk=0;sk<500;sk++){}  // the period of time the speaker close
    }
}