EEG Monitoring System

Jialei Yang

Objectives and Project Description

EEG (Electroencephalogram) signal is a widely used signal to monitor the status of patients or subjects. But when there are many users, getting data from these people living in different places is difficult. If all signals can be monitored together, it is more convenient.

So the system I proposed is an EEG monitoring system can apply to more than one user. The EEG data from different users will be sent to the same server. The server then put data in the Faircom database. Another program can read and display the required data.

Due to the limit of device, only one real EEG band is used in the system and other clients use simulated signals which are history records.

Implementation

- System Structure

The system consists of three main parts: Client, Server and Monitor. Figure 1 shows its main structure.

Figure 1: System structure

- Client

The Client is responsible for getting the EEG signals and sending to the server. The client and server connect using TCP protocol. As I mentioned before, there is only one real EEG band. So the client program has two options to choose, one is choose to link to the real EEG band, the other is choose one history file to read.

- Real EEG Band

The EEG band is developed by National Chiao Tung University. It comes with the software to connect with the EEG band by Bluetooth and receive the data to put in the text file. The structure of the EEG band shows in figure 2.

Figure 2: EEG band structure

Figure 3 shows an example of the text file.

Figure 3: text file

The first line of the text file is the communication parameters: channel numbers, resolution and sample rate. After that, every signal represents one row which consists of the time and signals of four channels.

Because the signals are already in a text file, so a windows program is developed to read this files and send to the client. In this part, the client works as the server of the windows program. They also communicate with TCP protocol. Figure 4 is the structure of the client linked to the real EEG band.

Figure 4: structure of the client linked to the real EEG band

- Simulated EEG Band

For the other clients will read data from the history EEG data. The data are saved in the local disc, so the clients just need to read the local text file and send to the server. The history EEG data have the same structure with the real one.

- Server

Server is aim to handle the operation of the clients and the monitoring program. There are two options to operate the database: read and write. The read option is for the monitoring program. It can receive the request of the monitoring program and read the corresponding data out to. The write option is for the clients. It can receive the data and put them into the database. Both communications are based on TCP.

- Monitoring program

The monitoring program is a Matlab GUI program on windows system. It sends request to the server according to the request of the user. And plot the signal data from the server. It uses TCP to communicate with the server.

- Database Structure

There are three tables in the database: Info (Table 1), Connect Record (Table 2) and Signal Record (Table 3).

Table 1: Info

| Field | Type | Format |
| --- | --- | --- |
| In_usrno | CT_INT2 | 1,2,3 |
| In_name | CT_STRING | “aa”,”bb” |
| In_gender | CT_FSTRING | “F”,”M” |
| In_age | CT_FSTRING | 20,40 |

Table 2: Connect Record

| Field | Type | Format |
| --- | --- | --- |
| Cr_usrno | CT_INT2 | 1,2,3 |
| Cr_sequence | CT_INT2 | 1,2,3 |
| Cr_date | CT_DATE | mm/dd/yyyy |
| Cr_starttime | CT_STRING | hh:mm:ss.sss |
| Cr_endtime | CT_STRING | hh:mm:ss.sss |

Table 3: Signal Record

| Field | Type | Format |
| --- | --- | --- |
| Sr_usrno | CT_INT2 | 1,2,3 |
| Sr_sequence | CT_INT2 | 1,2,3 |
| Sr_time | CT_DATE | mm/dd/yyyy |
| Cr_channel1 | CT_STRING | -0.0000802 |
| Cr_channel2 | CT_STRING | -0.0000802 |
| Cr_channel3 | CT_STRING | -0.0000802 |
| Cr_channel4 | CT_STRING | -0.0000802 |

The “Info” table records the personal information of the user. Every user has an unique user number (“usrno”). It is a pre-existing table before the system running. Every connection to the server needs to check if the user exists at first.

The “Connect Record” table records the sequence and date of connection for each user. Every time the user using the system, a new record will be added in this table to log the connection date, start and end time. The user number is the same as the “Info” table, and the sequence will increase as the number of connection.

The “Signal Record” table is the primary table to record all the signal data. The first two fields are the user number and sequence corresponding to the “Connect Record” table. The remaining five fields are the same as the data in the text file: time and signal of four channels.

- Detailed implementation

- Client

Figure 5 shows the basic flow chart of the client program

Figure 5:  flow chart of the client program

The inputs of the client program need to have four parameters: hostname, port, user number and signal type. The hostname and port are used for socket connection with the server. The user number is the identity of the user. The signal type specify whether use the real EEG band or the simulated ones. “0” represents the real EEG and “1”,”2”,”3” specify different history file.

After connecting with the server, they should exchange some essential message. The client first will tell the server it wants to write data. Then it will send the user number and the date to the server. The server will look up the user number in the “Info” table. If the user exists, the server will add a new connect record to the “Connect Record” table and send the confirmation message back to the client. If the user doesn’t exist, the server will send the stop message back to client and close this connection.

Then the program will execute different procedures according to the choice of real EEG or simulated EEG. In real EEG procedure, the program will set up as a server connects with the windows program. After the connection, it will receive the data and send to server. In simulated EEG procedure, the program will read from the text file and send to the server. For the simulated EEG procedure, there is a thread to monitor the input on of the user. If “stop” is typed on the screen, the connection will end.

Every time the client sends data to the server it will wait for the confirm message and then send another data.

- Windows program

The flow char of the windows socket program is showed in Figure 6.

Figure 6:  flow chart of the windows program

The windows program will check if the text file exists. After the file created by the EEG program, it then connect to the client. If the file reaches the EOF, the program will sleep 0.5s and read again. Because the sample rate of the EEG is 128, so every 7.8 ms the data should be written to the file. But after some experiments, I found that the data don’t write in the file every 7.8 ms. It seems write to the file after several row of data are acquired. So the 0.5s sleep will not end the procedure before the EEG band really closed. The sleep can check if the EEG connection is really end or just the new data hasn’t been written yet.

- Server

The flow char of the server program is showed in Figure 7.

Figure 7:  flow chart of the server program

In the beginning, the program should log onto the Faircom database and open the tables. Then socket is set up to wait for connection. If the connection is request, the server will fork a child process to handle the request. The main process will keep wait for other connections.

After receiving the first message which specify the read or write operation. Two different procedures will be called.

In read procedure, the server will receive the user number and sequence the monitor want to query. Then it will search the “Connect Record” table. If the record doesn’t exist, the error message will send to the monitoring program. Otherwise, the server will send the start time and end time back to the monitoring program. Then it will wait for the time and range the monitoring program want to display. Search these records in the “Signal Records” table and send back. If the records reach to the end before the range reached, the program will also end the connection.

In write procedure, the server will receive the user number as I mentioned before. After query the user number and return the results, the program will add a new record in “Connect Record” table. Then the communication procedure starts. The server receives the data and put into the “Signal Record” database after some processing. For the first signal and the last signal, the program will update the start time and end time in the “Connect Record” table.

When update and write to the database, the write lock will add to the table. So multi clients can write to the database same time.

- Monitoring program

The monitoring program is a Matlab GUI program. Figure 7 shows the interface of the program.

Figure 7: Interface of the monitoring program

On the left top of the interface is the basic setting: port and IP. Under them is the user number and sequence the user wants to check. After filling with these four blanks, the “inquiry” button sets up the TCP and connects to the server.

If the user number and sequence exist, the server will return the start time and end time to static textboxes in the right frame. The user now can choose the start time and the range it want to get. The range can choose from 10 seconds to 60 seconds. Then the “update” button used to send these message to the server and receive the data.

The four channels data then will plot on the bottom axes.

Experiments and Results

- Client to Server (Simulated EEG)

The first test is to send the data to the server using the simulated EEG. The following figures show the results.

Server

Client

The client chose the user 1 and the text file 1. The last line printed on the client is the head of the text file. It shows that the client can send data to the server.

Server

Client

The figures above show that when type “stop” on the client screen, the connection will end. The server prints the end time of the connection.

Now if I print the results in the “Signal Records” database, the data sent before are in it and the “Connect Records” table shows the start and end time. The figure shows below.

Server

If the client chose the user number doesn’t exist, the server returned the “user doesn’t exist message” and close the connection.

Server

Client

Info table

The following figures shows two clients sent data to the server at the same time.

Server

Client 1

Client 2

The client 1 is user 1 and client 2 is user 2. The server displayed both their data.

- Client to Server (Real EEG)

The second serials of tests are to use the real EEG data. Figure 8 is the EEG program interface which is developed by National Chiao Tung University.

Figure 8: Mindo EEG program

Figure 9 shows that when the text file hadn’t been there, the windows program would not connect to the client.

Figure 9: windows program with no file exist

But when the “save data” option checked in the EEG program, the text file was created. Now the windows program started to send data to the client. Figure 10 shows the process.

Figure 10: windows program start sending data

The following results show that the client received the data from the windows program and sent them to the server.

Server

Client

Figure 12 compared the data in the database and the data in the EEG text file. So every data in the EEG text file have been saved in the database.

Figure 12: comparison of the database and EEG file

- Monitoring Program

Figure 12 shows the interface when the requested data were sent to the monitoring program and plotted on the interface.

Figure 12: plot data on monitoring program

In this test, the actual existed data was less than the range the user chosen.

Figure 13 shows that the existed data was larger than the range, so it displayed the whole requested range.

Figure 13: plot data on monitoring program (whole range)

Discussion

The most important thing of this system is that every signal can be saved into the Faircom database. So as I showed in figure 11. The end of the EEG text file is the same as the data in the database. Although because of some limits; the windows program can’t read the text file and send data to the client with the same speed of the EEG program. At least the data are complete.

The reason the windows program can’t synchronize with the EEG program is because I don’t exactly know how the EEG program write data to the text file. I have tried to get the file status. But they don’t changed as expect.

Another way to send the data as fast as possible is to create a circular buffer in the client program. So the main procedure in the client can receive the data and save in the circular buffer, meanwhile a thread is created to handle the data. But because in the situation, the windows program can send data very fast and there will have chance that the data are read not the complete row. That means the EEG program hasn’t written the complete data to the file and the windows program has read them. The problem is as I said above; I don’t really know how the data are written to the file by the EEG program. And the period should be 7.8 ms, but it doesn’t look so. So in this method, there will be lot of wrong data.

As a result, the method I uses now is just use the client to control the speed of the windows program. When the client is ready, tell the windows program and send the data.

Source Code

- Client

#include <stdio.h>

#include <stdlib.h>

#include <unistd.h>

#include <string.h>

#include <sys/types.h>

#include <sys/socket.h>

#include <netinet/in.h>

#include <netdb.h>

#include <arpa/inet.h>

#include <time.h>

#include <rtai_lxrt.h>

#include <pthread.h>

#define MSG_SIZE 68			// message size

int sockfd;

void error(const char *msg)

{

perror(msg);

exit(0);

}

void readfromnetwork(int socksendto)

{

//server to listen

int sockfd, newsockfd, portno;

socklen_t clilen;

struct sockaddr_in serv_addr, cli_addr;

int flag=1;

int n;

char buffer[MSG_SIZE]; //for real signal

char buffer2[MSG_SIZE]; //send to client

char buffer3[MSG_SIZE]; //wait to server

sockfd = socket(AF_INET, SOCK_STREAM, 0); // Creates socket. Connection based.

if (sockfd < 0)

error("ERROR opening socket");

// fill in fields

bzero((char *) &serv_addr, sizeof(serv_addr));

portno = 2050;	// get port number from input

serv_addr.sin_family = AF_INET;		 // symbol constant for Internet domain

serv_addr.sin_addr.s_addr = INADDR_ANY; // IP address of the machine on which

// the server is running

serv_addr.sin_port = htons(portno);	 // port number

// binds the socket to the address of the host and the port number

if (bind(sockfd, (struct sockaddr *) &serv_addr, sizeof(serv_addr)) < 0)

error("ERROR on binding");

listen(sockfd, 5);			// listen for connections

clilen = sizeof(cli_addr);	// size of structure

// blocks until a client connects to the server

newsockfd = accept(sockfd, (struct sockaddr *) &cli_addr, &clilen);

close(sockfd);

memset(buffer2,0,MSG_SIZE);

strcpy(buffer2,"go"); //go message tell client to send

while(flag==1)

{

n=read(newsockfd,buffer,MSG_SIZE); //read data

if (n<=0) //if no data end connection

{

flag=0;

close(newsockfd);

}

if (write(socksendto,buffer,MSG_SIZE) < 0) //write to server

{

error("ERROR writing to socket");

}

read(socksendto,buffer2,MSG_SIZE);//block until server reply

write(newsockfd,buffer3,MSG_SIZE); //tell client go

}

fclose(fd);

exit(0);

}

void readfromtxt(FILE *fd,int sockfd,int* stopflag)

{

char buffer[MSG_SIZE];//send

char buffer2[MSG_SIZE]; //receive

//first line is parameter

fgets(buffer,MSG_SIZE,fd);

printf("%s\n",buffer);

bzero(buffer,MSG_SIZE);

while(*stopflag==0 && fgets(buffer,MSG_SIZE,fd)!=NULL) //read every line

{

// send message

if (write(sockfd,buffer,MSG_SIZE) < 0)

error("ERROR writing to socket");

bzero(buffer,MSG_SIZE);

read(sockfd,buffer2,MSG_SIZE);//block until server reply

}

fclose(fd);

exit(0);

}

void *Thread1(void *ptr) //stop the connection is type stop

{

int *stopflag=(int *)ptr;

char buffer[MSG_SIZE];

while(*stopflag==0)

{

fgets(buffer,MSG_SIZE-1,stdin); //check the user type

if(strcmp(buffer,"stop\n")==0)

{

*stopflag=1;

}

}

pthread_exit(NULL);

}

int main(int argc, char *argv[])

{

int portno,n;

struct sockaddr_in serv_addr;

struct hostent *server;

char buffer[MSG_SIZE];

/*get date*/

time_t now;

struct tm *tm_now;

/*txt file*/

FILE *fd;

char txt_name[10];

/*thread*/

pthread_t thread1;

int stopflag;

if (argc < 5)	// not enough arguments

{

fprintf(stderr,"usage %s hostname port usrno signal_type\n", argv[0]);

exit(0);

}

portno = atoi(argv[2]);		// port # was an input.

sockfd = socket(AF_INET, SOCK_STREAM, 0); // Creates socket. Connection based.

if (sockfd < 0)

error("ERROR opening socket");

server = gethostbyname(argv[1]);  // converts hostname input (e.g. 10.3.52.255)

if (server == NULL) {

fprintf(stderr,"ERROR, no such host\n");

exit(0);

}

// fill in fields of serv_addr

bzero((char *) &serv_addr, sizeof(serv_addr));	// sets all values to zero

serv_addr.sin_family = AF_INET;		// symbol constant for Internet domain

// copy to serv_addr.sin_addr.s_addr. Function memcpy could be used instead.

bcopy((char *)server->h_addr, (char *)&serv_addr.sin_addr.s_addr, server->h_length);

serv_addr.sin_port = htons(portno);		// fill sin_port field

// establish connection to the server

if (connect(sockfd,(struct sockaddr *) &serv_addr,sizeof(serv_addr)) < 0)

error("ERROR connecting");

bzero(buffer,MSG_SIZE);

strcpy(buffer,"write");

write(sockfd,buffer,MSG_SIZE); //tell server it want write

//first message date,usrno

bzero(buffer,MSG_SIZE);

time(&now);

tm_now=localtime(&now);

strftime(buffer,MSG_SIZE,"%m/%d/%Y",tm_now); //date

strcat(buffer,",");

strcat(buffer,argv[3]); //date+usrno

n=write(sockfd,buffer,MSG_SIZE);

//check user no

bzero(buffer,MSG_SIZE);

n=read(sockfd,buffer,MSG_SIZE);

if(strcmp(buffer,"stop")==0)

{

printf("user doesn't exist\n");

close(sockfd);

exit(0);

}

//thread: monitor stop

stopflag=0;

pthread_create(&thread1,NULL,Thread1,(void*)&stopflag);

//open txt

if (atoi(argv[4])==0) //0: real signa,. 1,2,3: txt name

{

readfromnetwork(sockfd); //read from real EEG

}

else

{

sprintf(txt_name,"eeg%s.txt",argv[4]);

printf("%s\n",txt_name);

if((fd=fopen(txt_name,"r"))==NULL)

{

printf("error");

exit(1);

}

readfromtxt(fd,sockfd,&stopflag); //real from Simulated EEG

}

pthread_join(thread1,NULL);

close(sockfd);			// close socket

return 0;

}

- Windows program

#include <stdio.h>

#include <stdlib.h>

#include <unistd.h>

#include <string.h>

#include <sys/types.h>

#include <winsock2.h>

#include "stdafx.h"

#include <sys/stat.h>

#include <windows.h>

#pragma comment(lib,"ws2_32.lib") //-lws2_32

#define MSG_SIZE 68			// message size

void error(const char *msg)

{

perror(msg);

exit(0);

}

int main(int argc, char *argv[])

{

int  portno, n;

char buffer[MSG_SIZE];

char buffer2[MSG_SIZE];

char buffer3[MSG_SIZE];

char ch;

int t; //waste time

FILE *fd;

int row=0;

int column=0;

//struct stat buf;

if (argc < 2)	// not enough arguments

{

fprintf(stderr,"usage %s hostname\n", argv[0]);

exit(0);

}

WORD sockVersion=MAKEWORD(2,2);

WSADATA data;

if(WSAStartup(sockVersion,&data)!=0)

{

return 0;

}

SOCKET sockfd=socket(AF_INET,SOCK_STREAM,IPPROTO_TCP);

if (sockfd==INVALID_SOCKET)

{

printf("invalid socket!");

return 0;

}

struct sockaddr_in serAddr;

serAddr.sin_family=AF_INET;

portno = 2050;		// port # 2050

serAddr.sin_port=htons(portno);

serAddr.sin_addr.S_un.S_addr=inet_addr(argv[1]);

//open file

while((fd=fopen("EEG.txt","r"))==NULL)

{ t++;}

// establish connection to the server

if(connect(sockfd,(struct sockaddr *)&serAddr,sizeof(serAddr))==SOCKET_ERROR)

{

printf("connect error!");

closesocket(sockfd);

return 0;

}

printf("connect");

while(1)

{

if(row==0 && fgets(buffer,MSG_SIZE,fd)!=NULL) //the first row don't need to send

{

printf("aa:%s",buffer);

//stat("EEG.txt",&buf);

row++;

}

if(row !=0 )

{

if(fgets(buffer,MSG_SIZE,fd)!=NULL)

{

//stat("EEG.txt",&buf);

printf("new:%d,%s",strlen(buffer),buffer);

// send message

n=send(sockfd,buffer,MSG_SIZE,0);

recv(sockfd,buffer2,MSG_SIZE,0);

row++;

}

else //if EOF wait then check again

{

Sleep(500);

if(fgets(buffer,MSG_SIZE,fd)==NULL)

{	break;} //end send

n=send(sockfd,buffer,MSG_SIZE,0);

recv(sockfd,buffer2,MSG_SIZE,0);

row++;

}

}

}

closesocket(sockfd);			// close socket

fclose(fd);

WSACleanup();

return 0;

}

- Server

#ifdef _WIN32_WCE

#undef UNICODE

#undef _UNICODE

#define main my_main

#endif

#include "ctdbsdk.h" /* c-tree headers */

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <sys/types.h>

#include <sys/socket.h>

#include <netinet/in.h>

#include <netdb.h>

#include <signal.h>

#include <time.h>

#include <rtai_lxrt.h>

#include <sys/ioctl.h>

#define LOCK_SUPPORT /*lock*/

#ifndef ctCLIENT

#ifdef NOTFORCE

#undef LOCK_SUPPORT

#endif

#endif

#define END_OF_FILE INOT_ERR  /* INOT_ERR is ctree's 101 error. See cterrc.h */

/* Global declarations */

CTHANDLE hSession;

CTHANDLE hDatabase;

CTHANDLE hTableInfo; //user info: UserNo, name, gender, age

CTHANDLE hTableConnRecd; //record of connection:UserNo, monitor date, start time, end time

CTHANDLE hTableSignalRecd; //record of signal: UserNo, time, channel1, channel2, channel3, channel4

CTHANDLE hRecordInfo;

CTHANDLE hRecordConnRecd;

CTHANDLE hRecordSignalRecd;

/* Function declarations */

#ifdef PROTOTYPE

VOID Initialize(VOID), Define(VOID), Manage(VOID), Done(VOID);

VOID Create_Info_Table(VOID), Create_ConnRecd_Table(VOID);

VOID Create_SignalRecd_Table(VOID);

VOID Add_Info_Records(VOID), Add_ConnRecd_Records(char*,int,int);

VOID Add_SignalRecd_Records(SignalRecd_DATA);

VOID Display_Info(VOID), Display_ConnRecd();

VOID Display_SignalRecd();

VOID Delete_Records(CTHANDLE hRecord);

VOID Check_Table_Mode(CTHANDLE hTable);

VOID update_ConnRecd(CTSTRING,int,int,int);

VOID Handle_Error(CTSTRING);

#else

VOID Initialize(), Define(), Manage(), Done();

VOID Create_Info_Table(), Create_ConnRecd_Table();

VOID Create_SignalRecd_Table();

VOID Add_Info_Records(), Add_ConnRecd_Records();

VOID Add_SignalRecd_Records();

VOID Display_Info(), Display_ConnRecd();

VOID Display_SignalRecd();

VOID Delete_Records();

VOID Check_Table_Mode();

VOID update_ConnRecd()

VOID Handle_Error();

#endif

//server part

#define MSG_SIZE 68			// message size

typedef struct { //signal record data sturcture

COUNT usrno,sequence;

CTSTRING time;

CTSTRING channel1, channel2, channel3, channel4;

} SignalRecd_DATA;

void dostuff(int); 			// function prototype

void error(const char *msg)

{

perror(msg);

exit(1);

}

//set up server and wait for connection

void start_server(NINT argc,pTEXT argv[])

{

int sockfd, newsockfd, portno, pid, j = 0;

socklen_t clilen;

struct sockaddr_in serv_addr, cli_addr;

if (argc < 2)

{

fprintf(stderr,"ERROR, no port provided\n");

exit(1);

}

sockfd = socket(AF_INET, SOCK_STREAM, 0); // Creates socket. Connection based.

if (sockfd < 0)

error("ERROR opening socket");

// fill in fields

bzero((char *) &serv_addr, sizeof(serv_addr));

portno = atoi(argv[1]);	// get port number from input

serv_addr.sin_family = AF_INET;		 // symbol constant for Internet domain

serv_addr.sin_addr.s_addr = INADDR_ANY; // IP address of the machine on which

// the server is running

serv_addr.sin_port = htons(portno);	 // port number

// binds the socket to the address of the host and the port number

if (bind(sockfd, (struct sockaddr *) &serv_addr, sizeof(serv_addr)) < 0)

error("ERROR on binding");

listen(sockfd, 5);			// listen for connections

clilen = sizeof(cli_addr);	// size of structure

while (1)

{

// blocks until a client connects to the server

newsockfd = accept(sockfd, (struct sockaddr *) &cli_addr, &clilen);

if (newsockfd < 0)

error("ERROR on accept");

j++;		// counter for the connections that are established.

pid = fork();

if (pid < 0)

error("ERROR on fork");

if (pid == 0)	// child process

{

printf("Connection #%d created\n",j);

close(sockfd);			// close socket

dostuff(newsockfd);	// call function that handles communication

printf("end %d\n",j);

exit(0);

}

else			// parent

{

close(newsockfd);

signal(SIGCHLD,SIG_IGN);	// to avoid zombie problem

}

} 	// end of while

close(sockfd);

}

/*

* main()

*

* The main() function implements the concept of "init, define, manage

* and you're done..."

*/

#ifdef PROTOTYPE

NINT main (NINT argc, pTEXT argv[])

#else

NINT main (argc, argv)

NINT argc;

TEXT argv[];

#endif

{

Initialize(); //link to faircom

Define();  //create or open tables

//socket

start_server(argc,argv);

//use it and choose the function in manage to delete or display the table

//Manage();

Done();

printf("\nPress <ENTER> key to exit . . .\n");

#ifndef ctPortWINCE

getchar();

#endif

return(0);

}

/*

* Initialize()

*

* Perform the minimum requirement of logging onto the c-tree Server

*/

#ifdef PROTOTYPE

VOID Initialize(VOID)

#else

VOID Initialize()

#endif

{

CTDBRET  retval;

printf("INIT\n");

if ((retval = ctdbStartDatabaseEngine())) 				/* This function is required when you are using the Server DLL model to start the underlying Server. */

Handle_Error("Initialize(): ctdbStartDatabaseEngine()"); 	/* It does nothing in all other c-tree models */

/* allocate session handle */

if ((hSession = ctdbAllocSession(CTSESSION_CTREE)) == NULL)

Handle_Error("Initialize(): ctdbAllocSession()");

hDatabase = hSession;

/* connect to server */

printf("\tLogon to server...\n");

if (ctdbLogon(hSession, "FAIRCOMS", "ADMIN", "ADMIN"))

Handle_Error("Initialize(): ctdbLogon()");

}

/*

* Define()

*

* Open the table, if it exists. Otherwise create and open the table

*/

#ifdef PROTOTYPE

VOID Define(VOID)

#else

VOID Define()

#endif

{

//3 tables

Create_Info_Table();

Create_ConnRecd_Table();

Create_SignalRecd_Table();

if ((hRecordInfo = ctdbAllocRecord(hTableInfo)) == NULL)

Handle_Error("Add_Info_Records(): ctdbAllocRecord()");

if ((hRecordConnRecd = ctdbAllocRecord(hTableConnRecd)) == NULL)

Handle_Error("Add_ConnRecd_Records(): ctdbAllocRecord()");

if ((hRecordSignalRecd = ctdbAllocRecord(hTableSignalRecd)) == NULL)

Handle_Error("Add_SignalRecd_Records(): ctdbAllocRecord()");

}

/*

* Manage()

*

* This function performs simple record functions of add, delete and gets

*/

#ifdef PROTOTYPE

VOID Manage(VOID)

#else

VOID Manage()

#endif

{

//Delete_Records(hRecordInfo);

//Delete_Records(hRecordConnRecd);

//Delete_Records(hRecordSignalRecd);

//Add_Info_Records();

//Display_Info();

//Display_ConnRecd();

//Display_SignalRecd();

}

/*

* Done()

*

* This function handles the housekeeping of closing tables and

* freeing of associated memory

*/

#ifdef PROTOTYPE

VOID Done(VOID)

#else

VOID Done()

#endif

{

printf("DONE\n");

/* close table */

printf("\tClose table...\n");

if (ctdbCloseTable(hTableInfo))

Handle_Error("Done(): ctdbCloseTable()");

if (ctdbCloseTable(hTableConnRecd))

Handle_Error("Done(): ctdbCloseTable()");

if (ctdbCloseTable(hTableSignalRecd))

Handle_Error("Done(): ctdbCloseTable()");

/* logout */

printf("\tLogout...\n");

if (ctdbLogout(hSession))

Handle_Error("Done(): ctdbLogout()");

/* free handles */

ctdbFreeRecord(hRecordInfo);

ctdbFreeRecord(hRecordConnRecd);

ctdbFreeRecord(hRecordSignalRecd);

ctdbFreeTable(hTableInfo);

ctdbFreeTable(hTableConnRecd);

ctdbFreeTable(hTableSignalRecd);

ctdbFreeSession(hSession);

/* If you are linked to the Server DLL, then we should stop our Server at the end of the program.   */

/* It does nothing in all other c-tree models */

ctdbStopDatabaseEngine();

}

/*

* Create_Create_Info_Table_Table()

*

* Open table Info, if it exists. Otherwise create it

* along with its indices and open it

*/

#ifdef PROTOTYPE

VOID Create_Info_Table(VOID)

#else

VOID Create_Info_Table()

#endif

{

CTHANDLE pField1, pField2, pField3, pField4;

CTHANDLE pIndex;

CTHANDLE pIseg;

/* define table CustomerMaster */

printf("\table info\n");

/* allocate a table handle */

if ((hTableInfo = ctdbAllocTable(hDatabase)) == NULL)

Handle_Error("Create_info_Table(): ctdbAllocTable()");

/* open table */

if (ctdbOpenTable(hTableInfo, "UserInfo", CTOPEN_NORMAL))

{

/* define table fields */

pField1 = ctdbAddField(hTableInfo, "In_userno", CT_INT2, 2);

pField2 = ctdbAddField(hTableInfo, "In_name", CT_STRING, 47);

pField3 = ctdbAddField(hTableInfo, "In_gender", CT_FSTRING, 1);

pField4 = ctdbAddField(hTableInfo, "In_age", CT_FSTRING, 3);

if (!pField1 || !pField2 || !pField3 || !pField4 )

Handle_Error("Create_CustomerMaster_Table(): ctdbAddField()");

/* define index . usrno*/

pIndex = ctdbAddIndex(hTableInfo, "in_userno_idx", CTINDEX_PADDING, NO, NO);

pIseg = ctdbAddSegment(pIndex, pField1, CTSEG_SCHSEG);

if (!pIndex || !pIseg)

Handle_Error("Create_CustomerMaster_Table(): ctdbAddIndex()|ctdbAddSegment()");

/* create table */

if (ctdbCreateTable(hTableInfo, "UserInfo", CTCREATE_NORMAL))

Handle_Error("Create_CustomerMaster_Table(): ctdbCreateTable()");

/* open table */

if (ctdbOpenTable(hTableInfo, "UserInfo", CTOPEN_NORMAL))

Handle_Error("Create_CustomerMaster_Table(): ctdbOpenTable()");

}

else

{

Check_Table_Mode(hTableInfo);

}

}

/*

* Create_Create_ConnRecd_Table_Table()

*

* Open table ConnRecd, if it exists. Otherwise create it

* along with its indices and open it

*/

#ifdef PROTOTYPE

VOID Create_ConnRecd_Table(VOID)

#else

VOID Create_ConnRecd_Table()

#endif

{

CTHANDLE pField1, pField2, pField3, pField4,pField5;

CTHANDLE pIndex1,pIndex2;

CTHANDLE pIseg1,pIseg2,pIseg3;

/* define table CustomerMaster */

printf("\table ConnRecd\n");

/* allocate a table handle */

if ((hTableConnRecd = ctdbAllocTable(hDatabase)) == NULL)

Handle_Error("Create_ConnRecd_Table(): ctdbAllocTable()");

/* open table */

if (ctdbOpenTable(hTableConnRecd, "ConnRecd", CTOPEN_NORMAL))

{

/* define table fields */

pField1 = ctdbAddField(hTableConnRecd, "Cr_userno", CT_INT2, 2);

pField2 = ctdbAddField(hTableConnRecd, "Cr_sequence", CT_INT2, 2);

pField3 = ctdbAddField(hTableConnRecd, "Cr_date", CT_DATE, 4);

pField4 = ctdbAddField(hTableConnRecd, "Cr_starttime", CT_FSTRING, 13);

pField5 = ctdbAddField(hTableConnRecd, "Cr_endtime", CT_FSTRING, 13);

if (!pField1 || !pField2 || !pField3 || !pField4 )

Handle_Error("Create_ConnRecd_Table(): ctdbAddField()");

/* define index usrno can be dupliex*/

pIndex1 = ctdbAddIndex(hTableConnRecd, "cr_userno_idx", CTINDEX_PADDING, YES, NO);

pIseg1 = ctdbAddSegment(pIndex1, pField1, CTSEG_SCHSEG);

//usrno and sequence, can't be dupliex

pIndex2 = ctdbAddIndex(hTableConnRecd, "cr_usersequence_idx", CTINDEX_PADDING, NO, NO);

pIseg2 = ctdbAddSegment(pIndex2, pField1, CTSEG_SCHSEG);

pIseg3 = ctdbAddSegment(pIndex2, pField2, CTSEG_SCHSEG);

if (!pIndex1 || !pIndex2 || !pIseg1 || !pIseg2 || !pIseg3)

Handle_Error("Create_ConnRecd_Table(): ctdbAddIndex()|ctdbAddSegment()");

/* create table */

if (ctdbCreateTable(hTableConnRecd, "ConnRecd", CTCREATE_NORMAL))

Handle_Error("Create_ConnRecd_Table(): ctdbCreateTable()");

/* open table */

if (ctdbOpenTable(hTableConnRecd, "ConnRecd", CTOPEN_NORMAL))

Handle_Error("Create_ConnRecd_Table(): ctdbOpenTable()");

}

else

{

Check_Table_Mode(hTableConnRecd);

}

}

/*

* Create_Create_SignalRecd_Table_Table()

*

* Open table SignalRecd, if it exists. Otherwise create it

* along with its indices and open it

*/

#ifdef PROTOTYPE

VOID Create_SignalRecd_Table(VOID)

#else

VOID Create_SignalRecd_Table()

#endif

{

CTHANDLE pField1, pField2, pField3, pField4;

CTHANDLE pField5, pField6, pField7;

CTHANDLE pIndex1,pIndex2;

CTHANDLE pIseg1,pIseg2,pIseg3,pIseg4,pIseg5;

/* define table CustomerMaster */

printf("\table ConnRecd\n");

/* allocate a table handle */

if ((hTableSignalRecd = ctdbAllocTable(hDatabase)) == NULL)

Handle_Error("Create_SignalRecd_Table(): ctdbAllocTable()");

/* open table */

if (ctdbOpenTable(hTableSignalRecd, "SignalRecd", CTOPEN_NORMAL))

{

/* define table fields */

pField1 = ctdbAddField(hTableSignalRecd, "Sr_userno", CT_INT2, 2);

pField2 = ctdbAddField(hTableSignalRecd, "Sr_sequence", CT_INT2, 2);

pField3 = ctdbAddField(hTableSignalRecd, "Sr_time", CT_FSTRING, 13);

pField4 = ctdbAddField(hTableSignalRecd, "Sr_channel1", CT_STRING, 11);

pField5 = ctdbAddField(hTableSignalRecd, "Sr_channel2", CT_STRING, 11);

pField6 = ctdbAddField(hTableSignalRecd, "Sr_channel3", CT_STRING, 11);

pField7 = ctdbAddField(hTableSignalRecd, "Sr_channel4", CT_STRING, 11);

if (!pField1 || !pField2 || !pField3 || !pField4 || !pField5|| !pField6|| !pField7)

Handle_Error("Create_SignalRecd_Table(): ctdbAddField()");

/* define index  usrno and sequence can be dupliex*/

pIndex1 = ctdbAddIndex(hTableSignalRecd, "sr_usersequence_idx", CTINDEX_PADDING, YES, NO);

pIseg1 = ctdbAddSegment(pIndex1, pField1, CTSEG_SCHSEG);

pIseg2 = ctdbAddSegment(pIndex1, pField2, CTSEG_SCHSEG);

//usrno sequence and time can't be dupliex

pIndex2 = ctdbAddIndex(hTableSignalRecd, "sr_seqtime_idx", CTINDEX_PADDING, NO, NO);

pIseg3 = ctdbAddSegment(pIndex2, pField1, CTSEG_DESCENDING);//time is in descending mode

pIseg4 = ctdbAddSegment(pIndex2, pField2, CTSEG_DESCENDING);//so the display time will be in order

pIseg5 = ctdbAddSegment(pIndex2, pField3, CTSEG_DESCENDING);

if (!pIndex1 || !pIseg1 || !pIseg2 ||!pIndex2|| !pIseg3|| !pIseg4|| !pIseg5)

Handle_Error("Create_ConnRecd_Table(): ctdbAddIndex()|ctdbAddSegment()");

/* create table */

if (ctdbCreateTable(hTableSignalRecd, "SignalRecd", CTCREATE_NORMAL))

Handle_Error("Create_SignalRecd_Table(): ctdbCreateTable()");

/* open table */

if (ctdbOpenTable(hTableSignalRecd, "SignalRecd", CTOPEN_NORMAL))

Handle_Error("Create_SignalRecd_Table(): ctdbOpenTable()");

}

else

{

Check_Table_Mode(hTableSignalRecd);

}

}

/*

* Check_Table_Mode()

*

* Check if existing table has transaction processing flag enabled.

* If a table is under transaction processing control, modify the

* table mode to disable transaction processing

*/

#ifdef PROTOTYPE

VOID Check_Table_Mode(CTHANDLE hTable)

#else

VOID Check_Table_Mode(hTable)

CTHANDLE hTable;

#endif

{

CTCREATE_MODE mode;

/* get table create mode */

mode = ctdbGetTableCreateMode(hTable);

/* check if table is under transaction processing control */

if ((mode & CTCREATE_TRNLOG))

{

/* change file mode to disable transaction processing */

mode ^= CTCREATE_TRNLOG;

if (ctdbUpdateCreateMode(hTable, mode) != CTDBRET_OK)

Handle_Error("Check_Table_Mode(); ctdbUpdateCreateMode");

}

}

/*

* Delete_Records()

*

* This function deletes all the records in the table

*/

#ifdef PROTOTYPE

VOID Delete_Records(CTHANDLE hRecord)

#else

VOID Delete_Records(hRecord)

CTHANDLE hRecord;

#endif

{

CTDBRET  retval;

CTBOOL   empty;

printf("\tDelete records...\n");

/* enable session-wide lock flag */

if (ctdbLock(hSession, CTLOCK_WRITE_BLOCK))

Handle_Error("Delete_Records(): ctdbLock()");

empty = NO;

retval = ctdbFirstRecord(hRecord);

if (retval != CTDBRET_OK)

{

if (retval == END_OF_FILE)

empty = YES;

else

Handle_Error("Delete_Records(): ctdbFirstRecord()");

}

while (empty == NO) /* while table is not empty */

{

/* delete record */

if (ctdbDeleteRecord(hRecord))

Handle_Error("Delete_Records(): ctdbDeleteRecord()");

/* read next record */

retval = ctdbNextRecord(hRecord);

if (retval != CTDBRET_OK)

{

if (retval == END_OF_FILE)

empty = YES;

else

Handle_Error("Delete_Records(): ctdbNextRecord()");

}

}

if (ctdbUnlock(hSession))

Handle_Error("Delete_Records(): ctdbLock()");

}

/*

* Add_Info_Records()

*

* This function adds Info_Records to a table in the database

*/

typedef struct {

COUNT usrno;

CTSTRING name,gender,age;

} INFO_DATA;

INFO_DATA data1[]=

{{1, "aaa", "F", "22"},{2, "bb", "F", "69"},{3, "cc", "M", "919"},{4, "dd", "F", "22"},

{5, "ee", "M", "30"},{6, "ff", "M", "22"},{7, "gg", "F", "22"},{8, "hh", "F", "27"}

}; //pre-exist records

#ifdef PROTOTYPE

VOID Add_Info_Records(VOID)

#else

VOID Add_Info_Records()

#endif

{

CTDBRET  retval;

CTSIGNED i;

CTSIGNED nRecords = sizeof(data1) / sizeof(INFO_DATA);

printf("\tAdd records...\n");

/* add data to table */

//Delete_Records(hRecordInfo);

for (i=0;i<nRecords;i++)

{

/* clear record buffer */

ctdbClearRecord(hRecordInfo);

retval = 0;

/* populate record buffer with data */

retval |= ctdbSetFieldAsSigned(hRecordInfo, 0, data1[i].usrno);

retval |= ctdbSetFieldAsString(hRecordInfo, 1, data1[i].name);

retval |= ctdbSetFieldAsString(hRecordInfo, 2, data1[i].gender);

retval |= ctdbSetFieldAsString(hRecordInfo, 3, data1[i].age);

if (retval)

Handle_Error("Add_hRecordInfo(): ctdbSetFieldAsString()");

/* add record */

if (ctdbWriteRecord(hRecordInfo))

printf("Add_hRecordInfo(): ctdbWriteRecord()");

//Handle_Error("Add_hRecordInfo(): ctdbWriteRecord()");

}

}

/*

* Add_ConnRecd_Records()

*

* This function adds ConnRecd records to a table in the database

*/

typedef struct {

COUNT usrno,sequence;

CTSTRING date,starttime,endtime;

} ConnRecd_DATA;

#ifdef PROTOTYPE

VOID Add_ConnRecd_Records(char* date_string ,int usrno ,int sequence)

#else

VOID Add_ConnRecd_Records()

#endif

{

CTDBRET  retval;

CTDATE   date;

ConnRecd_DATA data2;

data2.usrno=usrno;

data2.sequence=sequence;

data2.starttime=""; //at first no start and end time

data2.endtime="";

printf("\tAdd records...\n");

/* add data to table */

/* clear record buffer */

ctdbClearRecord(hRecordConnRecd);

retval = 0;

retval |= ctdbStringToDate(date_string, CTDATE_MDCY ,&date); //convert to data type

/* populate record buffer with data */

retval |= ctdbSetFieldAsSigned(hRecordConnRecd, 0, data2.usrno);

retval |= ctdbSetFieldAsSigned(hRecordConnRecd, 1,data2.sequence);

retval |= ctdbSetFieldAsDate(hRecordConnRecd, 2, date);

retval |= ctdbSetFieldAsString(hRecordConnRecd, 3, data2.starttime);

retval |= ctdbSetFieldAsString(hRecordConnRecd, 4, data2.endtime);

if (retval)

Handle_Error("Add_hRecordConnRecd(): ctdbSetFieldAsString()");

ctdbLock(hTableConnRecd, CTLOCK_WRITE_BLOCK); //tablelock

if (ctdbWriteRecord(hRecordConnRecd))

printf("Add_hRecordConnRecd(): ctdbWriteRecord()");

ctdbUnlockTable(hTableConnRecd);

}

/*

* Add_SignalRecd_Records()

*

* This function adds signal records to a table in the database

*/

#ifdef PROTOTYPE

VOID Add_SignalRecd_Records(SignalRecd_DATA data3)

#else

VOID Add_SignalRecd_Records()

#endif

{

CTDBRET  retval;

printf("%d %d %s %s %s %s %s\n",data3.usrno,data3.sequence,data3.time,data3.channel1,

data3.channel2,data3.channel3,data3.channel4);

/* add data to table */

/* clear record buffer */

ctdbClearRecord(hRecordSignalRecd);

retval = 0;

/* populate record buffer with data */

retval |= ctdbSetFieldAsSigned(hRecordSignalRecd, 0, data3.usrno);

retval |= ctdbSetFieldAsSigned(hRecordSignalRecd, 1, data3.sequence);

retval |= ctdbSetFieldAsString(hRecordSignalRecd, 2, data3.time);

retval |= ctdbSetFieldAsString(hRecordSignalRecd, 3, data3.channel1);

retval |= ctdbSetFieldAsString(hRecordSignalRecd, 4, data3.channel2);

retval |= ctdbSetFieldAsString(hRecordSignalRecd, 5, data3.channel3);

retval |= ctdbSetFieldAsString(hRecordSignalRecd, 6, data3.channel4);

if (retval)

Handle_Error("Add_hRecordSignalRecd(): ctdbSetFieldAsString()");

//lock and write

ctdbLock(hTableSignalRecd, CTLOCK_WRITE_BLOCK);

ctdbWriteRecord(hRecordSignalRecd);

ctdbUnlockTable(hTableSignalRecd);

}

/*

* Display_Info()

*

* This function displays the contents of a table. ctdbFirstRecord() and

* ctdbNextRecord() fetch the record. Then each field is parsed and displayed

*/

#ifdef PROTOTYPE

VOID Display_Info(VOID)

#else

VOID Display_Info()

#endif

{

CTDBRET  retval;

CTSIGNED    usrno;

TEXT     name[47+1];

TEXT     gender[1+1];

TEXT     age[3+1];

printf("\tDisplay records...");

/* read first record */

retval = ctdbFirstRecord(hRecordInfo);

if (retval != CTDBRET_OK)

Handle_Error("Display_hRecordInfo(): ctdbFirstRecord()");

while (retval != END_OF_FILE)

{

retval = 0;

retval |= ctdbGetFieldAsSigned(hRecordInfo, 0, &usrno);

retval |= ctdbGetFieldAsString(hRecordInfo, 1, name, sizeof(name));

retval |= ctdbGetFieldAsString(hRecordInfo, 2, gender, sizeof(gender));

retval |= ctdbGetFieldAsString(hRecordInfo, 3, age, sizeof(age));

if (retval)

Handle_Error("Display_hRecordInfo(): ctdbGetFieldAsString()");

printf("\n\t\t%lu %s %s %s\n",usrno, name,gender,age);

/* read next record */

retval = ctdbNextRecord(hRecordInfo);

if (retval == END_OF_FILE)

break;   /* reached end of file */

if (retval != CTDBRET_OK)

Handle_Error("Display_hRecordInfo(): ctdbNextRecord()");

}

}

/*

* Display_ConnRecd()

*

* This function displays the contents of a table. ctdbFirstRecord() and

* ctdbNextRecord() fetch the record. Then each field is parsed and displayed

*/

#ifdef PROTOTYPE

VOID Display_ConnRecd()

#else

VOID Display_ConnRecd()

#endif

{

CTDBRET  retval;

CTSIGNED    usrno;

CTSIGNED    sequence;

CTDATE    date;

TEXT    date_s[10+1];

TEXT     starttime[13+1];

TEXT     endtime[13+1];

printf("\tDisplay records...");

/* read first record */

retval = ctdbFirstRecord(hRecordConnRecd);

if (retval != CTDBRET_OK)

Handle_Error("Display_hRecordConnRecd(): ctdbFirstRecord()");

while (retval != END_OF_FILE)

{

retval = 0;

retval |= ctdbGetFieldAsSigned(hRecordConnRecd, 0, &usrno);

retval |= ctdbGetFieldAsSigned(hRecordConnRecd, 1, &sequence);

retval |= ctdbGetFieldAsDate(hRecordConnRecd, 2, &date);

retval |= ctdbGetFieldAsString(hRecordConnRecd, 3, starttime, sizeof(starttime));

retval |= ctdbGetFieldAsString(hRecordConnRecd, 4, endtime, sizeof(endtime));

/* convert date to vision type*/

retval |= ctdbDateToString(date, CTDATE_MDCY, date_s,sizeof(date_s));

if (retval)

Handle_Error("Display_hRecordConnRecd(): ctdbGetFieldAsString()");

printf("\n\t\t%lu %lu %s %s %s\n",usrno,sequence,date_s,starttime,endtime);

/* read next record */

retval = ctdbNextRecord(hRecordConnRecd);

if (retval == END_OF_FILE)

break;   /* reached end of file */

if (retval != CTDBRET_OK)

Handle_Error("Display_hRecordConnRecd(): ctdbNextRecord()");

}

}

/*

* Display_SignalRecd();()

*

* This function displays the contents of a table. ctdbFirstRecord() and

* ctdbNextRecord() fetch the record. Then each field is parsed and displayed

*/

#ifdef PROTOTYPE

VOID Display_SignalRecd()

#else

VOID Display_SignalRecd()

#endif

{

CTDBRET  retval=CTDBRET_OK;

CTSIGNED    usrno;

CTSIGNED    sequence;

TEXT     time[13+1];

TEXT     channel1[11+1];

TEXT     channel2[11+1];

TEXT     channel3[11+1];

TEXT     channel4[11+1];

printf("\tDisplay records...");

/* read first record */

retval = ctdbFirstRecord(hRecordSignalRecd);

if (retval != CTDBRET_OK)

Handle_Error("Display_hRecordSignalRecd(): ctdbFirstRecord()");

while (retval != END_OF_FILE)

{

retval = 0;

retval |= ctdbGetFieldAsSigned(hRecordSignalRecd, 0, &usrno);

retval |= ctdbGetFieldAsSigned(hRecordSignalRecd, 1, &sequence);

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 2, time, sizeof(time));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 3, channel1,sizeof(channel1));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 4, channel2,sizeof(channel2));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 5, channel3,sizeof(channel3));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 6, channel4,sizeof(channel4));

if (retval)

Handle_Error("Display_hRecordSignalRecd(): ctdbGetFieldAsString()");

printf("\n\t\t%lu %lu %s %s %s %s %s\n",usrno,sequence,time,channel1,channel2,channel3,channel4);

/* read next record */

retval = ctdbNextRecord(hRecordSignalRecd);

if (retval == END_OF_FILE)

break;   /* reached end of file */

if (retval != CTDBRET_OK)

Handle_Error("Display_hRecordSignalRecd(): ctdbNextRecord()");

}

}

/*

* Handle_Error()

*

* This function is a common bailout routine. It displays an error message

* allowing the user to acknowledge before terminating the application

*/

#ifdef PROTOTYPE

VOID Handle_Error(CTSTRING errmsg)

#else

VOID Handle_Error(errmsg)

CTSTRING errmsg;

#endif

{

printf("\nERROR: [%d] - %s \n", ctdbGetError(hSession), errmsg);

printf("*** Execution aborted *** \nPress <ENTER> key to exit...");

ctdbLogout(hSession);

ctdbFreeRecord(hRecordInfo);

ctdbFreeRecord(hRecordConnRecd);

ctdbFreeRecord(hRecordSignalRecd);

ctdbFreeTable(hTableInfo);

ctdbFreeTable(hTableConnRecd);

ctdbFreeTable(hTableSignalRecd);

ctdbFreeSession(hSession);

getchar();

exit(1);

}

/*updat the connect start time and end time. at the begining and end of the transfer*/

void update_ConnRecd(CTSTRING time,int fieldNum,int usrno,int sequence)

{

printf("%d,%d,%s\n",usrno,sequence,time);

if(ctdbClearRecord(hRecordConnRecd))//clear record handle

Handle_Error("update");

if(ctdbSetDefaultIndex(hRecordConnRecd,1)) //set index

Handle_Error("check_usrno():ctdbSetDefaultIndex");

if(ctdbSetFieldAsSigned(hRecordConnRecd,0,usrno))

Handle_Error("check_usrno():ctdbSetFieldAsSigned");

if(ctdbSetFieldAsSigned(hRecordConnRecd,1,sequence))

Handle_Error("check_usrno():ctdbSetFieldAsSigned");

if(ctdbFindRecord(hRecordConnRecd,CTFIND_EQ)!=CTDBRET_OK) //quire

printf("record not found\n");

else //find record

{

ctdbLockRecord(hRecordConnRecd, CTLOCK_WRITE_BLOCK);

if(ctdbSetFieldAsString(hRecordConnRecd,fieldNum,time))

Handle_Error("update:set");

if(ctdbWriteRecord(hRecordConnRecd)!=CTDBRET_OK)//update

Handle_Error("update:write");

ctdbUnlockRecord(hRecordConnRecd);

}

}

/*check if usrno exist in info table. call at the begining of the connection*/

int check_usrno(int usrno)

{

int usr_check=0; //check if usrno exist. 0: no, 1: yes

if(ctdbClearRecord(hRecordInfo))

Handle_Error("check_usrno():ctdbClearRecord");

if(ctdbSetFieldAsSigned(hRecordInfo,0,usrno))

Handle_Error("check_usrno():ctdbSetFieldAsSigned");

if(ctdbFindRecord(hRecordInfo,CTFIND_EQ)!=CTDBRET_OK)//find

printf("record not found\n");

else

usr_check=1;  //found record

return usr_check;

}

/*check which sequence is this time*/

int check_sequence(int usrno)

{

CTDBRET retval;

CTSIGNED t_sequence; //sequence of usrno in table connect record

int sequence=0; //

CTBOOL old_usr; //there are records related to this user

if(ctdbClearRecord(hRecordConnRecd))

Handle_Error("check_sequence():ctdbClearRecord");

if(ctdbSetFieldAsSigned(hRecordConnRecd,0,usrno))

Handle_Error("check_sequence():ctdbSetFieldAsSigned");

if(ctdbRecordSetOn(hRecordConnRecd,2)) //create set. because one user can have more

//than one connection record

Handle_Error("check_sequence():ctdbRecordSetOn");

else

{

if(ctdbFirstRecord(hRecordConnRecd))

{

sequence=0;  //found record;

}

else

{

old_usr=YES;

while(old_usr)

{

sequence++; //count sequence

if(ctdbGetFieldAsSigned(hRecordConnRecd,1,&t_sequence))

Handle_Error("check_sequence():ctdbGetFieldAsSigned");

//read next record

retval=ctdbNextRecord(hRecordConnRecd);

if(retval!=CTDBRET_OK)

{

if(retval==END_OF_FILE)

old_usr=NO;

else

Handle_Error("check_sequence():ctdbNextRecord");

}

}

ctdbRecordSetOff(hRecordConnRecd);

}

}

return sequence;

}

/*send signal to monitoring program*/

VOID Send_SignalRecd(CTHANDLE hRecordSignalRecd,int sendsock,int send_num,int s_usrno,int s_sequence)

{

CTDBRET  retval=CTDBRET_OK;

CTSIGNED    usrno;

CTSIGNED    sequence;

TEXT     time[13+1];

TEXT     channel1[11+1];

TEXT     channel2[11+1];

TEXT     channel3[11+1];

TEXT     channel4[11+1];

char buffer[MSG_SIZE]; //data

char buffer2[MSG_SIZE]; //receive confirm message

char buffer3[MSG_SIZE]; //send confirm message

int n,i=0;

memset(buffer3,0,MSG_SIZE);

strcpy(buffer3,"y"); //confirm message

while (i<send_num) //if not reach the data within the time range

{

ctdbLock(hTableSignalRecd, CTLOCK_SUSPEND); //suspend lock

retval = 0;

retval |= ctdbGetFieldAsSigned(hRecordSignalRecd, 0, &usrno);

retval |= ctdbGetFieldAsSigned(hRecordSignalRecd, 1, &sequence);

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 2, time, sizeof(time));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 3, channel1,sizeof(channel1));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 4, channel2,sizeof(channel2));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 5, channel3,sizeof(channel3));

retval |= ctdbGetFieldAsString(hRecordSignalRecd, 6, channel4,sizeof(channel4));

ctdbLock(hTableSignalRecd, CTLOCK_WRITE_BLOCK);

if (retval)

Handle_Error("Display_hRecordSignalRecd(): ctdbGetFieldAsString()");

//determine usrno sequence

if(usrno!=s_usrno || sequence!= s_sequence)

{

memset(buffer3,0,MSG_SIZE);

strcpy(buffer3,"n");

n=write(sendsock,buffer3,MSG_SIZE);

printf("usrno end\n");

break;

}

else

{

n=write(sendsock,buffer3,MSG_SIZE); //start send data

read(sendsock,buffer2,MSG_SIZE);//block until server reply

}

memset(buffer,0,MSG_SIZE);

strcpy(buffer,time);

n=write(sendsock,buffer,MSG_SIZE); //send time

read(sendsock,buffer2,MSG_SIZE);//block until server reply

memset(buffer,0,MSG_SIZE);

sprintf(buffer,"%s %s %s %s",channel1,channel2,channel3,channel4);

n=write(sendsock,buffer,MSG_SIZE);

read(sendsock,buffer2,MSG_SIZE);//block until server reply

printf("\n\t\t%lu %lu %s %s %s %s %s\n",usrno,sequence,time,channel1,channel2,channel3,channel4);

/* read prev record */

retval = ctdbPrevRecord(hRecordSignalRecd);//because the index is decesding mode

if (retval != CTDBRET_OK)

{

memset(buffer3,0,MSG_SIZE);

strcpy(buffer3,"n");

n=write(sendsock,buffer3,MSG_SIZE);

//Handle_Error("Send_SignalRecd(): ctdbPrevRecord()");

}

i++;

}

memset(buffer3,0,MSG_SIZE);

strcpy(buffer3,"n");

n=write(sendsock,buffer3,MSG_SIZE);

}

/*read data option*/

void readdata(int sock,int usrno,int sequence)

{

char buffer[MSG_SIZE];

TEXT starttime[13+1];

TEXT endtime[13+1];

TEXT send_start[13+1];

int send_num;

//read connect record;

ctdbLock(hTableConnRecd, CTLOCK_SUSPEND);

if(ctdbClearRecord(hRecordConnRecd))

Handle_Error("update");

if(ctdbSetDefaultIndex(hRecordConnRecd,1))

Handle_Error("readdata():ctdbSetDefaultIndex");

if(ctdbSetFieldAsSigned(hRecordConnRecd,0,usrno))

Handle_Error("readdata():ctdbSetFieldAsSigned");

if(ctdbSetFieldAsSigned(hRecordConnRecd,1,sequence))

Handle_Error("readdata():ctdbSetFieldAsSigned");

if(ctdbFindRecord(hRecordConnRecd,CTFIND_EQ)!=CTDBRET_OK)

{

memset(buffer,0,MSG_SIZE);

strcpy(buffer,"n");

printf("no record\n");  //no record;

write(sock,buffer,MSG_SIZE);

}

else//record exist

{

ctdbGetFieldAsString(hRecordConnRecd, 3, starttime, sizeof(starttime));

ctdbGetFieldAsString(hRecordConnRecd, 4, endtime, sizeof(endtime));

//send back the start and end time

memset(buffer,0,MSG_SIZE);

sprintf(buffer,"%s,%s",starttime,endtime);

write(sock,buffer,MSG_SIZE);

memset(buffer,0,MSG_SIZE);

read(sock,buffer,MSG_SIZE); //read the time and range;

//process the string

sscanf(buffer,"%s%d",send_start,&send_num);

printf("%s,%d,%d,%d\n",send_start,strlen(send_start),sizeof(send_start),send_num);

}

ctdbLock(hTableConnRecd, CTLOCK_WRITE_BLOCK);

//read signal

ctdbLock(hTableSignalRecd, CTLOCK_SUSPEND);

if(ctdbClearRecord(hRecordSignalRecd))

Handle_Error("readdata():ctdbClearRecord");

if(ctdbSetDefaultIndex(hRecordSignalRecd,1))

Handle_Error("readdata():ctdbSetDefaultIndex");

if(ctdbSetFieldAsSigned(hRecordSignalRecd,0,usrno))

Handle_Error("readdata():ctdbSetFieldAsSigned");

if(ctdbSetFieldAsSigned(hRecordSignalRecd,1,sequence))

Handle_Error("readdata():ctdbSetFieldAsSigned");

if(ctdbSetFieldAsString(hRecordSignalRecd,2,send_start))

Handle_Error("readdata():ctdbSetFieldAsStringhalo");

if(ctdbFindRecord(hRecordSignalRecd,CTFIND_GE)!=CTDBRET_OK)

{ //no record

ctdbLock(hTableSignalRecd, CTLOCK_WRITE_BLOCK);

printf("record not found\n");

memset(buffer,0,MSG_SIZE);

strcpy(buffer,"n");

printf("no record\n");  //no record;

write(sock,buffer,MSG_SIZE);

}

else

{

ctdbLock(hTableSignalRecd, CTLOCK_WRITE_BLOCK);

memset(buffer,0,MSG_SIZE);

strcpy(buffer,"y");

write(sock,buffer,MSG_SIZE);//confirm the monitoring program

Send_SignalRecd(hRecordSignalRecd,sock,send_num,usrno,sequence); //send signal

}

}

/*write data function*//

void writedata(int sock)

{

int flag=1;

int n,i=0;

char buffer[MSG_SIZE]; //buffer receive data

char buffer2[MSG_SIZE]; //buffer send confirm message

char *recv_date;//start monitor date

int recv_usrno; //user no

int sequence=0;

SignalRecd_DATA data3;

char c1[MSG_SIZE],c2[MSG_SIZE],c3[MSG_SIZE],c4[MSG_SIZE],c5[MSG_SIZE]; //split string

//read the first signal

bzero(buffer,MSG_SIZE);

read(sock,buffer,MSG_SIZE);

bzero(buffer2,MSG_SIZE);

strcpy(buffer2,buffer);

recv_date=strtok(buffer2,",");//"," as the demarcation

recv_usrno=atoi(strtok(NULL,","));

if(check_usrno(recv_usrno)==0)//no userno

{

memset(buffer,0,MSG_SIZE);

strcat(buffer,"stop");

write(sock,buffer,strlen(buffer)); //stop end connection

printf("no user\n");

flag=0;

}

else

{

memset(buffer,0,MSG_SIZE);

strcat(buffer,"go");

n=write(sock,buffer,MSG_SIZE); //start transfer

sequence=check_sequence(recv_usrno);

printf("sequence:%d\n",sequence);

Add_ConnRecd_Records(recv_date, recv_usrno,sequence+1);

//Display_ConnRecd();

data3.usrno=recv_usrno;

data3.sequence=sequence+1;

memset(buffer2,0,MSG_SIZE);

strcpy(buffer2,"go");

while(flag==1)

{

memset(buffer,0,MSG_SIZE);

n=read(sock,buffer,MSG_SIZE);

if(n<=0) //stop connection

{

flag=0;

update_ConnRecd(data3.time,4,data3.usrno,data3.sequence);//update time according to

//the last data

break;

}

write(sock,buffer2,MSG_SIZE); //tell client go

sscanf(buffer,"%s%s%s%s%s",c1,c2,c3,c4,c5);//deal with the buffer

data3.time=c1;

data3.channel1=c2;

data3.channel2=c3;

data3.channel3=c4;

data3.channel4=c5;

Add_SignalRecd_Records(data3);//add data

if(i==0)

{

update_ConnRecd(data3.time,3,data3.usrno,data3.sequence);//update time according to

//the first data

}

i++;

}

}

}

void dostuff (int sock)

{

char buffer[MSG_SIZE];

char temp[MSG_SIZE];

int usrno,sequence;

ioctl(sock,FIONBIO,0);

int n;

bzero(buffer,MSG_SIZE);

n=read(sock,buffer,MSG_SIZE); //read the option: read or write

sscanf(buffer,"%s%d%d",temp,&usrno,&sequence);

printf("%s,%d,%d\n",temp,usrno,sequence);

if (strcmp(buffer,"write")==0)

{

writedata(sock); //write function

}

else

{

readdata(sock,usrno,sequence); //read funtion

}

}

- Monitoring program

function varargout = gui(varargin)

% Begin initialization code - DO NOT EDIT

gui_Singleton = 1;

gui_State = struct('gui_Name',       mfilename, ...

'gui_Singleton',  gui_Singleton, ...

'gui_OpeningFcn', @gui_OpeningFcn, ...

'gui_OutputFcn',  @gui_OutputFcn, ...

'gui_LayoutFcn',  [] , ...

'gui_Callback',   []);

if nargin && ischar(varargin{1})

gui_State.gui_Callback = str2func(varargin{1});

end

if nargout

[varargout{1:nargout}] = gui_mainfcn(gui_State, varargin{:});

else

gui_mainfcn(gui_State, varargin{:});

end

% End initialization code - DO NOT EDIT

% --- Executes just before gui is made visible.

function gui_OpeningFcn(hObject, eventdata, handles, varargin)

% Choose default command line output for gui

handles.output = hObject;

% Update handles structure

guidata(hObject, handles);

% --- Outputs from this function are returned to the command line.

function varargout = gui_OutputFcn(hObject, eventdata, handles)

varargout{1} = handles.output;

function T_usrno_Callback(hObject, eventdata, handles) %user no text box

usrno=str2double(get(hObject,'String')); %get user no

if isnan(usrno)

set(hObject, 'String', 0);

errordlg('Input must be a number','Error');

end

handles.metricdata.T_usrno = usrno;

guidata(hObject,handles)

% --- Executes during object creation, after setting all properties.

function T_usrno_CreateFcn(hObject, eventdata, handles)

if ispc && isequal(get(hObject,'BackgroundColor'), get(0,'defaultUicontrolBackgroundColor'))

set(hObject,'BackgroundColor','white');

end

function T_seq_Callback(hObject, eventdata, handles) %sequence no text box

sequence=str2double(get(hObject,'String'));%get sequence no

if isnan(sequence)

set(hObject, 'String', 0);

errordlg('Input must be a number','Error');

end

handles.metricdata.T_seq = sequence;

guidata(hObject,handles)

% --- Executes during object creation, after setting all properties.

function T_seq_CreateFcn(hObject, eventdata, handles)

if ispc && isequal(get(hObject,'BackgroundColor'), get(0,'defaultUicontrolBackgroundColor'))

set(hObject,'BackgroundColor','white');

end

% --- Executes on button press in B_Inquiry.

function B_Inquiry_Callback(hObject, eventdata, handles)

msg_size=68;

%get usrno sequence

sequence=handles.metricdata.T_seq;%sequencd

usrno=handles.metricdata.T_usrno;%user number

ip=handles.metricdata.T_ip; %ip

port=handles.metricdata.T_port;%port

%%%%set tcp%%%%%%%%%%%%5

tcpipClient=tcpip(ip,port,'NetworkRole','Client');

set(tcpipClient,'InputBufferSize',msg_size);

set(tcpipClient,'Timeout',30);

fopen(tcpipClient);

%first socket message

sendtobuffer=['read',' ',int2str(usrno),' ',int2str(sequence)];  %"read"+usrno+sequence

fwrite(tcpipClient,sendtobuffer,'char'); %send

%%%%%%%%%%receive time%%%%%%%%%%%%

recv=char(fread(tcpipClient,msg_size,'char')); %wait confirm

if (recv(1)~='n')

str = deblank(recv');

str = regexp(str, ',','split'); %split the receving data

starttime=char(str(1)); %start time

endtime=char(str(2));%end time

set(handles.L_starttime,'string',starttime);

set(handles.L_endtime,'string',endtime);

set(handles.T_starttime,'string',starttime); %default starttime is the starttime of signal

else

errordlg('no record','Error');

end

handles.tcp=tcpipClient;

guidata(hObject,handles)

function T_starttime_Callback(hObject, eventdata, handles) %start time text box

starttime=get(hObject,'String');

handles.metricdata.T_starttime = starttime;

guidata(hObject,handles)

% --- Executes during object creation, after setting all properties.

function T_starttime_CreateFcn(hObject, eventdata, handles)

if ispc && isequal(get(hObject,'BackgroundColor'), get(0,'defaultUicontrolBackgroundColor'))

set(hObject,'BackgroundColor','white');

end

% --- Executes on button press in B_update.

function B_update_Callback(hObject, eventdata, handles) %update button ,

%choose the time

%and receive data

%send signal

tcpipClient=handles.tcp;

i=1; %count receive data number

samplerate=128;

msg_size=68; %message size

range=str2double(handles.metricdata.P_range); %time range

xaxis=samplerate*range; %get data number 128 samples per second

%x=0:1/samplerate:range; %x range comment if not dynamic plot

rawTime=zeros(xaxis,msg_size); %receiving data

rawData=zeros(xaxis,4);

%%%%%%send starttime and range%%%%%%%%%%

starttime=get(handles.T_starttime,'string'); %starttime

num=int2str(xaxis); %data number

fwrite(tcpipClient,[starttime,' ',num],'char');

%%%%%%%start receive data%%%%%%%%%

recv=char(fread(tcpipClient,msg_size,'char'));

if(recv(1)=='y')

while(1)

recv=char(fread(tcpipClient,msg_size,'char')); %if still has data

if (recv(1)=='n')

break; %transfer end

end

fwrite(tcpipClient,'go','char'); %reply to server

rawTime(i,:)=fread(tcpipClient,msg_size,'char'); %time filed

fwrite(tcpipClient,'go','char');

temp=char(fread(tcpipClient,msg_size,'char')); %four channel fields

rawData(i,:)=sscanf(temp,'%f')'; %raw data

%drawline_d( handles,i,x,range,rawData ); %this is dynamic plot. t

%comment it and uncomment the drawline function

%below if not use

%dynamic

fwrite(tcpipClient,'go','char');

i=i+1;

end

end

drawline( handles,i-1,rawData,rawTime,samplerate ); %use it if not dynamic

guidata(hObject,handles)

fclose(tcpipClient);

% --- Executes on selection change in P_range.

function P_range_Callback(hObject, eventdata, handles) %choose range

contents= cellstr(get(hObject,'String'));

range=contents{get(hObject,'Value')};

handles.metricdata.P_range = range;

guidata(hObject,handles)

% --- Executes during object creation, after setting all properties.

function P_range_CreateFcn(hObject, eventdata, handles)

if ispc && isequal(get(hObject,'BackgroundColor'), get(0,'defaultUicontrolBackgroundColor'))

set(hObject,'BackgroundColor','white');

end

contents= cellstr(get(hObject,'String'));%get intial range, which is 10s

range=contents{get(hObject,'Value')};

handles.metricdata.P_range = range;

guidata(hObject,handles)

function T_ip_Callback(hObject, eventdata, handles)

ip=get(hObject,'String');

handles.metricdata.T_ip = ip;

guidata(hObject,handles)

% --- Executes during object creation, after setting all properties.

function T_ip_CreateFcn(hObject, eventdata, handles)

if ispc && isequal(get(hObject,'BackgroundColor'), get(0,'defaultUicontrolBackgroundColor'))

set(hObject,'BackgroundColor','white');

end

function T_port_Callback(hObject, eventdata, handles) %port text box

port=str2double(get(hObject,'String'));

if isnan(port)

set(hObject, 'String', '2000');

errordlg('Input must be an port','Error');

end

handles.metricdata.T_port = port;

guidata(hObject,handles)

% --- Executes during object creation, after setting all properties.

function T_port_CreateFcn(hObject, eventdata, handles)

if ispc && isequal(get(hObject,'BackgroundColor'), get(0,'defaultUicontrolBackgroundColor'))

set(hObject,'BackgroundColor','white');

end

port=str2double(get(hObject,'String'));

handles.metricdata.T_port = port;

guidata(hObject,handles)

function drawline( handles,xaxis,rawData,rawTime,samplerate)

range=xaxis/samplerate; % time range

x=0:1/samplerate:range; %x range

x=x(2:end);  %skip 0

range_x=0:range/5:range; %x axis range

axes(handles.ch1);%channel 1

plot(x,rawData(1:xaxis,1),'-');

xlabel('time'); %set lael

set(gca,'XTick',range_x);

range_time=round(range_x*samplerate);

set(gca,'XTickLabel',[char(rawTime(1,:));char(rawTime(range_time(2:end),:)) ]); %label time

axes(handles.ch2);%channel 2

plot(x,rawData(1:xaxis,2),'-');

xlabel('time');

set(gca,'XTick',range_x);

range_time=round(range_x*samplerate);

set(gca,'XTickLabel',[char(rawTime(1,:));char(rawTime(range_time(2:end),:)) ]);

axes(handles.ch3);%channel 3

plot(x,rawData(1:xaxis,3),'-');

xlabel('time');

set(gca,'XTick',range_x);

range_time=round(range_x*samplerate);

set(gca,'XTickLabel',[char(rawTime(1,:));char(rawTime(range_time(2:end),:)) ]);

axes(handles.ch4);%channel 4

plot(x,rawData(1:xaxis,4),'-');

xlabel('time');

set(gca,'XTick',range_x);

range_time=round(range_x*samplerate);

set(gca,'XTickLabel',[char(rawTime(1,:));char(rawTime(range_time(2:end),:)) ]);

end

function drawline_d( handles,i,x,range,rawData )

axes(handles.ch1);%channel 1

plot(x(1:i),rawData(1:i,1),'-');

xlim([0 range]);

xlabel('time(s)');

axes(handles.ch2); %channel 2

plot(x(1:i),rawData(1:i,2),'-');

xlim([0 range]);

xlabel('time(s)');

axes(handles.ch3);%channel 3

plot(x(1:i),rawData(1:i,3),'-');

xlim([0 range]);

xlabel('time(s)');

axes(handles.ch4);%channel 4

plot(x(1:i),rawData(1:i,3),'-');

xlim([0 range]);

xlabel('time(s)');

end