Automated Water Level Controller

Project Report

Ian Bailey

ECE 4220: Real Time Embedded Computing

Luis Rivera

5/14/15

# Abstract

For my project, I chose to design an automatic water level controller. This projects main goal is to implement a system that can manage the water level of a body of water such as a swimming pool, or fish tank, automatically. I utilized a raspberry pi to run the code necessary to read a series of switches that indicate the water level of a tank. It then determines if the water is too high or too low, and will signal to turn on a pump or drain in order to fluctuate the water level of the system. In addition to automatic control, my design also implements a client/server feature that allows the user to remotely access the system and check the level and also manually control the system.

# Introduction

Have you ever been over to a friend’s swimming pool and the water was too high or too low? It’s easy to just assume that the water has been splashed out or has been evaporated away from the hot summer sun. Both of these could be very likely causes of low water level. Now if you ask your friend to fill it up, I’m sure they’d either walk around and turn a pump on, or possibly throw a water hose in it. While these are very simple solutions, they do take a long time to noticeably make a difference. That’s why I chose to design an automatic water level controller than can sense when water needs to be added to, or drained from, a body of water.

I chose this problem because I’ve worked at various water parks over the past few years as a summer job and recognized it as a serious issue. Having a system that can control its own water level sets the managers mind at ease. Many things can happen to change water level overtime, evaporation, splash out, rain, etc. And when the water is too low for the drains, or so high that it splashes out as soon as someone jumps in, you get complaints. Another serious aspect of it is that when one works with huge bodies of water, it takes forever to see a difference, and if a system were to constantly oversee the water, it would remain at a set level.

My controller is able to sense when the water is too high or low, and automatically controls it. For my design, I made the drain and pump LEDs to simulate them turning on. I actually manually control the level by draining my tank, or adding water to it. If the water reaches a low level, it turns on the pump, and adds water to the system. On the other end of things, if it’s too high, the controller tells a drain to open, and it will then drain water until its reached an acceptable level. In addition, my system also runs a server that when a client connects to it, they can remotely view the water level, and they can put it in manual mode and turn the drain and pump on and off at will.

# Background

My design is very similar to what many call auto top off systems. These systems check water level and signal a pump to turn on when water needs to be added into the tank. Then, once the desired level has been reached they shut off until the trigger level is reached again. My system does however have an additional feature of being able to drain the body of water if the level has reached a high level. The drain aspect of my design is used often with sump pump controllers however. They check the water level consistently, and if the water gets too high or if the sump pump fails, it kicks on a pump that drains the water out of the system.

My system is a little more complex than these because it allows for a water level tolerance, therefore if the water gets too high, or too low, it is managed. In addition, no other system that I came across while researching the topic had any way to remote into the controller. That is very beneficial in many different instances.

This design could be very useful in systems that do change water level frequently such as large swimming pools. Whenever water needs to be added, instead of having to go manually turn on the fresh water supply, the system would do it all itself. This implementation could also be pretty useful in systems that requires a certain amount of water pressure such as water towers and hydraulic systems because they require the correct amount of liquid to be present, therefore a level could be set and the controller would help ensure that the system doesn’t fail.

# Proposed method / System description / Implementation

The hardware of my automatic water level controller is broken down to basically four different components. The first is the raspberry pi that is running the c program that’s included in the appendix. The pi then interfaces with a breadboard using a breakout board (pictured in figure 1) in order for the GPIO (general purpose input output) pins to be mapped out and interfaced with onto the breadboard. The breadboard has a simple circuit that has two LEDs, one red LED that simulates the pumps status, and a yellow one that simulates the drains status. The inputs that I use are wired from the breadboard and into my tank. These inputs are ninety-degree float switches (figure 2). They’re mounted to the inside of the tank and sealed watertight. These switches are active when the float is in the up position, I did however mount the bottom switch upside down, in order to show that they could really be used in either orientation depending on how you do the read in the program. In addition, I also mounted a drain spigot (figure 3) to simulate draining the tank, and I simply pour water in when water needs to be pumped into the system. You can see how the entire hardware system integrates together in figure 4 pictured below.

The project.c file in the appendix is my main file. This program utilizes an API that was specifically designed for the raspberry pi called wiringPi. This API has a a very useful and quick implementation for reading and writing to the various GPOI pins that the raspberry pi has as well as a threads and semaphore implementation that I took advantage of in order to simplify my program. The website that I used to reference these tools is www.wiringpi.com.

Figure 2

Figure 1

Figure 3

Figure 4

I started my program by first handling the automation aspect of my system. I did this by first setting up my GPIO pins using the wiringPi equivalences to reference them. If you look at lines 22-26, that is where I assign the inputs and outputs their name and reference them to their pin number. My main program is very simple because it calls the setup function that I made to setup the GPIO pins and spawn my threads. The main then runs an infinite while loop to let my system run. In the setup function, I assign the pins that I mentioned earlier using the piMode function. I assigned the LEDs as outputs, and the switches as inputs. Then I spawn my threads. I created three threads in order to run the automation, and a fourth thread that runs the server.

In my first thread, labeled Low, I ran an infinite while loop that simply checks the low switch, if the water is lower than that, it sets the water level variable to be 0, indicating that it is low. My second thread, labeled High, is very similar. It checks the top switch and sees if the water level is over it, if so, it sets the level to 2, indicating that it is high. In both of these threads you’ll see me use the piLock and piUnlock functions. These are essentially semaphores. I lock them so that if any other part of my program is trying to read that value or modify that value they can’t until the key is unlocked.

My third thread is the most important to how the automation works. This thread handles all the various cases that the switches could present, and turns the simulation LEDs on and off as necessary using the digitalWrite command. The thread first runs an infinite while loop, like the others, however it’s not quite as simple as the others from that point on. It checks the current water level variable as well as reading the switches and determines what to do based on those results. It first checks if the level is 0, if so, it calls a while loop that holds that part in place until the middle or level switch goes up. When that happens it turns on the pump LED off, otherwise it is on while the water is going to the desired level. I did this because even though the water passes the low switch, I wanted the high and low switches to be warnings and not levels that water actually came to. The middle switch is the desired water level therefore it runs until it hits that. After the level switch is hit, it then changes the current level holder by once again locking the semaphore.  It then shuts the LEDs off.

Automation Flow Diagram

Next up in the ManageLevel thread, you see my program check if the current water level is -1 and if neither the water is below the low switch, or higher than the high switch. This is to handle the case that the program is started with water already between the two extremes. Here I simply set the current level holder to be 1, indicating that it is within the desired level range. Then, my last check in this thread is the case that the current level is 2, indicating that its too high. It turns on the drain LED and then does the same as the first case and calls a while to wait until the level is beneath the middle or level switch. Once it is below, it once again sets the level to 1, and shuts off the drain LED.

The last thing I accomplish in my program was my server thread. The server sets up and communicates on the hardcoded port 5000. It first waits to receive a message from the server and compares it to a few different cases. When the message received is LEVEL, the program sends back the current level variable to the client. In the case that the client sends MANUAL, it toggles on/off the manual control of the system, which allows from pump and drain control. I used a global variable labeled manual that indicated if the program was in manual mode or not. I had to add that into most of the checks in the first three threads so that it basically shuts all the checks off and goes completely manual. Once in manual mode, the user can pass the message PUMP or DRAIN, then followed by ON or OFF to manually turn the respective LED on or off. In the final case, whenever the client logs out and left the program in manual mode, it takes it out and shuts off the LEDs so that the appropriate one turns on whenever it goes back to automation.

That’s pretty much how my project.c program operates. My client code, also in the appendix, is a little shorter. It has the statically assigned IP of my raspberry pi hardcoded in as well as the port number in which the two programs exchange data. Once set up, this code essentially sends and receives in a while loop until the user inputs the ‘!’ character. It converts all the user input to uppercase so that it handles all the different ways to type in the keywords. If you can follow the explination of how the server side ran, the client runs as a mirror, it sends what was received, and receives what was sent by the server. Once closed, it does close the socket to cleanup a bit.

# Experiments and Results

When developing this project there were many small bumps in the roads and challenges that presented themselves. I began really by experimenting with the wiringPi API learning how to read and write to and from the GPIO pins. I did his by making a few simple programs in which I controlled the LEDs by using a 3.3V power across a 333Ohm resistor. I also played around with the switches to determine where active high and low was for that specific device.

Once I got the hang of the API, I began to develop my program. I just used jumper wires primarily to simulate the float switches, so that I didn’t have to add and remove water from my tank continuously. So I would just pull the wires in and out as needed to test the program. I first developed my first three threads in order to accomplish the automation portion of my design. Once I was satisfied with my algorithm, I plugged in the tank, which I had tested multiple times to ensure its waterproofness. I then tested my program for all different water levels, starting low to high, low to level back to low, high to low, and high back to level back to high.

Lastly, I had to implement the client server, and this was the most challenging and time consuming portion of my project. After following the various UDP client server examples that we were given during Lab 5, I realized that I couldn’t just broadcast like I did in the lab because my home network wasn’t allowing me to put anything to the broadcast IP address. Therefore, I decided to just statically assign the IP address of the pi running my server software so that I can just hard code it into my client code.

Once I went into the configuration files of the pi and statically assigned the IP, I began to test the various methods to do he client server. My biggest issue was being able to send and receive. Once I figured out the correct information to pass to sendto and recvfrom, I got the hang of it and began sending messages back and forth at ease. I did have to change some things around however, for instance I added the function of the client converting all the inputs to all caps so that my server didn’t have to be case sensitive for the messages.

Once I got the manual mode and level messages working, I did have to play around with entering and exiting manual mode and making sure that my program returns to auto mode in the correct water level. For example I didn’t want to have a low water level, get a manual command to turn on the drain pump, then the client to disconnect. So when the client does disconnect, it simply shuts the LEDs off, then runs back in automation mode realizing that instead the pump needs to be on. I tested numerous instances of my client server and the communication between the two.

## Discussion and Conclusions

In conclusion, my automated water level controller works pretty much to perfection on the automation side. It’s very fast to determine when the pump and drain needs to be on, and it handles any case that could realistically happen to water level in a tank. There were some difficulties when implementing the client server, and it’s not exactly the most effective, meaning that I do occasionally miss a message on the client, but that’s somewhat expected with the multi-processing aspect of my code, as well as UDP in general isn’t that most reliable form of communication between two networked computers.

I learned a much greater understanding of the client server relationship and how those work with each other. I had to do an abundant amount of research external to the course work, in addition I do believe that my implementation is pretty efficient since I’m running four spawned threads on a quad core device. Meaning that at all times, each of the four threads has a task that it is running, so it wouldn’t have to share the task, thus needing more of a time-variant scheduling algorithm. As formerly mentioned, my project does have a few problems I’d work out if I had more time with it, such as the reliability of the network communication, a more robust user interface while remotely connecting, as well as even implementing real pumps to act as the drain and pumps.

Taking my project to the next level would be the ideal thing to do from here. I have purchased a relay board that I would then used to put my led signal lead into, and power the pumps with a larger voltage source than my pi could produce. This would then turn the pump on via the relay when it gets a signal from the pi’s output. I didn’t go this far in this project however because I didn’t posses the funds to purchase the drain and pump and to implement it.

# Appendix

project.c

/*	Ian Bailey

ECE 4220 project

imby96@mail.missouri.edu

05/14/2015

*/

#include <stdio.h>

#include <string.h>

#include <errno.h>

#include <stdlib.h>

#include <unistd.h>

#include <sys/types.h>

#include <sys/socket.h>

#include <netinet/in.h>

#include <netdb.h>

#include <arpa/inet.h>

#include <wiringPi.h>

//values for IO pins on the pi, commnets next to represenent actual pi pin

const int pumpLED = 0;		//17

const int drainLED = 3;		//22

const int lowSwitch = 1;	//18

const int levelSwitch = 4;	//23

const int highSwitch = 5;	//24

// A 'key' which we can lock and unlock - values are 0 through 3

// This is interpreted internally as a pthread_mutex by wiringPi

// This is a simplified sempaphore implimentation

#define	KEY1	0	//this key is designated for locking the water level variable

#define KEY2	1	//this key is designated for locking the manual mode variable

// This defines the message size to strictly be 40 characters

#define MSG_SIZE	40

// This is the hardcoded port number for the server

#define PORT_NUM	"5000"

// Water Level holding variable

// Also holds the value of the previous water level

// -1 => initialized

// 0 => low water level warning

// 1 => at water level

// 2 => high water level warning

typedef struct WaterLevel{

int current;

int prev;

} waterLevel;

//initializing global variable

waterLevel level;

int manual;

//function prototype

void setup (void);

//thread that controls the filling of the tank when the water is below the

//desired level (Location of the level switch)

PI_THREAD (Low){

//inf loop

while(1){

//checks if the water is below the low level

if(digitalRead(lowSwitch) && manual == 0){

//uses a semaphore to protect my level data being exchanged

piLock(KEY1);

level.prev = level.current;

level.current = 0;

piUnlock(KEY1);

}

}

}

//thread that controls the filling of the tank when the water is below the

//desired level (Location of the level switch)

PI_THREAD (High){

while(1){

//checks if the water is above the high level

if(digitalRead(highSwitch) && manual == 0){

//uses a semaphore to protect my level data being exchanged

piLock(KEY1);

level.prev = level.current;

level.current = 2;

piUnlock(KEY1);

}

}

}

PI_THREAD (ManageLevel){

while(1){

//checks if low alarm was going off, keeps pumpLED on until the

//desired level is reached

if(level.current == 0 && manual == 0){

while(!digitalRead(levelSwitch) && manual == 0){

digitalWrite(drainLED, LOW);

digitalWrite(pumpLED, HIGH);

}

//changing the level holder

piLock(KEY1);

level.prev = level.current;

level.current = 1;

piUnlock(KEY1);

//shutting off the pump

digitalWrite(pumpLED, LOW);

}

//checks if the system starts with water level between the two alarms

else if(level.current == -1 && !digitalRead(lowSwitch) && !digitalRead(highSwitch) && manual == 0){

//changing the level holder

piLock(KEY1);

level.prev = level.current;

level.current = 1;

piUnlock(KEY1);

digitalWrite(drainLED, LOW);

digitalWrite(pumpLED, LOW);

}

//checks if high alarm was going off, keeps drainLED on until the

//desired level is reached

else if(level.current == 2 && manual == 0){

while(digitalRead(levelSwitch) && manual == 0){

digitalWrite(pumpLED, LOW);

digitalWrite(drainLED, HIGH);

}

//changing the level holder

piLock(KEY1);

level.prev = level.current;

level.current = 1;

piUnlock(KEY1);

//shutting off the drain

digitalWrite(drainLED, LOW);

}

}

}

// get sockaddr, IPv4 or IPv6:

void *get_in_addr(struct sockaddr *sa)

{

if (sa->sa_family == AF_INET) {

return &(((struct sockaddr_in*)sa)->sin_addr);

}

return &(((struct sockaddr_in6*)sa)->sin6_addr);

}

// Thread that exclusively runs the server for this program, it sets up

// the socket, then listens for any requests from clients and handles manual

// modes as well as level indicator

PI_THREAD (Server){

int sockfd, rv, numbytes;

struct addrinfo hints, *servinfo, *p;

struct sockaddr_storage their_addr;

socklen_t addr_len;

char s[INET6_ADDRSTRLEN];

char message[MSG_SIZE];

char buf[MSG_SIZE];

char buf2[MSG_SIZE];

int i = 0;

memset(&hints, 0, sizeof hints);

hints.ai_family = AF_UNSPEC; 		// set to AF_INET to force IPv4

hints.ai_socktype = SOCK_DGRAM;

hints.ai_flags = AI_PASSIVE; 		// use my IP

//obtaining the information for the servers IP

if ((rv = getaddrinfo(NULL, PORT_NUM, &hints, &servinfo)) != 0) {

perror("getaddrinfo: %s\n");

exit(-1);

}

// loop through all the results and bind to the first we can

for(p = servinfo; p != NULL; p = p->ai_next) {

if ((sockfd = socket(p->ai_family, p->ai_socktype, p->ai_protocol)) == -1) {

perror("socket");

continue;

}

if (bind(sockfd, p->ai_addr, p->ai_addrlen) == -1) {

close(sockfd);

perror("bind");

continue;

}

break;

}

//checks for a successful bind

if (p == NULL) {

perror("failed to bind socket\n");

exit(-1);

}

freeaddrinfo(servinfo);

addr_len = sizeof their_addr;

//loop for the thread to run

while (1){

//reciving a message from clients

if ((numbytes = recvfrom(sockfd, buf, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, &addr_len)) == -1) {

perror("recvfrom");

exit(-1);

}

//prints the message and the IP address of who it is from

printf("Message: \"%s\" from %s\n", buf, inet_ntop(their_addr.ss_family, get_in_addr((struct sockaddr *)&their_addr), s, sizeof s));

//handles the level case and send the level to the client

if (strcmp(buf, "LEVEL") == 0){

piLock(KEY1);

sprintf(message, "%d", level.current);

piUnlock(KEY1);

if ((numbytes = sendto(sockfd, message, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, addr_len)) < 0){

perror("sendto");

exit(-1);

}

}

//handles manual mode on/off

if (strcmp(buf, "MANUAL") == 0){

if (manual == 0){

piLock(KEY2);

manual = 1;

piUnlock(KEY2);

//shutting off drain and pump

digitalWrite(drainLED, LOW);

digitalWrite(pumpLED, LOW);

strcpy(message, "Entered manual mode");

}

else if (manual == 1){

//shutting off drain and pump

digitalWrite(drainLED, LOW);

digitalWrite(pumpLED, LOW);

piLock(KEY2);

manual = 0;

piUnlock(KEY2);

strcpy(message, "Exited manual mode");

}

//sends message to the client

if ((numbytes = sendto(sockfd, message, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, addr_len)) < 0){

perror("sendto");

exit(-1);

}

}

//handles the case to turn on/off the pump in manual mode

if (strcmp(buf, "PUMP") == 0 && manual == 1){

//prompts the client

strcpy(message, "ENTER [ON/OFF]");

if ((numbytes = sendto(sockfd, message, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, addr_len)) < 0){

perror("sendto");

exit(-1);

}

//loops until a valid response (on/off)

do{

bzero(buf2, MSG_SIZE);

bzero(message, MSG_SIZE);

if ((numbytes = recvfrom(sockfd, buf2, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, &addr_len)) == -1){

perror("recvfrom");

exit(-1);

}

//turns on the pump

if(strcmp(buf2, "ON") == 0){

digitalWrite(pumpLED, HIGH);

strcpy(message, "Pump turned on");

i = 1;

}

//turns off the pump

else if(strcmp(buf2, "OFF") == 0){

digitalWrite(pumpLED, LOW);

strcpy(message, "Pump turned off");

i = 1;

}

//sends success message to the client

if ((numbytes = sendto(sockfd, message, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, addr_len)) < 0){

perror("sendto");

exit(-1);

}

} while( i != 1);

//reseting i

i = 0;

}

//handles the case to turn on/off the drain in manual mode

if (strcmp(buf, "DRAIN") == 0 && manual == 1){

//prompts the client

strcpy(message, "ENTER [ON/OFF]");

if ((numbytes = sendto(sockfd, message, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, addr_len)) < 0){

perror("sendto");

exit(-1);

}

//loops until a valid response (on/off)

do{

bzero(buf2, MSG_SIZE);

bzero(message, MSG_SIZE);

if ((numbytes = recvfrom(sockfd, buf2, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, &addr_len)) == -1){

perror("recvfrom");

exit(-1);

}

//turns on the drain

if(strcmp(buf2, "ON") == 0){

digitalWrite(drainLED, HIGH);

strcpy(message, "Pump turned on");

i = 1;

}

//turns off the drain

else if(strcmp(buf2, "OFF") == 0){

digitalWrite(drainLED, LOW);

strcpy(message, "Pump turned off");

i = 1;

}

//sends success message to the client

if ((numbytes = sendto(sockfd, message, MSG_SIZE-1, 0, (struct sockaddr *)&their_addr, addr_len)) < 0){

perror("sendto");

exit(-1);

}

} while( i != 1);

//reseting i

i = 0;

}

//exits manual status when connection is closed

if (strcmp(buf, "REMOTE CONNECTION HAS BEEN CLOSED") == 0 && manual == 1){

//shutting off drain and pump

digitalWrite(drainLED, LOW);

digitalWrite(pumpLED, LOW);

piLock(KEY2);

manual = 0;

piUnlock(KEY2);

}

//cleaning the various used buffers

bzero(message, MSG_SIZE);

bzero(buf, MSG_SIZE);

bzero(buf2, MSG_SIZE);

}

}

int main(void){

setup();

printf("Project is running, press 'ctrl + c' to quit.\n");

while(1){}

return 0;

}

void setup(void){

//setting up IO pin use

wiringPiSetup();

//seting up LEDs as outputs

pinMode(pumpLED, OUTPUT);

pinMode(drainLED, OUTPUT);

//setting up float switches as inputs

pinMode(lowSwitch, INPUT);

pinMode(levelSwitch, INPUT);

pinMode(highSwitch, INPUT);

//initializes the waterlevel

piLock(KEY1);

level.current = -1;

piUnlock(KEY1);

digitalWrite(pumpLED, LOW);

digitalWrite(drainLED, LOW);

//starting my threads

piThreadCreate(Low);

piThreadCreate(High);

piThreadCreate(ManageLevel);

piThreadCreate(Server);

return;

}

client.c

/*	Ian Bailey

ECE 4220 project

imby96@mail.missouri.edu

05/14/2015

*/

#include <stdio.h>

#include <ctype.h>

#include <stdlib.h>

#include <unistd.h>

#include <errno.h>

#include <string.h>

#include <sys/types.h>

#include <sys/socket.h>

#include <netinet/in.h>

#include <arpa/inet.h>

#include <netdb.h>

#define SERVERPORT  "5000"                  //Hardcoded the port number

#define MAXBUFLEN   40                      //Hardcoded the length for the buffers

#define PIIPADDR    "192.168.1.100"         //Hardcoded the static IP address of the server (pi running my level controller)

int main(void)

{

int sockfd, i, rv, numbytes;

struct addrinfo hints, *servinfo, *p;

struct sockaddr_storage their_addr;

socklen_t addr_len;

char buf[MAXBUFLEN];

char upperBuf[MAXBUFLEN];

char recievedMessage[MAXBUFLEN];

memset(&hints, 0, sizeof hints);

hints.ai_family = AF_UNSPEC;

hints.ai_socktype = SOCK_DGRAM;

int j = 0;

int flag = 0;

//gets info for client computer

if ((rv = getaddrinfo(PIIPADDR, SERVERPORT, &hints, &servinfo)) != 0) {

fprintf(stderr, "getaddrinfo: %s\n", gai_strerror(rv));

return 1;

}

//loop through all the results and make a socket

for(p = servinfo; p != NULL; p = p->ai_next) {

if ((sockfd = socket(p->ai_family, p->ai_socktype, p->ai_protocol)) < 0) {

perror("client: socket");

continue;

}

break;

}

//checks for successful bind

if (p == NULL) {

fprintf(stderr, "client: failed to bind socket\n");

return 2;

}

addr_len = sizeof their_addr;

//sends message that the connection was established

if ((numbytes = sendto(sockfd, "REMOTE CONNECTION HAS BEEN ESABLISHED", MAXBUFLEN, 0, p->ai_addr, p->ai_addrlen)) < 0) {

perror("client: sendto connection");

exit(1);

}

//gets message to send and adds NULL terminator

scanf("%s", buf);

strcat(buf, "\0");

//loops until user enters '!'

while(buf[0] != '!'){

//converts the message to all upper case

for (i = 0; i < strlen(buf); i++){

upperBuf[i] = toupper(buf[i]);

}

//sends upper case message

if ((numbytes = sendto(sockfd, upperBuf, strlen(upperBuf), 0, p->ai_addr, p->ai_addrlen)) < 0) {

perror("client: sendto");

exit(1);

}

//if you requested the level

if (strcmp(upperBuf, "LEVEL\0") == 0){

//gets the message from the server of the current level

if ((numbytes = recvfrom(sockfd, recievedMessage, MAXBUFLEN-1, 0, p->ai_addr , &p->ai_addrlen)) < 0){

perror("recieving messagge");

exit(1);

}

//checks for a low level

if (strcmp(recievedMessage, "0") == 0){

printf("Water Level: Low (pump on)\n");

}

//checks for an acceptable level

else if (strcmp(recievedMessage, "1") == 0){

printf("Water Level: Acceptable Range\n");

}

//checks for a high level

else if (strcmp(recievedMessage, "2") == 0){

printf("Water Level: High (drain on)\n");

}

}

//if you requested to go into, or exit manual mode

if (strcmp(upperBuf, "MANUAL\0") == 0){

//gets the conformation and prints it (on or off)

if ((numbytes = recvfrom(sockfd, recievedMessage, MAXBUFLEN-1, 0, p->ai_addr , &p->ai_addrlen)) < 0){

perror("recieving messagge");

exit(1);

}

printf("%s\n", recievedMessage);

//changes the flag to keep track it manual is on or off

if (flag == 0)

flag = 1;

else

flag = 0;

}

//if in manual, and you request to controll the drain or the pump

if ((strcmp(upperBuf, "PUMP\0") == 0 && flag == 1) || (strcmp(upperBuf, "DRAIN\0") == 0 && flag == 1)){

if ((numbytes = recvfrom(sockfd, recievedMessage, MAXBUFLEN-1, 0, p->ai_addr , &p->ai_addrlen)) < 0){

perror("recieving messagge");

exit(1);

}

//checks to see if you and the server are on the same level

if (strcmp(recievedMessage, "ENTER [ON/OFF]") == 0){

printf("%s\n", recievedMessage);        //prints on/off

bzero(recievedMessage, MAXBUFLEN);      //resets the buffers

bzero(buf, MAXBUFLEN);

bzero(upperBuf, MAXBUFLEN);

//loop until a valid command

do {

//scans and loads the users input

scanf("%s", buf);

strcat(buf, "\0");

//converts the message to all upper case

for (i = 0; i < strlen(buf); i++){

upperBuf[i] = toupper(buf[i]);

}

//checks if the command was valid

if(strcmp(upperBuf, "ON\0") == 0 || strcmp(upperBuf, "OFF\0") == 0){

//sends upper case message

if ((numbytes = sendto(sockfd, upperBuf, strlen(upperBuf), 0, p->ai_addr, p->ai_addrlen)) < 0) {

perror("client: sendto");

exit(1);

}

j = 1;

}

} while (j != 1);

if ((numbytes = recvfrom(sockfd, recievedMessage, MAXBUFLEN-1, 0, p->ai_addr , &p->ai_addrlen)) < 0){

perror("recieving messagge");

exit(1);

}

printf("%s\n", recievedMessage);

}

}

//cleans up the buffers

bzero(buf, MAXBUFLEN);

bzero(upperBuf, MAXBUFLEN);

bzero(recievedMessage, MAXBUFLEN);

//reprompts for next message

scanf("%s", buf);

strcat(buf, "\0");

}

//sends connection closed message

if ((numbytes = sendto(sockfd, "REMOTE CONNECTION HAS BEEN CLOSED", MAXBUFLEN, 0, p->ai_addr, p->ai_addrlen)) < 0) {

perror("client: sendto closed");

exit(1);

}

//closes everything up

freeaddrinfo(servinfo);

close(sockfd);

return 0;

}