# ECE 4220 – Real Time Embedded Computing

Department of Electrical and Computer Engineering University of Missouri – Columbia Spring Term, 2015 May 12, 2015

# Remote Login with TS-7250

Submitted by

Jake Sweet, Electrical & Computer Engineering

Course Instructor

Mr. Luis A. Rivera Estrada

# Abstract

The proposed project is a tool that electrical and computer engineering students can use that will allow them constant access to the TS-7250 boards. This helps the amount of time needed to spend inside the lab and allows the campus to operate and function safely while still giving the students ample time to work on lab assignments or research work.

Index terms— Computer Engineering Education, Electrical Engineering Education, Embedded Circuits, Microprocessors, Network Servers, Parallel Programming, Programmable Logic Circuits, Public Domain Autonomous Agent, Real-Time Systems, Virtual Private Networks

Table of Contents

Abstract	1

Table of Figures	3

Chapter 1. Introduction	1

Report Scope and Purpose	1

Problem Statement	1

Project Concept	1

Goals and Objectives	1

Motivation	2

Report Overview	2

Chapter 2. Project Overview	3

System Overview	3

Software Architecture Overview	3

Chapter 3. Specifications	1

Libraries Used	1

Software Diagram	1

Chapter 4. Design and Theory of Operation	2

Hardware	2

Software	3

Chapter 5. Experiments and Results	5

Testing Issues	5

Test Procedure	6

Test Results	6

Chapter 6. Conclusions	1

Appendix A.	Traceability	1

Software Used	1

Appendix B.	Bibliography	1

Appendix C.	Handwritten Design Notes	0

Appendix D.	Software	0

Code on Pi	0

Program: resetboard.c	0

Header File: wiringPi.c	1

Database Program: database.cpp	1

# Table of Figures

Figure 1: Hardware Diagram	3

Figure 2: Software Diagram	1

Figure 3: Hardware Setup with Circuit	3

Figure 4 WiringPi GPIO readall command	8

Introduction

## Report Scope and Purpose

This report establishes the project-level documentation including the project concept, specifications, design and theory of operation, test plan, and evaluation of the demonstrated project.

## Problem Statement

The problem that I will solve with my project is the problem of having to physically be in the Linux laboratory to work with the TS-7250 boards. When logging into a TS-7250 a student must first acquire the board’s mutex. Once this is done the student will then have to reset the board to login. This is an issue because the only way to reset the boards at the moment is by the physical button that is located on the board.

## Project Concept

My project will allow students the ability to log into the board remotely while connected on MizzouWireless. This is accomplished by adding a board to the workstations that are currently in the lab. This will allow students to work on the boards whenever they are on campus no matter what time of day it is. This will also allow users to work with the boards if there is currently another class in session that is not using the boards

## Goals and Objectives

The goal of my project is to successfully login remotely to my partner board and this will allow me to reset the button of the TS-7250 remotely. My objectives are to successfully ssh into a partner board. Design a circuit that will be able to, through software, reset the TS-7250 boards.

## Motivation

The motivation that I had for this project was found because I am a commuting student and do not have the luxury of being on campus very often. I found the issue of having to be physically in the laboratory to actually work on the TS-7250 boards being very difficult. The laboratory seems to have many sessions using the lab throughout the week and as a commuting student it was quite burdensome to have to come to campus during the weekend. With this system I could potentially log into the board of my choice and program on it even if another class is in session, so long as the board has not already been acquired or being used.

## Report Overview

For the remainder of the report I will provide details on how should my project be operated, how it functions, and I will also provide details on how my project works. Chapter 2 will be my project overview. This chapter will summarize my project’s components and sub components. Chapter 3 will break into the details that compose my software program. In chapter 4 I explain my design theory and the operation of my project. Chapter 5 is the evaluation of my project after completion. The concluding chapter, chapter 6, of my report will summarize the conclusions that I have drawn from my project and what they mean.

Project Overview

In this chapter I will summarize the entire project.

## System Overview

- My project consisted of several hardware components. The hardware components required for my project were two circuit boards, an optoisolator, and the database hardware system.

- My project also was composed of various software components that made up the entire system.

- The hardware components are shown here in Figure 1.

Figure 1: Hardware Diagram

## Software Architecture Overview

- The majority of my software architecture is run on the Raspberry Pi.

- My Raspberry program would reset the TS-7250 board and check the FairCom database for a user name and password

- If this username and password was correct then the Pi would reset the TS-7250 board otherwise the Pi would reject the credentials.

Specifications

This chapter describes remote login software structure used on the Raspberry Pi 2 board.

## Libraries Used

The libraries that were used with my program broke down into two categories. Category one is a library required for the FairCom database access. These libraries are predetermined by the FairCom staff as initializers, managing methods, as well as methods to clean up the memory space after the program has accessed the database.

Category two is the libraries required to access the GPIO pins on the Raspberry board. There was only one particular library I used which was the wiringPi library. This library was to compile C code and access the GPIO pins easily.

## Software Diagram

The software setup is as follows for the project. These all are methods that will be accessible after the user is in the Raspberry Pi.

Figure 2: Software Diagram

Design and Theory of Operation

In this chapter I will detail the theory of operation between these components and subcomponents.

## Hardware

For the hardware part of my project I decided to go with the Raspberry Pi 2 as my second circuit board. I thought this would give me good flexibility as well as allow me to easily access libraries that are currently available to handle the GPIO properly. I also chose the Raspberry Pi because it has an ARM A-7 processor. I thought this would make the circuit board more ready to handle the FairCom database API. To carry out the board reset from the circuit board I chose to connect the GPIO pins from the Raspberry Pi board to an optoisolator. This gives the project the flexability of being able to be used by any board in the lab. The TS-7250 boards, currently, have red reset buttons that they are connected to. These reset buttons are connected to a headphone plug. If I use the optoisolator I can connect the optoisolator to a corresponding jack for each board. This would allow me to reset every board in the lab from one Raspberry Pi. This implies that I have long enough wires to reach each TS-7250. The other hardware components are previously designed components, the FairCom database, NFS1 server, and the TS-7250.

Figure 3 below depicts the hardware setup between the Raspberry Pi and the circuit required to reset the TS-7250 board. This setup does not use the headphone jack as would the actual system since I am just testing this for continuity between the optoisolator pins. This circuit will trigger when the Raspberry Pi sends a logic high signal to the proper GPIO that is paired with the 2N33 optoisolator. This will allow the current in the Collector-Emitter that will close the circuit, thus acting as a boardreset.

Figure 3: Hardware Setup with Circuit

## Software

For the software portion of my project I use a C program to set the GPIO pins. Luckily I was able to take advantage of the WiringPi library to access the GPIO. The user would first have to be on the MizzouWireless network because the user is to access everything through NFS1. This means that if NFS1 were accessible outside of MizzouWireless then the user could potentially be able to access the TS-7250 from anywhere they have internet access. Unfortunately a user cannot access NFS1 outside of the campus network at the moment. Given that a user has successfully accessed NFS1 then the user would simply have to ssh into the raspberry pi with the following: ssh <username>@<pi’s IP address>. The raspberry pi would keep a list of all the users that are able to login to the raspberry pi on the FairCom database server. The raspberry pi checks the server for the proper username and once a valid username is found the user gains access to the Raspberry Pi. Once successfully logged into the pi the user would then have to login to the try and acquire the TS-7250 board’s mutex. This is done with a simple boardsetup <board number> command. The user must then navigate to whichever directory the boardreset program is listed. By default the boardreset program will be in the home/pi directory. Thus, the user should be able to start the executable file and the board will be reset. A user should be able to tell whether or not the board has been reset by immediately trying to login to the board and receiving an “ERROR Board Not reset” message. The user should then, giving enough time for the board to reset, be able to login to the TS-7250 board.

Experiments and Results

This section is my own evaluation of my design and implementation of my project. I will outline my testing procedure as well as the results

## Testing Issues

The first problem that I ran into with my project was trying to ssh into my Raspberry Pi 2. I began by taking my circuit board up to the Linux laboratory in Lafferrre and connecting the Pi directly through Ethernet connection to the NFS1 server. I then logged into the server and then I tried to login to the Raspberry Pi. I found that I was experiencing ‘time-out’ errors. I consulted with a couple TA’s and they stated that there may be a Firewall setting that is not allowing me to ssh into the network. This resulted in my project’s first compromise. My project then was going to be tested using my home network. This would allow me assurance that any issues I encounter would not be the result of some security protocols that the school is required to have.

The next issue that I had was properly using the FairCom system. I knew the FairCom system was going to be difficult to implement given that I wanted to put it onto the Raspberry Pi. This was because I lacked any of the setup instructions and tutorials that would have been easier to implement on the TS-7250 boards. This did not stop me from wanting to complete the task I had set out to accomplish. I began by contacting the FairCom support team to ask about using the given product key to implement on the Raspberry Pi. The individual who contacted me first stated that we were only allowed the one API that previously determined. He did tell me though that I might be able to implement the API given that the Raspberry Pi 2 comes with an ARM processor. Thus, I began to try and implement an API that was not initially set up to work on the Raspberry Pi system.

## Test Procedure

To test my system I had to try and use a work around method that would essentially be the same as the board’s intended use. This procedure was broken into a couple smaller tests that led up to the overall task to be tested. The first test was to check and make sure that running the program from the pi directly worked. I have attached a short video clip showing this test. The second test involved me logging into the board and running the software program remotely from the pi. To do this I simply logged into my Raspberry Pi over my home network and ran the software that I had developed. In order to determine if the Raspberry Pi was using the optoisolator correctly I simply set up a small LED test circuit. This circuit used an LED connected to the proper power pins. When I ran the program the proper GPIO pin would set to low and ground the LED. This would create current through the LED and I would see light. This simulates the headphone jack connected to the reset button on the TS-7250 boards that are in the Linux lab.

## Test Results

I was unable to check the FairCom system for usernames and passwords. I’m still unsure if the problem is with settings on my Raspberry Pi or if the Pi does not have the required hardware to operate on the FairCom system. After numerous attempts at trying to compile the tutorial for the FairCom system without any success I simply changed the tutorial code to reflect what I wished to accomplish. The FairCom file that I include in this document does not search a table for user names and passwords as I originally planned. I was able to change the tutorial to reflect the table that I wished to check for the username pairs, but the software only prints off this table. Research on server setups and systems showed me that instead of having a database hold usernames and passwords what could be done that I liked was creating a special key value pair that the ssh connection checks for on the client machine. This way every client machine that connects to the server is directly responsible for their connection. This would, from what I have learned about these types of connections, be added security as opposed to checking a database. This would also cut costs of my system since there would be no fee’s for upkeep of a database that only holds usernames and passwords. I was able to contact the staff at FairCom who seemed unsure on whether or not I would have much success with the API I was given. I was however successful in setting up the server on my home network and resetting the board as I sought out to accomplish. I did try to reach Mr. Fischer several times about helping work on the NFS1 server, but it seem as though he was very busy and did not have time to work with someone outside of his already very large group of students he is directly responsible for. I think given more time I would have tried to work with the IT department on what I should actually be trying to do to circumvent any unnecessary security protocols the school has setup to solve my issue of connecting to the pi through the NFS1 server.

I did have quite a lot of success using the wiringPi API. Figure 4 below shows some of my results using the wiringPi API. One of the many functions that are available is gpio readall. This function return the entire list of GPIO pins on the Raspberry Pi and their value. It also shows whether or not the pin is declared as an INPUT or OUTPUT. This helped me many times in troubleshooting whether or not my code was operating correctly. This API also came with functions such as void pinMode(int pin, int mode). This as well as other functions that the API provided were extremely helpful in setting up, operating on, and checking the GPIO pins.

Figure 4 WiringPi GPIO readall command

Conclusions

During this project I was able to learn a lot from embedded systems. I was able to connect into a commercial board and develop on this board from a remote location. Another success of this project was that I was able to accurately operate and access the GPIO pins of the board as well as working with the Raspbian operating system. I started from a blank slate (the Raspberry Pi arriving at my home) to a fully functional working board that can be changed to reset a TS-7250 board. I believe going all the way from this blank state and then downloading the operating system, setting up the proper compiler options, downloading the proper software, and finding the right libraries was a much larger portion of this project than I initially thought. I did not realize that Raspberry did not have a plug-and-play setup. This took me a while to better understand my options and choose what software I needed and what exactly that offered my system. Much of my project consisted of research with regards to the FairCom system as well as the Raspberry Pi hardware and software.

With the FairCom system I initially did not realize that the setup that our class received was specifically for the TS-7250. The issue still did not unveil the complexities that it held after I spoke with the FairCom Technician. It was not until I realized that most of the Read Me files as well as the tutorials on the FairCom help website were all either for Windows based applications or unique individualized files that assisted students with loading the software onto the TS-7250 baords. This did not hinder me from learning the FairCom system and trying to complete the tutorial programs. I did enjoy learning about database applications and how easy they can be if we use the proper API. The FairCom API was very simple and easy to manipulate, change, and customize. I did play with the software slightly, but was unable to incorporate it into my project correctly. I found that it would be best if my efforts were spent in other ways.

The Raspberry Pi research took up another large portion of my time due to the lack of knowledge of what was necessary for compiling and working with the Raspberry Pi. I also struggled with trying to decide on what language to use as the Raspbian operating system was more python friendly than any other language, but I did not have any experience in python. I also considered using java because I recently took a class in java. Eventually I settled on using C for my project. I found a great API library that seemed widely used. Using wiringPi, it was not very difficult writing to the GPIO pins or reading from them. The issue I did have with the Raspberry Pi was establishing a proper connection to the hardware through NFS1. I still am unsure if the Raspberry Pi was being blocked by some University of Missouri IT software or if there was some settings or files that I may have incorrectly modified. The majority of this project did depend greatly on the state of the software on Raspberry Pi. I learned that the most important part of choosing a hardware portion of a project is knowing the hardware and preinstalled software’s on the hardware. Since I did not fully know what came on the hardware or how exactly it should be setup I had issues with simplicities such as connecting to Wi-Fi.

Traceability

## Software Used

Gordon Henderson. WiringPi. GPIO Version 2.26.

Raspberry Pi Foundation. Raspbian Debian Wheezy. Kernel Version 3.18.

FairCom. FairCom ISAM Record Oriented Interface c-treeDB.

Bibliography

[1]	Gordon Henderson. (2015.). GPIO Interface Library for the Raspberry Pi  [Online]. Available: http://wiringpi.com/

[2]	FairCom. (2015). FairCom ReadMe Files.

Handwritten Design Notes

Software

# Code on Pi

This section contains all the code implemented on the physical Raspberry Pi.

## Program: resetboard.c

#include <wiringPi.h>

#include <stdio.h>

#include <stdlib.h>

//also add -I/usr/local/include -L/usr/local/lib -lwiringPi

int checkReset(int x);

//This function sets pin1 to high and pin2 to low

//waits for 1 second and then reverses the setting of the pins

void resetBoard(int pin1){

printf("Trying to reset board %d\n",pin1);

//set pin to output

pinMode(pin1,OUTPUT);

//set pin value

digitalWrite(pin1,HIGH);

//delay in milliseconds

delay(100);

//reset pin value

digitalWrite(pin1,LOW);

}

int checkReset(int checkpin){

int temp;

temp=digitalRead(checkpin);

if(temp==0){

printf("ERROR Board Not reset\n");

return temp;

}

if(temp==1){

printf("The board was reset\n");

return temp;

}

else {

printf("ERROR SOMETHING WENT WRONG\n");

return -1;

}

}

int main(int argc, char *argv[]){

unsigned int delay=100;

int boardSelection = 0;

if(wiringPiSetup()==-1)

exit(1);

if(argc!=2)

printf("Program usage error\nUsage is: ./a.out <boardNumbertoReset>\n");

boardSelection=atoi(argv[1]);

//this switch statement opperates on the boardSelection

switch(boardSelection){

case 1:

resetBoard(0);

checkReset(1);

break;

case 2:

resetBoard(1);

checkReset(2);

break;

//This could be continued for any number of boards up to 40

default:

break;

}

}

return 0;

}

## Header File: wiringPi.c

## Database Program: database.cpp

/*

* Functionally, this application will perform the following:

*  1.  Logon onto a session

*  2.  Add 1 table with some fields

*  3.  Populate the table with a few records

*  4.  Display the contents of the table

*/

#ifdef _WIN32_WCE

#undef UNICODE

#undef _UNICODE

#define main my_main

#endif

/* Preprocessor definitions and includes */

#include "ctdbsdk.h" /* c-tree headers */

#define END_OF_FILE INOT_ERR  /* INOT_ERR is ctree's 101 error. See cterrc.h */

/* Global declarations */

CTHANDLE hSession;

CTHANDLE hDatabase;

CTHANDLE hTable;

CTHANDLE hRecord;

/* Function declarations */

#ifdef PROTOTYPE

VOID Initialize(VOID), Define(VOID), Manage(VOID), Done(VOID);

VOID Add_Records(VOID), Display_Records(VOID);

VOID Delete_Records(CTHANDLE), Check_Table_Mode(CTHANDLE);

VOID Handle_Error(CTSTRING);

#else

VOID Initialize(), Define(), Manage(), Done();

VOID Add_Records(), Display_Records();

VOID Delete_Records(), Check_Table_Mode();

VOID Handle_Error();

#endif

/*

* main()

*

* The main() function implements the concept of "init, define, manage

* and you're done..."

*/

#ifdef PROTOTYPE

NINT main (NINT argc, pTEXT argv[])

#else

NINT main (argc, argv)

NINT argc;

TEXT argv[];

#endif

{

Initialize();

Define();

Manage();

Done();

printf("\nPress <ENTER> key to exit . . .\n");

#ifndef ctPortWINCE

getchar();

#endif

return(0);

}

/*

* Initialize()

*

* Perform the minimum requirement of logging onto the c-tree Server

*/

#ifdef PROTOTYPE

VOID Initialize(VOID)

#else

VOID Initialize()

#endif

{

CTDBRET  retval;

printf("INIT\n");

if ((retval = ctdbStartDatabaseEngine())) 				/* This function is required when you are using the Server DLL model to start the underlying Server. */

Handle_Error("Initialize(): ctdbStartDatabaseEngine()"); 	/* It does nothing in all other c-tree models */

/* allocate session handle */

if ((hSession = ctdbAllocSession(CTSESSION_CTREE)) == NULL)

Handle_Error("Initialize(): ctdbAllocSession()");

hDatabase = hSession; /* database not used in this tutorial */

/* connect to server */

printf("\tLogon to server...\n");

if (ctdbLogon(hSession, "FAIRCOMS", "ADMIN", "ADMIN"))

Handle_Error("Initialize(): ctdbLogon()");

}

/*

* Define()

*

* Open the table, if it exists. Otherwise create and open the table

*/

#ifdef PROTOTYPE

VOID Define(VOID)

#else

VOID Define()

#endif

{

CTHANDLE hField1, hField2;

printf("DEFINE\n");

/* allocate a table handle */

if ((hTable = ctdbAllocTable(hDatabase)) == NULL)

Handle_Error("Define(); ctdbAllocTable()");

/* open table */

printf("\tOpen table...\n");

if (ctdbOpenTable(hTable, "usrInfo", CTOPEN_NORMAL))

{

/* define table fields */

printf("\tAdd fields...\n");

hField1 = ctdbAddField(hTable, "usrID", CT_FSTRING, 4);

hField2 = ctdbAddField(hTable, "usrPSWD", CT_FSTRING, 9);

if (!hField1 || !hField2

Handle_Error("Define(); ctdbAddField()");

/* create table */

printf("\tCreate table...\n");

if (ctdbCreateTable(hTable, "usrInfo", CTCREATE_NORMAL))

Handle_Error("Define(); ctdbCreateTable()");

if (ctdbOpenTable(hTable, "usrInfo", CTOPEN_NORMAL))

Handle_Error("Define(); ctdbOpenTable()");

}

else

Check_Table_Mode(hTable);

}

/*

* Manage()

*

* This function performs simple record functions of add, delete and gets

*/

#ifdef PROTOTYPE

VOID Manage(VOID)

#else

VOID Manage()

#endif

{

printf("MANAGE\n");

/* allocate a record handle */

if ((hRecord = ctdbAllocRecord(hTable)) == NULL)

Handle_Error("Manage(): ctdbAllocRecord()");

/* delete any existing records */

Delete_Records(hRecord);

/* populate the table with data */

Add_Records();

/* display contents of table */

Display_Records();

}

/*

* Done()

*

* This function handles the housekeeping of closing tables and

* freeing of associated memory

*/

#ifdef PROTOTYPE

VOID Done(VOID)

#else

VOID Done()

#endif

{

printf("DONE\n");

/* close table */

printf("\tClose table...\n");

if (ctdbCloseTable(hTable))

Handle_Error("Done(): ctdbCloseTable()");

/* logout */

printf("\tLogout...\n");

if (ctdbLogout(hSession))

Handle_Error("Done(): ctdbLogout()");

/* free handles */

ctdbFreeRecord(hRecord);

ctdbFreeTable(hTable);

ctdbFreeSession(hSession);

/* If you are linked to the Server DLL, then we should stop our Server at the end of the program.   */

/* It does nothing in all other c-tree models */

ctdbStopDatabaseEngine();

}

/*

* Check_Table_Mode()

*

* Check if existing table has transaction processing flag enabled.

* If a table is under transaction processing control, modify the

* table mode to disable transaction processing

*/

#ifdef PROTOTYPE

VOID Check_Table_Mode(CTHANDLE hTable)

#else

VOID Check_Table_Mode(hTable)

CTHANDLE hTable;

#endif

{

CTCREATE_MODE mode;

/* get table create mode */

mode = ctdbGetTableCreateMode(hTable);

/* check if table is under transaction processing control */

if ((mode & CTCREATE_TRNLOG))

{

/* change file mode to disable transaction processing */

mode ^= CTCREATE_TRNLOG;

if (ctdbUpdateCreateMode(hTable, mode) != CTDBRET_OK)

Handle_Error("Check_Table_Mode(); ctdbUpdateCreateMode");

}

}

/*

* Delete_Records()

*

* This function deletes all the records in the table

*/

#ifdef PROTOTYPE

VOID Delete_Records(CTHANDLE hRecord)

#else

VOID Delete_Records(hRecord)

CTHANDLE hRecord;

#endif

{

CTDBRET  retval;

CTBOOL   empty;

printf("\tDelete records...\n");

empty = NO;

retval = ctdbFirstRecord(hRecord);

if (retval != CTDBRET_OK)

{

if (retval == END_OF_FILE)

empty = YES;

else

Handle_Error("Delete_Records(): ctdbFirstRecord()");

}

while (empty == NO) /* while table is not empty */

{

/* delete record */

if (ctdbDeleteRecord(hRecord))

Handle_Error("Delete_Records(): ctdbDeleteRecord()");

/* read next record */

retval = ctdbNextRecord(hRecord);

if (retval != CTDBRET_OK)

{

if (retval == END_OF_FILE)

empty = YES;

else

Handle_Error("Delete_Records(): ctdbNextRecord()");

}

}

}

/*

* Add_Records()

*

* This function adds records to a table in the database from an

* array of strings

*/

typedef struct {

CTSTRING usrID, usrPSWD;

} CUSTOMER_DATA;

CUSTOMER_DATA data[] = {

"jlsyr4", "nogirlsallowed",

"jvax3x", "password",

};

#ifdef PROTOTYPE

VOID Add_Records(VOID)

#else

VOID Add_Records()

#endif

{

CTDBRET  retval;

CTSIGNED i;

CTSIGNED nRecords = sizeof(data) / sizeof(CUSTOMER_DATA);

printf("\tAdd records...\n");

/* add data to table */

for (i = 0; i < nRecords; i++)

{

/* clear record buffer */

ctdbClearRecord(hRecord);

retval = 0;

/* populate record buffer with data */

retval |= ctdbSetFieldAsString(hRecord, 0, data[i].usrID);

retval |= ctdbSetFieldAsString(hRecord, 1, data[i].usrPSWD);

if (retval)

Handle_Error("Add_Records(): ctdbSetFieldAsString()");

/* add record */

if (ctdbWriteRecord(hRecord))

Handle_Error("Add_Records(): ctdbWriteRecord()");

}

}

/*

* Display_Records()

*

* This function displays the contents of a table. ctdbFirstRecord() and

* ctdbNextRecord() fetch the record. Then each field is parsed and displayed

*/

#ifdef PROTOTYPE

VOID Display_Records(VOID)

#else

VOID Display_Records()

#endif

{

CTDBRET  retval;

TEXT     usrid[6+1];

TEXT     usrpwd[47+1];

printf("\tDisplay records...");

/* read first record */

retval = ctdbFirstRecord(hRecord);

if (retval != CTDBRET_OK)

Handle_Error("Display_Records(): ctdbFirstRecord()");

while (retval != END_OF_FILE)

{

retval = 0;

retval |= ctdbGetFieldAsString(hRecord, 0, usrid, sizeof(usrid));

retval |= ctdbGetFieldAsString(hRecord, 1, usrpwd, sizeof(usrpwd));

if (retval)

Handle_Error("Display_Records(): ctdbGetFieldAsString()");

printf("\n\t\t%-8s%10s\n",usrid, usrpwd);

/* read next record */

retval = ctdbNextRecord(hRecord);

if (retval == END_OF_FILE)

break;   /* reached end of file */

if (retval != CTDBRET_OK)

Handle_Error("Display_Records(): ctdbNextRecord()");

}

}

/*

* Handle_Error()

*

* This function is a common bailout routine. It displays an error message

* allowing the user to acknowledge before terminating the application

*/

#ifdef PROTOTYPE

VOID Handle_Error(CTSTRING errmsg)

#else

VOID Handle_Error(errmsg)

CTSTRING errmsg;

#endif

{

printf("\nERROR: [%d] - %s \n", ctdbGetError(hSession), errmsg);

printf("*** Execution aborted *** \nPress <ENTER> key to exit...");

ctdbLogout(hSession);

ctdbFreeRecord(hRecord);

ctdbFreeTable(hTable);

ctdbFreeSession(hSession);

getchar();

exit(1);

}

/* end of ctdb_tutorial3.c */