Real Time Embedded Computing

Final Project:

Report

Project Name:

Weeping Sirens

Done by:

Teekayu Klongtruajrok		(tkq66)

ABSTRACT

Weeping Siren is a physical threat detection system, aiming for convenient integration into a bigger system. The system contains a module both in the kernel space that handles hardware and software interrupts, and a module in the user space that performs data analysis and threat reporting. The current system is compatible with the TS-7250 board, and takes in data from A/D Channel 1. The A/D is interfaced inside the user space module, in which it is fitted into a Simple Linear Regression plot, with the independent variable being the timestamp. The sensor data is that of the brightness. The slope of the linear regression tells how fast the brightness changes, implying that an object crashing it is imminent, thus qualifying as a threat. Thus the linear regression slope is the threat classifier for this system. Upon detecting the threat, various interrupts are triggered and finally there is a response module. The response module for this prototype changes the LED light from green to red, pauses the program, send the alert message over UDP socket, and report a time difference for the user to analyze the effectiveness of the response. This system could be integrated into any system, with the choice of the threat classifier and choice of responses. The linear regression module could easily be replaced by any algorithm, as well as a prototype response block. The project yields an acceptable response time and is responsive to quick change in light flux.

TABLE OF CONTENTS

INTRODUCTION	3

BACKGROUND	4

PROPOSED METHOD/SYSTEM DESCRIPTION/IMPLEMENTATION	6

EXPERIMENTS AND RESULTS	14

DISCUSSION AND CONCLUSIONS	21

APPENDICES A – THE CODE	24

A.1) KERNEL SPACE MODULE	24

<embedded_project_kernel.c>	24

A.2) USER SPACE	31

<embedded_project_user.c>	31

<ep93xx_adc.h>	48

<eeprog.h>	50

<peekpoke.h>	52

INTRODUCTION

This project aims to provide a fast and accurate threat detection and response system that is easily customizable. Physical threats are ever-imposing on electronic devices. There are a wide range of physical threats, be it from liquid, destructive electromagnetic interference, extreme changes in thermodynamic variables, or a simple impact of a concrete object. Upon exposure to these threats, many physical devices are rendered obsolete – take for example a phone that is dropped from the second floor. To say that his is undesirable would be an understatement. There are various threat-detection and response system implemented in the industry, for example on drones that fly over Syria or rovers that collects data on Mars, but those systems are very application specific and is not customizable for general consumer and small-scale electronics.

Since sensors are the only interface a machine has to the outside world, it could be implied that with sufficiently diverse and enough of those sensors to accurately map out the continuous nature of ambient variables, then the machine could extrapolated accurately the status of its surroundings. Upon this proposition, this project will serve as proof-of-concept by concentrating on just one threat: physical collision. With an accurate threat-classification module and a sensor interface, a system could be built that would serve as a core of any threat-detection. Upon making this abstraction, the project could be said to also trying to provide a core API of physical threat-detection, with the user having to only implement their own response module – or their own classification algorithm, if they so choose.

The problem with threat-detection always leads back to response time. With internal clocks of computers being a limited resource and the number of CPU cores having a lower limit of just one, one cannot always rely on the real-time scheduling on the software side. However, with an existence of interrupt mechanisms, a fast response time can be guaranteed.

BACKGROUND

There are three existing spheres of knowledge that this project is building on. The first is the knowledge of real-time system computing, in which the project needs the capability of existing multithreading, socket communication, pipe communication, and interrupt handling mechanism to ensure effective data transferring. Multi-threading is done through employing the pthread library to create real-time tasks on the user space, possibly to concurrently calculate threats while another thread await messages from the socket, both of which could cause a response. Socket communication is needed to transform all machines with this system into both a client and a server to be susceptible to other machine’s threats. Pipe communication is done by using real-time FIFO, this ensure data transfer between the kernel space module and the user space module. Finally, the interrupt handling acts as a concurrently status checking, which allows for an almost-immediate response time when a threat is detected.

The second sphere of knowledge is the analog-to-digital (A/D) conversion. The manufacturer of the TS-7250 board provides a Cirrus EP9302 that features 5 channels. Handling the (A/D) requires the user to interface with the ADCClk, ADCSWLock, ADCClkDiv, ADCEN , ADCPD, ADCSwitch, and the ADCResult registers. Keeping track of these registers added an additional complexity to the project. This is not to mention that the value received is in an un-interpreted raw data format, which may or may not make sense for post-processing. In order to translate the raw data into an understandable voltage value, the user now needs to handle a non-volatile EEPROM on the TS-7250 which stored the calibration values for each channel, then calibrate each value of the channel accordingly before making sure that the EEPROM is consistent. Due to difference in compiler priorities, the dereferencing of the location of the EEPROM might not yield the desired content, since the compiler could decide that it wants to do a byte write instead. This causes the user to take a precaution and take the data directly from the registers via the LOAD and STORE commands. Due to these complexities, the implementer believes that it exceeds the scope of this class and this project. The manufacturer’s software development kit for the TS-7250 provides the code for the A/D interface as well as the calibration scheme, thus it could be implied that these implementations are standard, and there would be no innovation in re-inventing the wheel. For these reasons, the implementer felt it is justified to build upon these existing implementations of A/D handling.

The third sphere of knowledge if the pattern classification algorithm that is to be employed in this project. As established in the introduction, the hypothesis of the condition for imminent violence is rapid change in flux of various external variables. In this case, that the project is taking in only the brightness values, the desired change to be observe is the change in brightness, implying an object that is about to collide is moving towards the sensor at a quick enough speed to cause violent collision. Linear Regression is a standard statistical method that very straightforwardly implies a rate of change of the data, thus this algorithm is chosen. The linear regression model tries to fit discrete data points into a linear line. The formula for calculating the model is shown in Figure 1.

Figure 1: Linear regression formula

Linear regression could be extended to an n-dimension vector, given n is a positive integer. Thus if there is n number of sensors, this algorithm, could extend to accommodate such case. The value of interest is the slope beta-hat (), and that will be the classification value for threat detection.

PROPOSED METHOD/SYSTEM DESCRIPTION/IMPLEMENTATION

The way the system works is that first there is raw data coming into the A/D. The A/D converts the analog voltage into digital data, which is then processed so that the system now have a numerical representation of the voltage data. This data is feed into the threat-analysis system. If a threat is detected, both respond and send the message to all other machines in the network. This could be seen in the flow chart below.

Figure 2: Hardware flow chart

The sensor used to read ambient data is a photodiode QSD2030F. This is sent to an operational amplifier to amplify the DC offsets, which is proportional to the flux intensity. The output of the op-amp circuit is fed into Channel 1 of the A/D. The circuit is show below. The op-amp is the LM741 chip.

Figure 3: Circuit diagram

An adjustment to that diagram during implementation is that the Positive and negative source voltages are taken in from two 9V DC batteries. This guarantee a truly DC signal with no periodic noise from the machine, like the ones found in function generators or wall-sockets. The circuits could be seen the following pictures.

Figure 4: Built circuit

Figure 5: Power source

Figure 6: Connection to A/D

Figure 7: Connection to auxiliary board

The kernel module acts primarily as interrupt handlers. Interrupt is chosen over real-time checking of statuses in an infinite busy loop because it does not take as much resources. A busy loop hogs all the resources and potentially causes starvation. Thus to use interrupts for all status checking and handling provides a faster response time and a guaranteed flow for the program. The kernel module could be seen below.

Figure 8: Kernel module flow chart

The user space module first sets up the socket communications, port the registers for LEDs, software interrupts, and necessary registers for the A/D. It then creates two threads: the linear regression and a receiver thread for FIFO. The linear regression is a periodic real-time process, reading in data from A/D and calculating the slope every 1 milliseconds. The receiver thread is always in blocking as long as there is nothing in the FIFO, and set the alert status accordingly. If alert is set to high, a response function is called. This is the part where the user of this system can freely customize, depending on how the user wants to respond to a threat. The linear regression thread first takes in a sample of 100 data points to construct a standard linear model of the ambient, assuming steady state. The function that reads in from the A/D is a modification of an existing code provided by the manufacturer, thus it is not shown in the flow chart. After the initialization is done, the linear regression slope is re-calculated for each period. If the slope is greater than a certain threshold, then triggers the hardware interrupt. This will cause the interrupt handler to send a high signal through the FIFO to the receiver thread, thus the alert is now set to high and the linear regression calculation stops as it exits the periodic loop. If the button on Port B is pressed, hardware interrupt is triggered and send a low signal through the FIFO, thus alert signal is now low, linear regression restarts, and the response function respond accordingly. The flow chart is shown below.

Figure 9: User space module flow chart (excluding respond module)

The prototype response module is called whenever the alert signal is set. First, in order for the server to also be able to act as client, the server socket is set to broadcast IP. Then, if alert is 1, send an alert message to everyone in the same networking port, turn on red LEDs, then report the response time from the threat detection in the linear regression module up until this point. If the alert signal is 0, send an OK message through the socket and turn off the red LED’s. The flow chart is shown below.

Figure 10: User space module, response function

EXPERIMENTS AND RESULTS

First, an experiment is done on the photodiode to make sure that it yields a noticeable response to change in flux. Upon measuring the circuit’s voltage response on the oscilloscope, a DC offset have a range of values between 200 mV and 50 mV is observed, the former being during full light exposure and the latter being when fully covered in darkness. The result of the experiment is shown below.

Figure 11: Measured voltage, full light exposure. Power source is from the power supply of the PXI1033 machine.

Figure 12: Measured voltage, fully covered. Power source is from the power supply of the PXI1033 machine.

Figure 13: Measured voltage, full light exposure. Power source is from the 9 V DC batteries.

Figure 14: Measured voltage, fully covered. Power source is from the 9 V DC batteries.

As could be observed, a commercial power supply provides substantial noise, thus the decision to use the 9V battery is justified.

When testing the A/D’s ability to read value, a the resulting data confirms the observed value in the oscilloscope, such that when fully exposed to light, the value yield approximately 200. The following data is printed from the program:

AVERAGE SENSOR'S VALUES:

sensor: 211.002000, time: 601.217061

sensor: 210.672000, time: 601.237032

sensor: 210.804000, time: 601.257022

sensor: 210.804000, time: 601.276998

sensor: 210.804000, time: 601.296982

sensor: 210.870000, time: 601.308131

sensor: 210.870000, time: 601.328099

sensor: 211.002000, time: 601.348078

sensor: 211.200000, time: 601.368061

sensor: 210.936000, time: 601.388043

sensor: 211.002000, time: 601.408023

sensor: 210.870000, time: 601.428005

sensor: 211.134000, time: 601.447982

sensor: 211.332000, time: 601.467964

sensor: 211.266000, time: 601.487949

sensor: 211.332000, time: 601.507935

sensor: 211.398000, time: 601.537901

sensor: 211.398000, time: 601.557887

sensor: 211.200000, time: 601.577872

Next, when testing the linear regression algorithm’s during steady state, the slope have an average value of 0. This makes sense, because during steady state, the flux is not expected to change thus the linear point became a straight line that approximate the X-intercept. However, when a large change in flux happens, the slope grew exponentially. From the result below, it takes 12 data points to get from approximately 0 slope to 5, which is the experimentally determined threshold. This implies approximately 0.0001 seconds since the threat started. The following values are returned:

LINEAR REGRESSION'S SLOPE AS TRIGGER:

slope = -0.025732, intercept = 224.103706

slope = -0.003647, intercept = 209.576933

slope = 0.023635, intercept = 191.626105

slope = 0.066418, intercept = 163.479823

slope = 0.125705, intercept = 124.472289

slope = 0.226535, intercept = 58.135220

slope = 0.365511, intercept = -33.327656

slope = 0.564453, intercept = -164.266292

slope = 0.820646, intercept = -332.894469

slope = 1.122401, intercept = -531.531100

slope = 1.474929, intercept = -763.601431

slope = 1.853198, intercept = -1012.644718

slope = 2.250750, intercept = -1274.418808

slope = 2.671984, intercept = -1551.807599

slope = 3.095761, intercept = -1830.906908

slope = 3.519865, intercept = -2110.246832

slope = 3.912076, intercept = -2368.635682

slope = 4.260228, intercept = -2598.075698

slope = 4.587526, intercept = -2813.808363

slope = 4.880736, intercept = -3007.132940

slope = 5.146420, intercept = -3182.363814

Alert signal is: 1

BAD received

RESPONSE TIME: 0.002165

Something went wrong responding to threat.

As could be seen from the print-out, the threat detection causes the program to pause, and message is read from the UDP socket. When the button is pressed, the program resumes running and returning values. A more accurate picture is shown in the video attached along with this report.

DISCUSSION AND CONCLUSIONS

There are a couple of details that is worth mentioning. The timestamp value yielded by the time library in ANSI C is massive. Performing various processing operations on that kind of number will easily causes overflow and inaccurate data. Thus there is an inherent need to scale down the values. The unit used to describe the sensor data is in mV, and the data ranges from 200 to 50. This means a resolution of 150 possible integers, not counting the floating point values. Assuming there is n possible floating points that the CPU can handle, thus there are 150*n possible values to be calculated and determine the slope. Considering this, it is desirable to scale down the timestamp value such that it is compatible with the sensor values. Attempt to scale up the sensor values to the unit of the timestamp (that is, to multiply the sensor values by 10^10) causes overflow and only yield 0 as a result. Thus scaling up is not a viable option. To the implementer’s discretion, the initial value of the timestamp is set to 600. This is done by subtracting the current time stamp by 600, setting that as a base value, and subtract all the succeeding timestamps by such values before calculating the linear regression slope. This means that every time the linear regression thread is reset, the time stamp will start at 600. Thus, with the sensor value around the value of 200 and the time stamp at 600, the slope calculation is possible without fearing overflow.

There is another problem that comes with selecting the appropriate time stamp scale, and that is the sensitivity of the calculated value, As the timestamp values grew larger, looking back at the linear regression formula, the slope will gradually approaches 0, This implies that the slope will become less sensitive as the time goes on. Thus it is crucial to constantly reset the linear regression thread. Fortunately, the degradation in sensitivity happens so slowly that, in the context of testing the effectiveness of threat-reporting, it takes over 30 minutes before there is noticeable degradation.

The slope value that acts as a threshold that will trigger the threat detection is also experimentally determined, and at the discretion of the implementer.

There is also a question of the slope being too sensitive at lower timestamp values. At low timestamp values, the slope can be too sensitive such that mere act of walking across the room can cause the threat-detection to activate. This is due to the high-quality of the photodiode, being so sensitive to changes in flux. This begs the question about what classifies as a threat and what doesn’t. This problem brings out an interesting possible extension for this project: to extend Weeping Sirens to take in data from a diverse set of sensors in order to perform clustering and determine the pattern that could be classified as an actual threat. Without that level of information, the accuracy of this system still approaches the worst case scenario, in which everything is a threat – which is obviously not the case. This is beyond the scope of this course, but this is nevertheless the potential of this project, to uncover knowledge about humanity that could be used to assist in various industries, for example military or medical systems.

The speed of the response time observed in this project is less than one second, which approaches the split-second reaction time needed for a diverse scenario of physical collision. The primary overhead in the system are the context-switching between threads, the period in which linear regression is performed, the complexity of the linear regression algorithm itself, and the complexity of the response function. The context switching time is dependent upon the machine, thus the way to increase response time is to scale-up the system. The period in which the linear regression could be performed is dependent upon how fast the A/C can be ready. The ideal value suggested in the TS-7250 data sheet is 2 microseconds, with a small range of offset. Thus the period of the linear regression task could be made smaller than 1 millisecond. The complexity of the linear regression task is O(n), with the constraint that it only process one dimension vector and furthermore it assumes the type of data which have a clear linear relationship. Those two constraints could be modified, as they must if the project were to ever become significant, such that the algorithm processes an n dimensional array and there exist such sensors that does not comply with the pattern of linearity with respect to time, then the complexity grew grotesquely. Thus there is a need for a more scale-able algorithm, with the possibility of being even more accurate, than linear regression. The complexity of the response function in this project is O(1). Depending on how the user wants to respond to threat, this have a potential to grow as well. So all in all, there are ways to improve the system’s respond time, which further adds to the potential in extending to the project.

Finally, there needs to be an explanation as to the naming sense of this project. In Greek mythology, Sirens are usually the villain of the story, singing enchanting music to lure seamen to shipwreck on the coast. That had been such a commonly accepted mental image of Sirens that no one really tried to understand the humanity of Sirens. Sirens sing whenever they see a ship coasting by. They are also stuck on an island that they could never leave. Considering for a moment that Sirens have human sentiments, then that means that the days in which there are no ships, they must’ve felt intensely lonely. Whenever a ship came by, it is no wonder they would crave attention, to be rescued, to be loved. Alas, the curse of their voice creates a vicious cycle, comparable to that of Sisyphus, that everyone they tried to reach out to end up dead, shipwrecked and maimed and yet they seek out this sense of fulfillment of human connection that they can never have. Thus the only thing that is left for such a creature is to continue to sing, to reach out, and to live. For if they were to give up, went into recluse and deny their own sense of longing, which would be to deny their own humanity. What other choice do they have then but to jump down the cliff to the rocks as well, to end such misery? There is no hope, even in death for such creatures. Thus they must persevere. Singing, howling at every ship that passes by.

This project is kind of like that. The sensor is so sensitive that it sounded alarm at the most minute changes in light, thus this system is the siren craving attention.

APPENDICES A – THE CODE

A.1) KERNEL SPACE MODULE

<embedded_project_kernel.c>

#ifndef MODULE

#define MODULE

#endif

#ifndef __KERNEL__

#define __KERNEL__

#endif

#include <linux/module.h>

#include <linux/kernel.h>

#include <unistd.h>

#include <rtai.h>

#include <stdio.h>

#include <asm/io.h>

#include <rtai_sched.h>

#include <rtai_fifos.h>

#define IDLE_TIME_CHECK 10      //time to sleep in an infinite check loop so

//the process doesn't hog the memory

#define SOUND_PERIOD    2       //the period, in millisecond, to play sound

//in the sound task

RTIME period;                   // 1 ms time.

// if 1, means threat detected

// if 0, means just chill

int alert_sig = 1;              // alert signal to report to user space

// module

int sound = 0;                  // toggle to play sound

static RT_TASK sound_task;

unsigned char *map, *mapVIC, *PortB, *PortBDR, *PortF, *PortFDR, *GPIOBIntEn, *GPIOBIntType1, *GPIOBIntType2, *GPIOBEOI, *GPIOBDB, *RawIntStsB;

unsigned int *VIC2IntEnable, *VIC2SoftIntClear;

MODULE_LICENSE("GPL");

//returns either 0 or 1, which is the content of the particular bit

//bitwise "AND" operation to the number shifted to the number of bitPosition

//and the bitvector 0000 0001, thus what will be left is either 1 or 0.

//return -1 if error occurs

int getBitContent(int bitPosition, int content);

//returns the content that had been assigned value at bitPosition

//to set to 1, we use the "OR" operation

//to set to 0, we use the "XOR" operation

// but to check that setting to 0 is valid or not, we first "AND"

// with 0x** to see if the result is 0x** or 0x00. If 0x00, do nothing.

// return -1 in case of error

int assignBitContent(int bitPosition, int content, int value);

//RT TASK, TURN ON RED LIGHT FOR A PERIOD OF TIME

//IF GREEN LIGHT IS DONE OR WALKING BUTTON IS PRESSED

static void rt_process_sound(int t);

//When the software interrupt triggers, send the alert signal

//through the FIFO so that the user space can deal with it

//accordingly.

//FIFO channel is FIFO 0

// alert = 0 --> no alert

// alert = 1 --> threat detected

void alert();

//When the software interrupt triggers, meaning button on auxiliary board

//had been pressed, send the alert signal

//through the FIFO so that the user space can deal with it

//accordingly.

//FIFO channel is FIFO 0

// alert = 0 --> turn off threat detection

//turn off the sound

void chill();

int init_module(void) {

int temp, j;

printk("MODULE INSTALLED\n");

//rt_set_periodic_mode();

//make the period 1 ms

period = start_rt_timer(nano2count(1000000));

//Map to registers and port locations

//map the base for Port B and the VIC

map = (unsigned char*)__ioremap(0x80840000, 4096, 0);

mapVIC = (unsigned char*)__ioremap(0x800C0000, 4096, 0);

PortB = (unsigned int*) (map+0x04);

PortBDR = (unsigned int*)(map+0x14);

PortF = (unsigned int*)(map + 0x30);

PortFDR = (unsigned int*)(map + 0x34);

VIC2IntEnable = (unsigned int*)(mapVIC+0x10);

VIC2SoftIntClear = (unsigned int*)(mapVIC+0x1C);

//setting the direction of port B, which will output

//the alert signal

*PortBDR = 0xE0;

//set direction to output for speaker

temp = assignBitContent(1, (int)*PortFDR, 1);

*PortFDR = temp;

//reset lights on auxiliary board

//turn on everything except green

temp = assignBitContent(5, (int)*PortB, 0);

*PortB = temp;

temp = assignBitContent(6, (int)*PortB, 0);

*PortB = temp;

temp = assignBitContent(7, (int)*PortB, 1);

*PortB = temp;

//map registers to control edge-sensitivity and rising/falling edge signals

GPIOBEOI = (unsigned int*)(map+0xB4);

GPIOBDB = (unsigned int*)(map+0xC4);

GPIOBIntEn = (unsigned int*)(map+0xB8);

GPIOBIntType1 = (unsigned int*)(map+0xAC);

GPIOBIntType2 = (unsigned int*)(map+0xB0);

//map registers to enable specific interrupt lines

RawIntStsB = (unsigned int*)(map+0xC0);

//set registers to be edge-sensitive and rising edge signals

*GPIOBIntType1 |= 0x1F;

*GPIOBIntType2 &= 0xE0;

*GPIOBDB |= 0x1F;

//request irq for hardware

rt_request_irq(59, chill, 0, 1);

*GPIOBEOI |= 0x1f;

*GPIOBIntEn |= 0x1f;

//Enable irq for hardware

rt_enable_irq(59);

//request irq for software

rt_request_irq(63, alert, 0, 1);

*VIC2SoftIntClear |= 0x80000000;

*VIC2IntEnable |= 0x80000000;

//Enable irq for Software

rt_enable_irq(63);

//Create a fifo for extra credit and one for software interrupt

rtf_create(0, sizeof(int));

//start rt task

rt_task_init(&sound_task, rt_process_sound, 0, 256, 0, 0, 0);

//Set first tone to be C

rt_task_make_periodic(&sound_task, rt_get_time()+1*period, SOUND_PERIOD*period);

return 0;

}

void cleanup_module(void){

printk("MODULE UNINSTALLED\n");

//disable irq

rt_disable_irq(63);

rt_disable_irq(59);

//release irq

rt_release_irq(63);

rt_disable_irq(59);

//destroy fifos

rtf_destroy(0);

//stop rt timer

stop_rt_timer();

}

//When the software interrupt triggers, meaning button on auxiliary board

//had been pressed, send the alert signal

//through the FIFO so that the user space can deal with it

//accordingly.

//FIFO channel is FIFO 0

// alert = 0 --> turn off threat detection

//turn off the sound

void chill(){

//Disable software interrupt line

rt_disable_irq(59);

alert_sig = 0;

sound = 0;

//send alert value to user space module

rtf_put(0, &alert_sig, sizeof(int));

//clear interrupt

*GPIOBEOI |= 0x1f;

//re-enable the interrrupt line

rt_enable_irq(59);

}

//When the software interrupt triggers, send the alert signal

//through the FIFO so that the user space can deal with it

//accordingly.

//FIFO channel is FIFO 0

// alert = 1 --> threat detected

//toggle on the sound as well

void alert(){

//Disable software interrupt line

rt_disable_irq(63);

alert_sig = 1;

sound = 1;

//send alert value to user space module

rtf_put(0, &alert_sig, sizeof(int));

//clear interrupt

*VIC2SoftIntClear |= 0x80000000;

//re-enable the interrupt

rt_enable_irq(63);

}

//RT TASK, TURN ON RED LIGHT FOR A PERIOD OF TIME

//IF GREEN LIGHT IS DONE OR WALKING BUTTON IS PRESSED

static void rt_process_sound(int t){

int btn_temp = -1;

int toggle_sound = 0;       //switches between 0 and 1 between periods

int light_temp = 0;

int modified_bit_vector;

while(1){

while(sound == 1){

//toggle the value from period to period

if(toggle_sound == 0){

toggle_sound = 1;

} else if(toggle_sound == 1){

toggle_sound = 0;

}

modified_bit_vector = assignBitContent(1, (int)*PortF, toggle_sound);

*PortF = modified_bit_vector;

//wait for next period to play

rt_task_wait_period();

}

//when no buttons pressed, wait then check after a while

rt_sleep(IDLE_TIME_CHECK*period);

}

}

//returns either 0 or 1, which is the content of the particular bit

//bitwise "AND" operation to the number shifted to the number of bitPosition

//and the bitvector 0000 0001, thus what will be left is either 1 or 0.

//return -1 if error occurs

int getBitContent(int bitPosition, int content){

int mask = 0x01;

int shifted = 0;

int result = -1;

shifted = content >> bitPosition;

result = shifted & mask;

//printk("\nResult at bit %d of %d is %d\n", bitPosition, content, result);

return result;

}

//returns the content that had been assigned value at bitPosition

//to set to 1, we use the "OR" operation

//to set to 0, we use the "XOR" operation

// but to check that setting to 0 is valid or not, we first "AND"

// with 0x** to see if the result is 0x** or 0x00. If 0x00, do nothing.

// return -1 in case of error

int assignBitContent(int bitPosition, int content, int value){

int bit_zero = 0x01;

int bit_one = 0x02;

int bit_two = 0x04;

int bit_three = 0x08;

int bit_four = 0x10;

int bit_five = 0x20;

int bit_six = 0x40;

int bit_seven = 0x80;

int mask_bit = 0;

int modified_content = -1;

int checkContent = -1;

switch (bitPosition){

case 0:

mask_bit = bit_zero;

break;

case 1:

mask_bit = bit_one;

break;

case 2:

mask_bit = bit_two;

break;

case 3:

mask_bit = bit_three;

break;

case 4:

mask_bit = bit_four;

break;

case 5:

mask_bit = bit_five;

break;

case 6:

mask_bit = bit_six;

break;

case 7:

mask_bit = bit_seven;

break;

default:

//printk("\nThe fuck, dude?\n");

return -1;

}

if(value == 1){

//turn on by using OR

modified_content = content | mask_bit;

//printk("\nAssigning to 1\n");

}else if(value == 0){

//check if we can turn it off or not

checkContent = content & mask_bit;

//if that particular bit is already 0, then nothing to do

if(checkContent == 0x00){

modified_content = content;

//printk("\nAlready 0\n");

} else if (checkContent == mask_bit) {

//if that particular bit is 1, then we can turn it to 0 with XOR

modified_content = content ^ mask_bit;

//printk("\nAssigning to 0\n");

}

} else{

//printk("\nContent can only be 1 or 0. Twat.\n");

return -1;

}

//printk("\nYou chose to turn bit %d to %d from %d the result is %d\n", bitPosition, value, content, modified_content);

return modified_content;

}

A.2) USER SPACE

<embedded_project_user.c>

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <stddef.h>

#include <unistd.h>

#include <sys/types.h>

#include <sys/socket.h>

#include <sys/stat.h>

#include <sys/mman.h>

#include <netinet/in.h>

#include <netdb.h>

#include <arpa/inet.h>

#include <errno.h>

#include <time.h>

#include <math.h>

#include <rtai.h>

#include <rtai_lxrt.h>

#include <rtai_sched.h>

#include <rtai_fifos.h>

#include <pthread>

#include <fcntl.h>

#include <semaphore.h>

#include <assert.h>

#include "peekpoke.h"

#include "eeprog.h"

#include "ep93xx_adc.h"

#define MSG_SIZE        20// message size

#define HOST_NAME_MAX   255

#define DATA_PAGE       0x80840000

#define CALIB_LOC       2027         //location of calibration values

#define LR_PERIOD       1           //time in ms in which linear regression recalculates

#define WORKING_CHANNEL 1           //the channel we are interested in reading from

#define CALIBRATE_TO_INT 1000       //modifier constant to data to transform

//from floating point to integer

//to lessen computation cost

//determined experimentally with respect to

//convenience

#define SCALE_UP_TIME   1           //modifier constant to scale up the time stamp

//in case the number became too small

#define BASELINE_TIME   600         //the ideal starting value for time, given its

//computational compatibility with the sensor value

//which ranges from 200 mV to 50 mV (determined

//experimentally)

//need this to reset SCALE_DOWN_TIME

//since slope gets less and less sensitive with

//greater time. need to also keep in mind not to

// make it TOO sensitive

#define SCALE_UP_SLOPE  1000000     //a constant modifier that increases the slope value

//to a manageable number

#define SAMPLE_SIZE     50              //size of the array to calculate linear regression

#define TRIGGER_SLOPE   5               //the linear regression slope that signifies a threat

//determined experimentally, subject to change

//depending upon context

RTIME timer;                //time in milliseconds

int kernel_fifo_id = -1;    //fifo id that will communicate with kernel

int alert_sig = 0;          //signal that determines when to turn on red light

//and stop all operations

int SCALE_DOWN_TIME = 0;    //a value to be subtracted from the timestamp

//to scale down the timestamp value

// so that any calculations would be measureable

static unsigned long dr_page, adc_page, syscon_page, pld_page; //mapped ports

//for ADC

long unsigned int *mapVIC, *VIC2SoftInt, *VIC2IntEnable, *VIC2SoftIntClear; //software interrupt register

long unsigned int *map, *PortB, *PortBDR; //port B register

// assuming direction had already been set by

// kernel. if not, mask PortBDR to 0xE0

double old_time;            //time when the program receives alert

double new_time;            //time when program finishes dealing with alert

int stored_cal[5][2];       //stored calibration values for ADC, first dimension are

// 5 channel, second dimension are 2 calibration values

int virgin = TRUE;          //calibration detected?

//NETWORKING VARIABLES FOR SENDING DATA

//binding addresses for socket communication

struct sockaddr_in server;

struct sockaddr_in from;

socklen_t fromlen;

int sock;

void error(const char *msg);

//CONVERT TIMEVAL STRUCTURE TO DOUBLE FOR EASIER PROCESSING

//COMPRESS TO FORMAT: (double) <tv_sec>.<usec>

double timeval_to_double(struct timeval in);

/************************************************************************

*DESCRIPTION: Figure out the calibration value of the board and store them

*stored_cal global variable. In eeprom 2027->2046 are the calibration values. 2027->2036

*are the zero volt calibration values and 2037->2046 are the 2.5V calibration

*values. Each calibration value is 16 bits written using little endian

***********************************************************************/

static void init_adc_channel_calibration();

/******************************************************************************

*DESCRIPTION: Read the data from the desired channel before calibrating it accordingly

*******************************************************************************/

double read_ADC_channel(int ch_num);

//convert string of ip address into integer, replacing '.' wit 0s

int IPToInt(char* ip_addr);

//returns either 0 or 1, which is the content of the particular bit

//bitwise "AND" operation to the number shifted to the number of bitPosition

//and the bitvector 0000 0001, thus what will be left is either 1 or 0.

//return -1 if error occurs

int getBitContent(int bitPosition, int content);

//returns the content that had been assigned value at bitPosition

//to set to 1, we use the "OR" operation

//to set to 0, we use the "XOR" operation

// but to check that setting to 0 is valid or not, we first "AND"

// with 0x** to see if the result is 0x** or 0x00. If 0x00, do nothing.

// return -1 in case of error

int assignBitContent(int bitPosition, int content, int value);

//calculates the linear regression of the incoming data

// ADC only deals with 1 channel as to simplify this project

// thus we read in from only channel 1

// first takes in data for the first 100 values to

//  create a baseline model and set thresholds

//  then each incoming value will be evaluated

//  if it breaches the threshold, then set the software interrupt

void* linear_regression();

//THREAD FUNCTION: reads in data from the fifo

//if software interrupt triggers in kernel space module

// then data will be sent through FIFO ID 0

// if the read data is 1, then alert happens

//  and we turn on the red light on auxiliary board

//  and turn off every other light

void* check_alert();

// when an alert had been triggered, call this function

// this is suppose to be a prototype function where

// anyone can use to implement their own response to a

// threat. For this project, the response function

// will turn off all LEDs in the auxiliary board except

// the red one, then report a time stamp.

int response();

int main(int argc, char *argv[]){

pthread_t receiver, linear_regression_thr;

void* ret_val;

int length, n;

int boolval = 1; // for a socket option

int port_no = -1;

int host_ip_num = -1;

int temp = 0;

//structure storing data for acquiring IP address

struct hostent *he;

char buffer_msg[MSG_SIZE];

char host_ip[100];

char hostname[HOST_NAME_MAX]; // from the linux documentation:

//SUSv2 guarantees that "Host names are limited to 255 bytes".

//POSIX.1-2001 guarantees that "Host names

//(not including the terminating null byte) are

//limited to HOST_NAME_MAX bytes".

//On Linux, HOST_NAME_MAX is defined with the value 64,

//which has been the limit since Linux 1.0

//(earlier kernels imposed a limit of 8 bytes).

if (argc < 2){

fprintf(stderr, "ERROR, no port provided\n");

printf("\nBut it's ok, since you are in this lab, the port is always 2000. Just think about how lucky you are.\n");

port_no = 2000;

//exit(0);

} else {

port_no = atoi(argv[1]);

}

/**************************************************************************

*   BEGIN NETWORK COMMUNICATION SETUP

**************************************************************************/

sock = socket(AF_INET, SOCK_DGRAM, 0); // Creates socket. Connectionless.

if (sock < 0)

error("Opening socket");

length = sizeof(server);             // length of structure

bzero(&server, length);             // sets all values to zero. memset() could be used

server.sin_family = AF_INET;         // symbol constant for Internet domain

server.sin_addr.s_addr = INADDR_ANY; // IP address of the machine on which

// the server is running

server.sin_port = htons(port_no); // port number

// binds the socket to the address of the host and the port number

if (bind(sock, (struct sockaddr *) &server, length) < 0)

error("binding");

// set broadcast option

if (setsockopt(sock, SOL_SOCKET, SO_BROADCAST, &boolval, sizeof(boolval))< 0){

printf("error setting socket options\n");

exit(-1);

}

fromlen = sizeof(struct sockaddr_in); // size of structure

//figuring out the host IP address

if (gethostname(hostname, HOST_NAME_MAX) != 0) { //get self's host name

//to figure out IP from

printf("\nError gethostname: %s\n", strerror(errno));

exit(-1);

}

if ((he = gethostbyname(hostname)) == NULL) { //get the pointer to

//structure hostent

//for the given host name

// get the host info

herror("gethostbyname");

exit(-1);

}

//h_addr is an element of the hostent structure

//that stores the host's own address.

//since the result is returned as a pointer, we need to dereference it

//as well.

//inet_ntoa() function converts the Internet host address in,

//given in network byte order, to a string in IPv4 dotted-decimal notation.

//The string is returned in a statically allocated buffer, which subsequent

//calls will overwrite.

strcpy(host_ip, inet_ntoa(*((struct in_addr*) he->h_addr))); //host_ip now stores

//the string of IP of

//the host

host_ip_num = IPToInt(host_ip);

printf("\nHost IP string: %s, num %d\n", host_ip, host_ip_num);

/**************************************************************************

*   END NETWORK COMMUNICATION SETUP

***************************************************************************/

/**************************************************************************

*   BEGIN MAP PORTS

***************************************************************************/

//map board to correct location on TS-7250

int board = open("/dev/mem", O_RDWR|O_SYNC);

//VIC registers

mapVIC = (long unsigned*)mmap(0, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED, board, 0x800C0000);

VIC2SoftInt = (long unsigned)(mapVIC) | 0x18;

VIC2IntEnable = (long unsigned)(mapVIC) | 0x10;

VIC2SoftIntClear = (long unsigned)mapVIC | 0x1C;

//port b registers

map = (long unsigned*)mmap(0, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED, board, 0x80840000);

PortB = (long unsigned)(map) | 0x04;

PortBDR = (long unsigned)(map) | 0x14;

//map ports for ADC

dr_page = (unsigned long)mmap(0, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED, board, DATA_PAGE);

assert(&dr_page != MAP_FAILED);

spistart = (unsigned long)mmap(0, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED, board, SPI_PAGE);

assert(&spistart != MAP_FAILED);

adc_page = (unsigned long)mmap(0, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED, board, ADC_PAGE);

assert(&adc_page != MAP_FAILED);

syscon_page = (unsigned long)mmap(0, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED , board, SYSCON_PAGE);

assert(&syscon_page != MAP_FAILED);

pld_page = (unsigned long)mmap(0, getpagesize(), PROT_READ|PROT_WRITE, MAP_SHARED, board, PLD_PAGE);

assert(&pld_page != MAP_FAILED);

//initialize ADC

init_ADC(adc_page, syscon_page);

init_adc_channel_calibration();

printf("\nADC initialized\n");

/**************************************************************************

*   END MAP PORTS

**************************************************************************/

/**************************************************************************

*   BEGIN REAL TIME TASKS SETUP

**************************************************************************/

timer = start_rt_timer(nano2count(1000000));

//create the thread that awaits data from fifo

if(pthread_create(&receiver, NULL, (void*)check_alert, NULL) != 0){

printf("\nError creating alert thread: \n%s\n\n", strerror(errno));

}

if(pthread_create(&linear_regression_thr, NULL, (void*)linear_regression, NULL) != 0){

printf("\nError creating LR thread: \n%s\n\n", strerror(errno));

}

/**************************************************************************

*   END REAL TIME TASKS SETUP

**************************************************************************/

/**************************************************************************

*   RT LOOP

*   for continuous communication

**************************************************************************/

printf("\nSocket setup complete\n");

while (1){

// bzero: to "clean up" the buffer. The messages aren't always the same length...

bzero(buffer_msg, MSG_SIZE); // sets all values to zero. memset() could be used

// receive from a client

n = recvfrom(sock, buffer_msg, MSG_SIZE, 0, (struct sockaddr *) &from,&fromlen);

if (n < 0)

error("recvfrom");

//printf("Received a datagram. It says:\n%s\n", buffer_msg);

// To send a broadcast message, we need to change IP address to broadcast address

// (.255). If we don't change it (with the following line of code), the message

// would be transmitted to the address from which the message was received.

from.sin_addr.s_addr = inet_addr("10.3.52.255"); // broadcast address

//HANDLING DIFFERENT REQUESTS

if (strcmp(buffer_msg, "GOOD") == 0) { //other machines reporting that

//things are ok now

bzero(buffer_msg, MSG_SIZE);

printf("\nGOOD received\n");

if(alert_sig != 0){

//reset the sig status

alert_sig = 0;

//close red light and turn on green light

temp = assignBitContent(5, (int)*PortB, 0);

*(PortB) = temp;

temp = assignBitContent(6, (int)*(PortB), 0);

*(PortB) = temp;

temp = assignBitContent(7, (int)*(PortB), 1);

*(PortB) = temp;

}

} else if(strcmp(buffer_msg, "BAD") == 0){  //other machines telling

//us that things are

//getting bad

printf("\nBAD received\n");

if(alert_sig != 1){

alert_sig = 1;

//close green light and turn on red light

temp = assignBitContent(5, (int)*(PortB), 1);

*(PortB) = temp;

temp = assignBitContent(6, (int)*(PortB), 0);

*(PortB) = temp;

temp = assignBitContent(7, (int)*(PortB), 0);

*(PortB) = temp;

//triggers software interrupt

*VIC2SoftInt = 0x80000000;

}

}

}

stop_rt_timer();

return 0;

}

// when an alert had been triggered, call this function

// this is suppose to be a prototype function where

// anyone can use to implement their own response to a

// threat. For this project, the response function

// will turn off all LEDs in the auxiliary board except

// the red one, then report a time stamp.

// IF all goes well, return 1, if not then return 0.

int response(){

int status = 0;

int temp;

double response_time = 0;

struct timeval time_buffer;

char message[MSG_SIZE];

server.sin_addr.s_addr = inet_addr("10.3.52.255"); // broadcast address

if(alert_sig == 1){

strcpy(message, "BAD");

//send out alert messages

if(sendto(sock, message, MSG_SIZE, 0, (struct sockaddr *) &server, sizeof(struct sockaddr_in)) < 0){

//printf("Error sending message! %s", strerror(errno));

error("sendto");

}

//close green light and turn on red light

temp = assignBitContent(5, (int)*(PortB), 1);

*(PortB) = temp;

temp = assignBitContent(6, (int)*(PortB), 0);

*(PortB) = temp;

temp = assignBitContent(7, (int)*(PortB), 0);

*(PortB) = temp;

//report timestamp

gettimeofday(&time_buffer, NULL);

new_time = timeval_to_double(time_buffer);

response_time = new_time - old_time;

//REPORT RESPONSE TIME

printf("\nRESPONSE TIME: %lf\n", response_time);

} else if(alert_sig == 0){

strcpy(message, "GOOD");

//send out OK messages

if(sendto(sock, message, MSG_SIZE, 0, (struct sockaddr *) &server, sizeof(struct sockaddr_in)) < 0){

//printf("Error sending message! %s", strerror(errno));

error("sendto");

}

//close red light and turn on green light

temp = assignBitContent(5, (int)*(PortB), 0);

*(PortB) = temp;

temp = assignBitContent(6, (int)*(PortB), 0);

*(PortB) = temp;

temp = assignBitContent(7, (int)*(PortB), 1);

*(PortB) = temp;

}

return status;

}

//THREAD FUNCTION: reads in data from the fifo

//if software interrupt triggers in kernel space module

// then data will be sent through FIFO ID 0

// if the read data is 1, then alert happens

//  and we turn on the red light on auxiliary board

//  and turn off every other light

// No need to set up RT task, since we depend on

// read as a blocking function, thus it is not

// dependent upon scheduling schemes

void* check_alert(){

struct timeval time_buffer;

int temp = -1;

//OPENING FIFO ACCESS --> ASSUMING THAT ID OF THE FIFO IS 0

if((kernel_fifo_id = open("/dev/rtf/0", O_RDONLY))<0){

printf("\nERROR OPENING FIFO from /dev/rtf/0\n");

exit(-1);

}

//continuously read from kernel

while(1){

//reading from the fifo that was sent from kernel space module

read(kernel_fifo_id, &temp, sizeof(int));

alert_sig = temp;

printf("\nAlert signal is: %d\n", alert_sig);

/* PROTOTYPE BLOCK

ARBITRARY RESPONSE */

if(response() == 0){

printf("\nSomething went wrong responding to threat.\n");

}

/* END OF PROTOTYPE BLOCK*/

}

pthread_exit(0);

}

//calculates the linear regression of the incoming data

// ADC only deals with 1 channel as to simplify this project

// thus we read in from only channel 1

// first takes in data for the first 100 values to

//  create a baseline model and set thresholds

//  then each incoming value will be evaluated

//  if it breaches the threshold, then set the software interrupt

//

//  Simple Linear Regression: y = b_0 + x*b_1

//  the independent variable x is time

//  the dependent variable y is the sensor value

//  b_0 is the intercept, b_1 is the slope

void* linear_regression(){

RT_TASK* task1 = rt_task_init(nam2num("lnr"), 0, 512, 256);

double init_sensor_collection[SAMPLE_SIZE];

double init_time_collection[SAMPLE_SIZE];

struct timeval time_buffer;

double slope, intercept = 0.0;

double x_sum, y_sum, xy_sum, x_square_sum = 0.0;

double temp_time, temp_sensor = 0.0;

int i;

//set timing for task

//argumentS: task name, start time, period

rt_task_make_periodic(task1, rt_get_time()+2*timer, LR_PERIOD*timer);

while(1){

//init block

slope = 0.0;

intercept = 0.0;

x_sum = 0.0;

y_sum = 0.0;

xy_sum = 0.0;

x_square_sum = 0.0;

gettimeofday(&time_buffer, NULL);

SCALE_DOWN_TIME = timeval_to_double(time_buffer) - BASELINE_TIME;

if(alert_sig == 0){             //init the linear regression model

//when there are no threat

//collect the initial data, 100 samples

//for both the time and the sensor data

for(i=0;i<SAMPLE_SIZE;i++){

init_sensor_collection[i] = CALIBRATE_TO_INT*read_ADC_channel(WORKING_CHANNEL);

gettimeofday(&time_buffer, NULL);

init_time_collection[i] = SCALE_UP_TIME*(timeval_to_double(time_buffer) - SCALE_DOWN_TIME);

//LOOK AT THIS DEBUG LINE TO DETERMINE HOW MUCH OFFSET

//FOR TIME STAMP

printf("\nsensor: %lf, time: %lf\n", init_sensor_collection[i], init_time_collection[i]);

//calculate the sum terms

x_sum += init_time_collection[i];

y_sum += init_sensor_collection[i];

xy_sum += (init_time_collection[i]*init_sensor_collection[i]);

x_square_sum = (init_time_collection[i]*init_time_collection[i]);

}

//calculate slope

slope = SCALE_UP_SLOPE*(((SAMPLE_SIZE*xy_sum) - (x_sum*y_sum))/((SAMPLE_SIZE*x_square_sum) - (x_sum*x_sum)));

//calulate intercept

intercept = (y_sum - slope*x_sum)/SAMPLE_SIZE;

printf("\nINIT slope = %lf, intercept = %lf\n", slope, intercept);

}

while(alert_sig == 0){              //continue detecting threat while

//there is nothing detected yet

temp_sensor = CALIBRATE_TO_INT*read_ADC_channel(WORKING_CHANNEL);

gettimeofday(&time_buffer, NULL);

temp_time = SCALE_UP_TIME*(timeval_to_double(time_buffer) - SCALE_DOWN_TIME);

//re-calculate the sum terms

//removing the oldest term, and calculate first

x_sum += temp_time - init_time_collection[0];

y_sum += temp_sensor - init_sensor_collection[0];

xy_sum += (temp_time*temp_sensor) - (init_time_collection[0]*init_sensor_collection[0]);

x_square_sum = (temp_time*temp_time) - (init_time_collection[0]*init_time_collection[0]);

//rearrange the array, pop out oldest element and push in new one

for(i=0;i<(SAMPLE_SIZE-1);i++){

init_time_collection[i] = init_time_collection[i+1];

init_sensor_collection[i] = init_sensor_collection[i+1];

}

init_time_collection[SAMPLE_SIZE-1] = temp_time;

init_sensor_collection[SAMPLE_SIZE-1] = temp_sensor;

//re-calculate slope

slope = SCALE_UP_SLOPE*(((SAMPLE_SIZE*xy_sum) - (x_sum*y_sum))/((SAMPLE_SIZE*x_square_sum) - (x_sum*x_sum)));

//re-calulate intercept

intercept = (y_sum - slope*x_sum)/SAMPLE_SIZE;

//printf("\nINIT slope = %lf, intercept = %lf\n", slope, intercept);

printf("\nslope = %lf, intercept = %lf\n", slope, intercept);

if(slope >= TRIGGER_SLOPE){     //THREAT DETECTED

//get time stamp

gettimeofday(&time_buffer, NULL);

old_time = timeval_to_double(time_buffer);

//triggers software interrupt

*VIC2SoftInt = 0x80000000;

}

rt_task_wait_period();

}

//let go of the CPU for 10 milliseconds to allow other

//processes a chance to run

usleep(10000);

}

pthread_exit(0);

}

/************************************************************************

*DESCRIPTION: Figure out the calibration value of the board and store them

*stored_cal global variable. In eeprom 2027->2046 are the calibration values. 2027->2036

*are the zero volt calibration values and 2037->2046 are the 2.5V calibration

*values. Each calibration value is 16 bits written using little endian

***********************************************************************/

static void init_adc_channel_calibration(){

int i, j, ch, addr;

char buffer[20];

/* intialize the eeprom */

POKE16(pld_page, (PEEK16(pld_page) & ~CS_MSK)); //disable CS

POKE32((spistart + SSPCR1), 0x10);  //turn on transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for unbusy

while ((PEEK32(spistart + 0xc) & 0x5) != 0x1);   // empty FIFO's

POKE16(pld_page, (PEEK16(pld_page) | CS_MSK));   //enable CS

POKE32(spistart, 0xc7);                          // set SPI mode 3, 8bit

POKE32(spistart + 0x10, 0x2);                    // divide clk by 2

POKE32(spistart + 0x4, 0x0);                     // stop transmit

while((ee_rdsr(dr_page) & 0x1) == 0x1);//wait for unbusy

ee_wren(dr_page);

ee_wrsr(dr_page, 0x0);              // make eeprom R/W

while((ee_rdsr(dr_page) & 0x1) == 0x1);//wait for unbusy

/* Read in the calibration values */

//printf("Calibration Values = \n[ ");

addr = CALIB_LOC;

for(i = 0; i < 20; i++)

{

ee_read_byte(dr_page, addr, &buffer[i]);

//check if board has stored calibration values

if(buffer[i] != 0xFF)

virgin = FALSE;

addr++;

}

//convert to 16 bit values

j = 0;

ch = 0;

for(i = 0; i < 20; i = i + 2)

{

if(i == 10)

{

ch = 0;

j = 1;

}

stored_cal[ch][j] = (buffer[i] | (buffer[i+1] << 8));

//printf("(%d, %d) = 0x%x | ", ch, j, stored_cal[ch][j]);

ch++;

}

//printf(" ]\n");

//make eeprom RO

ee_wren(dr_page);

ee_wrsr(dr_page, 0x1c);

}

/******************************************************************************

*DESCRIPTION: Read the data from the desired channel before calibrating it accordingly

*******************************************************************************/

double read_ADC_channel(int ch_num){

double val, full_scale;

int cur_ch;

int result;

//printf("\nStored Calibration: %x %x\n", stored_cal[ch_num][0], stored_cal[ch_num][1]);

/* intialize the eeprom */

POKE16(pld_page, (PEEK16(pld_page) & ~CS_MSK)); //disable CS

POKE32((spistart + SSPCR1), 0x10);  //turn on transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for unbusy

while ((PEEK32(spistart + 0xc) & 0x5) != 0x1);   // empty FIFO's

POKE16(pld_page, (PEEK16(pld_page) | CS_MSK));   //enable CS

POKE32(spistart, 0xc7);                          // set SPI mode 3, 8bit

POKE32(spistart + 0x10, 0x2);                    // divide clk by 2

POKE32(spistart + 0x4, 0x0);                     // stop transmit

while((ee_rdsr(dr_page) & 0x1) == 0x1);//wait for unbusy

ee_wren(dr_page);

ee_wrsr(dr_page, 0x0);              // make eeprom R/W

while((ee_rdsr(dr_page) & 0x1) == 0x1);//wait for unbusy

//select channel address

switch(ch_num)

{

case 0:

cur_ch = ADC_CH0;

break;

case 1:

cur_ch = ADC_CH1;

break;

case 2:

cur_ch = ADC_CH2;

break;

case 3:

cur_ch = ADC_CH3;

break;

case 4:

cur_ch = ADC_CH4;

break;

}

//Read the adc values for all 5 channels

//read_7xxx_adc(adc_result);

//discard first two samples

read_channel(adc_page, cur_ch);

read_channel(adc_page, cur_ch);

usleep(10000);

result = read_channel(adc_page, cur_ch);

/*if(virgin == TRUE)

printf("No calibration values found...\n");

else

printf("Board has been calibrated by Technologic Systems...\n");

*/

full_scale = (((((double)(stored_cal[ch_num][1] + 0x10000)

- stored_cal[ch_num][0]) / 2.5 ) * 3.3 ));

if(result < 0x7000)

result = result + 0x10000;

if(virgin == TRUE)  //use approximation

{

result = result - 0x9E58;

val = ((double)result * 3.3) / 0xC350;

}

else                //use calibration values

{

result = result - stored_cal[ch_num][0];

val = ((double)result * 3.3) / full_scale;

}

//printf("Channel %d: %3.3fV\n", i, val);

//make eeprom RO

ee_wren(dr_page);

ee_wrsr(dr_page, 0x1c);

return val;

}

//CONVERT TIMEVAL STRUCTURE TO DOUBLE FOR EASIER PROCESSING

//COMPRESS TO FORMAT: (double) <tv_sec>.<usec>

double timeval_to_double(struct timeval in){

double out = 0.0;

//double i = ((double)(in.tv_usec))/1000000;

out = in.tv_sec+(((double)(in.tv_usec))/1000000);

//printf("\nCONVERTING TO DOUBLE: usec is %lf out time is %lf\n", i, out);

return out;

}

void error(const char *msg){

perror(msg);

exit(0);

}

//convert string of ip address into integer, replacing '.' wit 0s

int IPToInt(char* ip_addr) {

int i = 0;

char buffer_ip[100];

int ip_num = -1;

for (i = 0; i < strlen(ip_addr); i++) {

//change all the '.' to 0's

if (ip_addr[i] == '.') {

buffer_ip[i] = '0';

} else {

buffer_ip[i] = ip_addr[i];

}

}

buffer_ip[i] = '\0';

ip_num = atoi(buffer_ip);  //this is used to compare the host ip with others

//in case of tie vote

return ip_num;

}

//returns either 0 or 1, which is the content of the particular bit

//bitwise "AND" operation to the number shifted to the number of bitPosition

//and the bitvector 0000 0001, thus what will be left is either 1 or 0.

//return -1 if error occurs

int getBitContent(int bitPosition, int content){

int mask = 0x01;

int shifted = 0;

int result = -1;

shifted = content >> bitPosition;

result = shifted & mask;

//printk("\nResult at bit %d of %d is %d\n", bitPosition, content, result);

return result;

}

//returns the content that had been assigned value at bitPosition

//to set to 1, we use the "OR" operation

//to set to 0, we use the "XOR" operation

// but to check that setting to 0 is valid or not, we first "AND"

// with 0x** to see if the result is 0x** or 0x00. If 0x00, do nothing.

// return -1 in case of error

int assignBitContent(int bitPosition, int content, int value){

int bit_zero = 0x01;

int bit_one = 0x02;

int bit_two = 0x04;

int bit_three = 0x08;

int bit_four = 0x10;

int bit_five = 0x20;

int bit_six = 0x40;

int bit_seven = 0x80;

int mask_bit = 0;

int modified_content = -1;

int checkContent = -1;

switch (bitPosition){

case 0:

mask_bit = bit_zero;

break;

case 1:

mask_bit = bit_one;

break;

case 2:

mask_bit = bit_two;

break;

case 3:

mask_bit = bit_three;

break;

case 4:

mask_bit = bit_four;

break;

case 5:

mask_bit = bit_five;

break;

case 6:

mask_bit = bit_six;

break;

case 7:

mask_bit = bit_seven;

break;

default:

//printk("\nThe fuck, dude?\n");

return -1;

}

if(value == 1){

//turn on by using OR

modified_content = content | mask_bit;

//printk("\nAssigning to 1\n");

}else if(value == 0){

//check if we can turn it off or not

checkContent = content & mask_bit;

//if that particular bit is already 0, then nothing to do

if(checkContent == 0x00){

modified_content = content;

//printk("\nAlready 0\n");

} else if (checkContent == mask_bit) {

//if that particular bit is 1, then we can turn it to 0 with XOR

modified_content = content ^ mask_bit;

//printk("\nAssigning to 0\n");

}

} else{

//printk("\nContent can only be 1 or 0. Twat.\n");

return -1;

}

//printk("\nYou chose to turn bit %d to %d from %d the result is %d\n", bitPosition, value, content, modified_content);

return modified_content;

}

<ep93xx_adc.h>

#define ADC_PAGE          0x80900000

#define ADCRESULT_OFFSET  0x0008

#define SDR_MASK          0x80000000

#define DATA_OFFSET       0x0008

#define DATA_MASK         0xFFFF

#define ADCSWITCH_OFFSET  0x0018

#define ADC_CH0           0x0608

#define ADC_CH1           0x0680

#define ADC_CH2           0x0640

#define ADC_CH3           0x0620

#define ADC_CH4           0x0610

#define ADCSWLOCK_OFFSET  0x0020

#define UNLOCK_VAL        0xAA

#define SYSCON_PAGE       0x80930000

#define ADCCLKDIV_OFFSET  0x0090

#define SYSCON_UNLOCK     0x00C0

#define TSEN_MASK         0x80000000

#define DEVICECFG_OFFSET  0x0080

#define ADCPD_MASK        0x02

#define ADCEN_MASK        0x20000

#define TRUE              1

#define FALSE             0

/*  prototypes  */

void init_ADC(unsigned long adc_page, unsigned long syscon_page);

int read_channel(unsigned long adc_page, unsigned short channel);

static char is_ADC_busy(unsigned long adc_page);

void init_ADC(unsigned long adc_page, unsigned long syscon_page)

{

unsigned long val;

/*    set TSEN bit     */

val = PEEK32(syscon_page + ADCCLKDIV_OFFSET);

//unlock the software lock

POKE32(syscon_page + SYSCON_UNLOCK, UNLOCK_VAL);

POKE32(syscon_page + ADCCLKDIV_OFFSET, TSEN_MASK | val);

/*    set ADCEN bit    */

val = PEEK32(syscon_page + DEVICECFG_OFFSET);

POKE32(adc_page + SYSCON_UNLOCK, UNLOCK_VAL); //unlock the soft lock

POKE32(syscon_page + DEVICECFG_OFFSET, val | ADCEN_MASK);

/*    clear ADCPD bit  */

val = PEEK32(syscon_page + DEVICECFG_OFFSET);

POKE32(adc_page + SYSCON_UNLOCK, UNLOCK_VAL); //unlock the soft lock

POKE32(syscon_page + DEVICECFG_OFFSET, val & ~ADCPD_MASK);

}

int read_channel(unsigned long adc_page, unsigned short channel)

{

unsigned long val;

POKE32(adc_page + ADCSWLOCK_OFFSET, UNLOCK_VAL); //unlock the soft lock

//write ADCSwitch reg to select channel

POKE32(adc_page + ADCSWITCH_OFFSET, channel);

while(is_ADC_busy(adc_page)); //poll ADCResult

//read result from data regisyyter

val = PEEK32(adc_page + DATA_OFFSET) ;

val = val & DATA_MASK;

return val;

}

static char is_ADC_busy(unsigned long adc_page)

{

unsigned long val;

val = PEEK32(adc_page + ADCRESULT_OFFSET);

if((val & SDR_MASK) == SDR_MASK)

return TRUE;

return FALSE;

}

<eeprog.h>

/* TS7250 EEPROM info */

#define SSPCR1           0x04

#define SSPCPSR          0x10

#define CS_MSK           0x02

#define SSP_DATA         0x08

#define SPI_PAGE    0x808a0000

#define PLD_PAGE    0x23000000

static unsigned long spistart;

// Write Disable

void ee_wrdi(unsigned long dr_page) {

POKE32(spistart + 0x8, 0x4);                     // push command byte

POKE32(spistart + 0x4, 0x10);                    // start transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for completion

PEEK32(spistart + 0x8);                          // pop byte 1 (ignore)

POKE32(spistart + 0x4, 0x0);                     // stop transmit

}

// Write Enable

void ee_wren(unsigned long dr_page) {

POKE32(spistart + 0x8, 0x6);                     // push command byte

POKE32(spistart + 0x4, 0x10);                    // start transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for completion

PEEK32(spistart + 0x8);                          // pop byte 1 (ignore)

POKE32(spistart + 0x4, 0x0);                     // stop transmit

}

// Read Status Register

unsigned long ee_rdsr(unsigned long dr_page) {

unsigned long sreg;

POKE32(spistart + 0x8, 0x5);                     // push command byte

POKE32(spistart + 0x8, 0x0);                     // push for recv space

POKE32(spistart + 0x4, 0x10);                    // start transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for completion

PEEK32(spistart + 0x8);                          // pop byte 1 (ignore)

sreg = PEEK32(spistart + 0x8) & 0xff;            // pop recv'd status

POKE32(spistart + 0x4, 0x0);                     // stop transmit

return sreg;

}

// Write Status Register

void ee_wrsr(unsigned long dr_page, unsigned long sreg) {

POKE32(spistart + 0x8, 0x1);                     // push command byte

POKE32(spistart + 0x8, sreg);                    // push new status

POKE32(spistart + 0x4, 0x10);                    // start transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for completion

PEEK32(spistart + 0x8);                          // pop byte 1 (ignore)

PEEK32(spistart + 0x8);                          // pop byte 2 (ignore)

POKE32(spistart + 0x4, 0x0);                     // stop transmit

}

// Write to Memory Array

void ee_write_byte(unsigned long dr_page, unsigned long addr,

unsigned char *dat) {

POKE32(spistart + 0x8, 0x2);                     // push command byte

POKE32(spistart + 0x8, (addr & 0xff00) >> 8);    // push addr msb

POKE32(spistart + 0x8, (addr & 0xff));           // push addr lsb

POKE32(spistart + 0x8, dat[0]);                  // push data byte 0

POKE32(spistart + 0x4, 0x10);                    // start transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for completion

PEEK32(spistart + 0x8);                          // pop byte 1 (ignore)

PEEK32(spistart + 0x8);                          // pop byte 2 (ignore)

PEEK32(spistart + 0x8);                          // pop byte 3 (ignore)

PEEK32(spistart + 0x8);                          // pop byte 4 (ignore)

POKE32(spistart + 0x4, 0x0);                     // stop transmit

}

// Read from Memory Array

void ee_read_byte(unsigned long dr_page, unsigned long addr,

unsigned char *dat) {

POKE32(spistart + 0x8, 0x3);                     // push command byte

POKE32(spistart + 0x8, (addr & 0xff00) >> 8);    // push addr msb

POKE32(spistart + 0x8, (addr & 0xff));           // push addr lsb

POKE32(spistart + 0x8, 0x0);                     // push for recv space

POKE32(spistart + 0x4, 0x10);                    // start transmit

while ((PEEK32(spistart + 0xc) & 0x10) == 0x10); // wait for completion

PEEK32(spistart + 0x8);                          // pop byte 1 (ignore)

PEEK32(spistart + 0x8);                          // pop byte 2 (ignore)

PEEK32(spistart + 0x8);                          // pop byte 3 (ignore)

dat[0] = PEEK32(spistart + 0x8);                 // pop data byte 1

POKE32(spistart + 0x4, 0x0);                     // stop transmit

}

<peekpoke.h>

// We can't expect that a dereference of an unsigned short * always

// produces a ldrh (LOAD) or strh (STORE) since the compiler may choose to use

// a byte write instead.  Hence, we emit the peeks and pokes using

// inline assembler.  --JO

static inline unsigned short PEEK16(unsigned long addr) {

unsigned short ret;

asm volatile (

"ldrh %0, [ %1 ]\n"

: "=r" (ret)

: "r" (addr)

: "memory"

);

return ret;

}

static inline void POKE16(unsigned long addr, unsigned short dat) {

asm volatile (

"strh %1, [ %0 ]\n"

:

: "r" (addr), "r" (dat)

: "memory"

);

}

static inline unsigned long PEEK32(unsigned long addr) {

unsigned long ret;

asm volatile (

"ldr %0, [ %1 ]\n"

: "=r" (ret)

: "r" (addr)

: "memory"

);

return ret;

}

static inline void POKE32(unsigned long addr, unsigned long dat) {

asm volatile (

"str %1, [ %0 ]\n"

:

: "r" (addr), "r" (dat)

: "memory"

);

}

static inline unsigned char PEEK8(unsigned long addr) {

unsigned char ret;

asm volatile (

"ldrb %0, [ %1 ]\n"

: "=r" (ret)

: "r" (addr)

: "memory"

);

return ret;

}

static inline void POKE8(unsigned long addr, unsigned char dat) {

asm volatile (

"strb %1, [ %0 ]\n"

:

: "r" (addr), "r" (dat)

: "memory"

);

}

#define TRUE  0x01

#define FALSE 0x00